3GPP TSG-WG SA2 Meeting #161	S2-2402472

# 26 February – 1 March, 2024, Athens		(revision of S2-2401451)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5220 | rev | 3 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBSR mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2024-01-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN3 has specified the MBSR migration procedures in rel-18 TS 38.401 with additional information provided in S2-2400065/R3-238043. It needs to be further reflected in SA2 specs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the description of MBSR mobility with reference to TS 38.401. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing certain aspects of the MBSR mobility. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35A.3.1, 5.35A.3.2, 5.35A.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Changes of Revision 3(see yellow colour): In case of MBSR with activated PDU session, the existing handover procedures is used to support mobility of MBSR between different IAB-donor gNBs. In case of MBSR with no PDU session or MBSR with no activated PDU session, the following procedes are used to support mobility of MBSR between different IAB-donor gNBs: - During Xn based inter NG-RAN handover, the details are as described in clause 8.2 of TS 38.423. - During N2 based handover, the details are as described in clause 8.4.1 and clause 8.4.2 of TS 38.413. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.35A.3.1	UE mobility between a fixed cell and MBSR cell

For UEs in RRC_IDLE and RRC_INACTIVE state when a MBSR goes out-of-service, procedure for cell (re-)selection as specified in TS 38.304 [50] for RRC_IDLE and RRC_INACTIVE is used between MBSR cell and other cells.

If the MBSR goes out-of-service (due to e.g. MBSR moves to an area where the MBSR is not allowed to provide the relay service), the NG-RAN triggers the handover procedures to the neighbouring cells for UEs in RRC_CONNECTED state served by the MBSR. Any change of authorization state of the MBSR is also handled according to clause 5.35A.4.

The IAB-donor-CU triggers handover procedure when it is possible for the UEs accessing emergency service and being served by the MBSR, if MBSR is about to become unavailable to provide the services.

# * * * * Next change * * * *

5.35A.3.2	UE mobility between MBSR cells

Similar to the behaviours described in clause 5.35A.3.1, UEs and NG-RAN use existing procedures as specified in clause 8.23 of TS 38.401 [42], TS 23.502 [3], or TS 38.304 [50] to handle the mobility between MBSR cells.

# * * * * Next change * * * *

5.35A.3.x	MBSR mobility

The mobility of MBSR between different IAB-donor gNBs is described in clause 8.23 of TS 38.401 [42].

In case of MBSR with activated PDU session, the existing handover procedures is used to support mobility of MBSR between different IAB-donor gNBs.

In case of MBSR with no PDU session or MBSR with no activated PDU session, the following procedes are used to support mobility of MBSR between different IAB-donor gNBs:

-	During Xn based inter NG-RAN handover, the details are as described in clause 8.2 of TS 38.423[99].

-	During N2 based handover, the details are as described in clause 8.4.1 and clause 8.4.2 of TS 38.413[34].

# * * * * End of changes * * * *