3GPP TSG-WG SA2 Meeting #161	S2-2402496

# Athens, Greece, Feb 26 – Mar 1, 2024		(revision of S2-2401561)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1243 | rev | 2 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on URSP rule enforcement reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [China Mobile, ZTE, Ericsson, Nokia], Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As specified in 23.503, the UE may report URSP rule enforcement results, i.e. connection capabilities of the enforced rules, to PCF so that PCF can be aware of specific URSP rule(s) being enforced and take action accordingly. Meanwhile it also allows UE sends multiple enforcement reports in a PDU session modification request and enforced rules contain multiple connection capabilities. Then in some cases the PCF may not be able to identify the specific enforced rules. Assume we have three rules for a PDU session with connection capabilities as follows: rule 1: CC1 and CC2 rule 2: CC2 and CC3 Rule 3: CC3 The writer of the contribution believes this case is possible because the CC is determined by the services or applications that the PDU session is supposed to provide, and same type of applications can share the same CC in different rules. If CC1, CC2 and CC3 are reported in a merged list, the PCF can not identify what rule has been enforced, rule 1 and rule 2 or rule 1 and rule 3. Instead if the CC combination of each rule is different from other rules, and the report include all CC values per rule, PCF can identify the rule being enforced based on the combination of CCs in the report. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the UE need to report all the CCs in each enforced rule; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Clarifying the details of the activation of URSP rule enforcement reporting. Separating the description about whether and how to send the URSP rule enforcement report. Alignment of description and some rewording to improve readability. This CR makes CR#1120 (approved in S2-2401567 at SA2#160AHe) obsolete. |  |  |  |  |  |  |  |  |

**** First Change ****

#### 6.6.2.4	Support of URSP rule enforcement reporting

The UE can report URSP rule enforcement to PCF, so that PCF can be made aware when a given UE enforces specific URSP rule(s), and may trigger suitable actions upon the reception of such reporting.

In order to activate the URSP rule enforcement for a URSP rule containing Connection Capabilities in the Traffic descriptor (see clause 6.6.2.1), for a UE that has indicated the capability of reporting URSP rule enforcement to network (see clause 4.2.2.2.2 of TS 23.502 [3]), the PCF sets the Indication for reporting URSP rule enforcement in a URSP rule sent to the UE (see clause 6.6.2.1).

NOTE 1:	A rule with the "match-all" Traffic descriptor cannot contain Connection Capabilities in the Traffic descriptor. The format and values of the Traffic descriptor component type identifier are defined in clause 5.2 of TS 24.526 [19].

A UE supporting URSP rule enforcement reporting shall send a URSP rule enforcement report to the SMF if at the time the UE associates a newly detected application to a PDU Session (based on URSP evaluation result, see clause 6.6.2.3), the URSP rule includes an Indication for reporting URSP rule enforcement as well as Connection Capabilities in the Traffic descriptor (see clause 6.6.2.1). The UE shall send a URSP rule enforcement report, i.e. all Connection Capabilities contained in the Traffic descriptor of the associated URSP rule, to the SMF when:

-	the UE associates a newly detected application to a new PDU Session, by including the URSP rule enforcement report in the PDU Session Establishment Request (see clause 4.3.2.2.1 of TS 23.502 [3]), or

-	the UE associates a newly detected application to an existing PDU Session, by sending a PDU Session Modification Request (see clause 4.3.3.2 of TS 23.502 [3]) including the URSP rule enforcement report.

NOTE 2:	UE reporting of URSP rule enforcement can increase the amount of signalling in the network. Use of this feature is recommended to be restricted to URSP rules for specific application traffic on specific UEs based on the deployment choices of the operator.

If the UE enforces several URSP rules for multiple applications and these multiple applications are all associated to the same PDU Session, in order to reduce the number of uplink NAS messages, the UE may include more than one URSP rule enforcement report in one PDU Session Modification Request (see clause 4.3.3.2 of TS 23.502 [3]) or in a PDU Session Establishment Request (see clause 4.3.2.2.1 of TS 23.502 [3]), and each URSP rule enforcement report includes all Connection Capabilities contained in the Traffic descriptor of the associated URSP rule.

NOTE 2:	A rule with the "match-all" traffic descriptor cannot contain Connection Capabilities in the Traffic descriptor. The format and values of the Traffic descriptor component type identifier are defined in clause 5.2 of TS 24.526 [19].

The PCF receives URSP rule enforcement reports for a given UE via the Policy Control Request Trigger "UE reporting Connection Capabilities from associated URSP rule" (see clause 6.1.3.5).

When the PCF serving the PDU session is not the same as the PCF serving the UE, the PCF serving the UE subscribes to the PCF serving the PDU Session to receive the URSP rule enforcement report for the UE via PCF event reporting (see clause 6.1.3.18 and the related procedure in clause 4.16.16 of TS 23.502 [3]).

For LBO roaming session case, the H-PCF for the UE subscribes to the V-PCF to receive the URSP rule enforcement report for the UE via PCF event reporting (see clause 6.1.3.18) during the UE Policy Association Establishment or Modification.

The PCF for the UE may check whether the URSP rule enforcement report and the related PDU Session parameters (e.g. DNN/S-NSSAI) are compliant to the corresponding URSP rule of the UE. If the PCF for the UE found an inconsistency, the PCF for the UE may perform appropriate actions (e.g. initiating slice replacement procedure).

Policy control decisions based on awareness of URSP rule enforcement are described in clause 6.1.6.

**** End of Changes ****