3GPP TSG SA WG2 Meeting #161	 S2-2402885

Athens, Greece, February 26 – March 1, 2024 											(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5352 | Rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Allowed NSSAI and NSAC for Network slice with NS-AoS not matching deployed TA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Note 2 in clause 5.15.18.2 warns of NSAC problem when using the approach currently agreed in the TS 23.501 NOTE 2: Since the S-NSSAI location availability information is not a used as a trigger for the UE to perform MRU due to mobility, i.e. the UE performs MRU due to mobility upon changing to a new TA outside the UE's Registration Area, the S-NSSAI remains registered and is included in the Allowed NSSAI when the UE exits the NS-AoS. If the S-NSSAI is subject for NSAC, the S-NSSAI is counted towards NSAC as described in clause 5.15.11 also when the UE is outside the NS-AoS. This CR addresses this issue by introducing a different behaviour for allowed NSSAI than partially allowed NSSAI. See also the discussion paper in S2-2402889 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Differntiate the handling of Allowed and Partially Allowed NSSAI and allow indication or a S-NSSAI in the Partially Allowed NSSAI if the AMF needs not know the exit or UE from NS-AoS of a S-NSSAI in CM-Idle mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to properly operate NSAC when NS-AoS is smaller than deployed TA. So either TA need to be reconfigured to support NSAC, or NSAC cannot be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.18.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

More CHANGES

#### 5.15.18.2	S-NSSAI location availability information handling for supporting UEs

S-NSSAI location availability information defines additional restrictions to the usage of an S-NSSAI in TAs where the Network Slice availability does not match the TA boundaries. The AMF is configured per S-NSSAI whether to send the S-NSSAI location availability information to supporting UEs.

The S-NSSAI location availability information sent to the UE includes, for each applicable S-NSSAI of the Configured NSSAI, Location information indicating the cells of TAs in the RA where the related S-NSSAI is available if the S-NSSAI is not available in all the cells of the TA.

If the UE has indicated that the UE supports S-NSSAI location availability information in the 5GMM Core Network Capability (see clause 5.4.4a), the AMF may, based on OAM configuration, configure the UE with S-NSSAI location availability information for one or more S-NSSAIs when the AMF allocates an RA where the Network Slice availability does not match whole TAs, by including the S-NSSAI location availability information in the Registration Accept message or the UE Configuration Command message. A UE that receives S-NSSAI location availability information applies the information as follows.

1.	If the S-NSSAI is rejected in the RA or rejected partially in the RA or rejected with a cause code that allows attempting to register the S-NSSAI again, the UE can request the S-NSSAI only if the S-NSSAI location availability information indicates that the S-NSSAI is available at the cell where the UE is camping.

2.	If the S-NSSAI is in the Partially Allowed NSSAI and the UE is in a cell within the RA but outside the NS-AoS of the S-NSSAI, the following applies:

a.	The UE shall not activate User Plane resources for any already established PDU Session with that S-NSSAI.

b.	The UE shall not send user data as payload of a NAS message (see clause 5.31.4.1) in uplink direction.

c.	For an already established PDU Session, the UE and the SMF may initiate signalling for PDU Session release procedure or PDU Session modification procedure (i.e. for PS data off status change reporting).

3.	If the S-NSSAI is in the Partially Allowed NSSAI, and the UE moves to a cell inside the RA but outside the NS-AoS of the S-NSSAI, the UE shall not perform a Mobility Registration Update. If the S-NSSAI is in the Partially Allowed NSSAI and the UE in CM-CONNECTED state moves from a cell inside the NS-AoS to a cell outside the NS-AoS and the User Plane resources are active for a PDU Session on that S-NSSAI, the NG-RAN releases the User Plane resources as described in step 1d of clause 4.3.4.2 of TS 23.502 [3] and, based on the received cause code included by the gNB when it releases the PDU session resources, the SMF deactivates the PDU Session as described in clause 3a of clause 4.3.4.2 of TS 23.502 [3]. .

4.	If the S-NSSAI is in the Allowed NSSAI, and the UE moves to a cell inside the RA but outside the NS-AoS of the S-NSSAI, the UE shall perform a Mobility Registration Update. If the UE is in CM-CONNECTED mode, the NG-RAN releases the User Plane resources of the PDU sessions associated with the S-NSSAI as described in step 1d of TS 23.502 [3] and the SMF, based on the received cause code included by the gNB when it releases the PDU session resources, releases the PDU session as described in clause 3a of clause 4.3.4.2 of TS 23.502 [3].

If the S-NSSAI is subjected to NSAC, then the AMF shall restrict the RA so that the S-NSSAI is supported in all the TAs of the RA and includes the S-NSSAI in the Allowed NSSAI. If a S-NSSAI is not subject to NSAC, the AMF may include the S-NSSAI in a Partially Allowed NSSAI (even if the S-NSSAI is supported in all the TAs of the RA) if the AMF needs not know when the UE exits the S-NSSAI NS-AoS in when the UE is in CM-IDLE mode.

NOTE 1:	By Radio Resource Management and existing mechanisms in NG-RAN, handover can be used to keep the UE in the NS-AoS or steer the UE to enter the NS-AoS as long as radio conditions allow it.

NOTE 2:	Since the S-NSSAI location availability information is not a used as a trigger for the UE to perform MRU due to mobility, i.e. the UE performs MRU due to mobility upon changing to a new TA outside the UE's Registration Area, the S-NSSAI remains registered and is included in the Allowed NSSAI when the UE exits the NS-AoS. If the S-NSSAI is subject for NSAC, the S-NSSAI is counted towards NSAC as described in clause 5.15.11 also when the UE is outside the NS-AoS.

End of CHANGES