3GPP TSG-SA WG2 Meeting #S2-161	S2-2402928

Athens, Greece, Feburary 26  –  March 1, 2024

Source: 	Samsung

Title: 	Solution for KI#9: NW exposure of HO for L4S enabled QoS flow

Document for: 	Approval

Agenda Item: 	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract of the contribution: This contribution proposes a new solution for NW exposure of HO and ECN. (related to KI #9).

- Introduction

Per KI #9, we study how to enhance network exposure mechanism to better support the network information/capability exposed to the application layer. In particular, the key issue includes the following aspects:

-	Whether and how XR related network capability/information exposure towards the application layer needs to be enhanced.

2 Proposal

It is proposed to add the following contents to TR 23.700-70.

# * * * * First change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #9> |  |
| #xx | O |  |
| #2 |  |  |

# * * * * 2nd change(new text) * * * *

6.X	Solution #X: NW exposure of HO event for L4S enabled QoS flow

6.X.1	Key Issue mapping

Editor's note:	This clause lists the key issue(s) addressed by this solution.

The solution addresses Key Issue #9(e.g. Enhancement for XR related network information exposure).

6.X.2	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Sub-clause(s) may be added to capture details.

When a QoS flow is enabled for L4S mechanism, for the downlink traffic NG-RAN congestion status is detected by UE and UE provides the corresponding congestion feedback to AS.

Right after handover, UE may send some congestion feedback to AS based on the congestion information of the previous serving NG-RAN. So that such congestion feedback may provide wrong information to AS.

In order to prevent AS’s mis-interpretation of 5GS status, 5GS should inform the HO event and let the AS neglect the congestion feedback right after handover.

6.X.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

The Figure 6.x.2-1 shows the procedure for HO event notification for the L4S enabled QoS flow:

Figure 6.x.2-1: Procedure for HO event notification for the L4S enabled QoS flow

The steps of Figure 6.x.2-1 are described as follows:

Step 1: An AF requests HO notification for L4S enabled media flow by invoking the Nnef_AFSessionWithQoS Create service operation as described in clause 4.15.6.6 of 3GPP TS 23.502, which forwarded to PCF.

Step 2: The PCF creates PCC rules that includes HO notification for L4S enabled flow, and sends it to the SMF if there is a corresponding PDU Session.

NOTE: In case of PDU Session establishment procedure, the SMF receiving the PDU Session establishment request from UE fetchs SM policy from the PCF

Step 3: Based on the PCC rules received from the PCF, SMF generates/updates the QoS profile to request NG-RAN to notify the HO event for the L4S enabled QoS flow.

Step 4/5: so the QoS flow is enabled L4S congestion reporting. Then HO occurs.

Step 6/7: On HO completion, target NG-RAN provides notification(“HO just before”, invalid ECN feedback) over GTP-U to UPF and UPF provides the notificaiton to AF/AS via Nupf_EventExposure_notify.

Step 8: Application server neglect the ECN feedback arrived in a configured short period in order not to mis-intepree the 5GS condition.

6.X.4	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

AF/AS:

-	AF requests HO notification for L4S enabled media flow.

-  AS neglects ECN feedback, if any, arrived in a short period after receiving the notification.

UPF:

-	provides the notification to AF/AS based on NG-RAN’s notification over GTP-U.

NG-RAN:

-	provides notification(“HO just before”, invalid ECN feedback) over GTP-U to UPF on HO completion.

# * * * * End of changes * * * *