3GPP TSG-WG SA2 Meeting #161 	S2-2403014

# 26 Feb – 01 Mar 2024, Athens, Greece	     	       (revision of S2-2403014)

Source:	Google

Title:	(KI#2) Solution for handling Feeder link Unavailability event

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_Ph3 / Rel-19

Abstract: This paper proposes a new Sol#X for KI#1 to resolve open issues for handling feederlink unavailability event for TR23.700-29.

# 1. Discussion

This PCR proposes a new solution for Key Issue #1: to resolve the open issues for feeder link unavailability event.

When a RAN node (eNB or gNB) on boarding regenerative based satellite (denoted as RAN-Sat) flies from service area A associated to ES1 to service area B associated to ES2, the feeder link of the satellite with NTN gateway on the ground may be unavailable while the UE is still with service link. Due to the change of service areas, the serving AMF/MME on the ground may be different after feeder link unavailability event is finished. In this case, the UE needs to perform registration request for mobility registration update.

To allow the UE preparing for such service interruption, the UEs accessing regenerative based satellite needs to be aware of such feeder link unavailability event especially when it would result in AMF/MME changes.

# 2. Proposal

It is proposed to capture the following changes vs. TR 23.700-29.

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |  |
| 1 | X |  |  |  |
| 2 | X |  |  |  |
| 3 | X |  |  |  |
| 4 | X |  |  |  |
| 5 | X |  |  |  |
| 6 | X |  |  |  |
| 7 | X |  |  |  |
| 8 | X |  |  |  |
| 9 | X |  |  |  |
| 10 | X |  |  |  |
| 11 |  | X |  |  |
| 12 |  | X |  |  |
| 13 |  | X |  |  |
| 14 |  | X |  |  |
| 15 |  | X |  |  |
| 16 |  | X |  |  |
| 17 |  | X |  |  |
| 18 |  | X |  |  |
| 19 |  | X |  |  |
| 20 |  | X |  |  |
| 21 |  | X |  |  |
| 22 |  | X |  |  |
| 23 |  | X |  |  |
| 24 |  | X |  |  |
| 25 |  | X |  |  |
| 26 |  | X |  |  |
| 27 |  | X |  |  |
| 28 |  |  | X |  |
| 29 |  |  | X |  |
| 30 |  |  | X |  |
| 31 |  |  | X |  |
| 32 |  |  | X |  |
| 33 |  |  | X |  |
| X |  | X |  |  |

# * * * * Second change (All new text)* * * *

## 6.X	Solution #X: Support of Feeder link Unavailability Period

### 6.X.1	Description

When a UE accessing RAN node (eNB or gNB) on boarding regenerative based satellite (denoted as RAN-Sat) flies from service area A associated to ES1 at time T1 to service area B associated to ES2 at time T3, the feeder link of the satellite with NTN gateway on the ground is unavailable for a period of time, in which the feeder link may need to switchover from ES1 to ES2 as in Figure 6.X.1-1. The following 5GS/EPS enhancement are needed for handling feeder link unavailability event.

- During RAN-Sat mobility, the UE having service link with RAN-Sat may suffer service interruption due to several reasons, e.g. RAN-Sat does not support S&F operation mode, UE does not need delay tolerant services provided by RAN-Sat in S&F mode, etc.

- For transparent payload-based satellite access, the UE enters CM-idle state due to unavailable service link when moving out of coverage. However, for regenerative payload-based satellite access, feeder link unavailability event may occur in RAN-Sat access coverage, the UE with available service link would be in CM-connected state during feeder link unavailability event.

- Due to the change of service areas, the serving AMF/MME on the ground may be different after regaining feeder link or feeder link switchover is completed. In this case, the UE needs to perform registration request for mobility registration update.

Different from the transparent payload-based satellite, for a RAN node on boarding the regenerative based satellite, the TNL association over the feeder link would result in longer delay to transfer RAN-Sat configuration from an old AMF/MME to a target AMF/MME if the AMF/MME changes is needed due to the change of the services areas. To allow the UE preparing for such service interruption, e.g. requesting Unavailability Period Duration for saving UE power consumption, the UEs accessing regenerative based satellite needs to be aware of such feeder link unavailability event especially when it would result in AMF/MME changes.

Editor’s Note: It is FFS whether feeder link switchover event would result in service interruption due to long delay for TNL association over the feeder link to transfer RAN-Sat configuration from an old AMF/MME to a target AMF/MME if the AMF/MME changes is needed due to the change of the services areas.

Figure 6.X.1-1: Feeder link Uavailablile Event for UE having service link with RAN on boarding satellite mobility

The principles of this solution are applied to EPS/5GS as follows:

- The UE can obtain feeder link unavailability event information from RAN-Sat via SIB info, local stored satellite information, or AMF. Accordingly, either the UE or the AMF determines if feeder link unavailability event is about to happen.

- The AMF determines whether the change of the AMF/MME is needed after feeder link unavailability event and indicates to the UE if registration request procedure for mobility registration update is need after feeder link unavailability is finished.

  - For AMF aware feeder link unavailability event, the indication can be sent via UE Configuration Update command message including Configuration Update indication.

  - For UE aware feeder link unavailability event, the indication can be sent via Registration Accept message including a mobility update indication for feeder link unavailability event in respond to the Registration Request message from the UE.

- Enhance the support of discontinuous network coverage for satellite access in TS23.501 clause 5.4.13 and the support of Unavailability Period as indicated in TS23.501 clause 5.4.1.4 for supporting feeder link unavailability event as follows:

  - For Mobility management and power saving optimization, see TS 23.501 clause 5.4.13.1.

  - The feeder link unavailability event results in service interruption that includes FL unavailability period duration and the potential FL switchover delay for FL connection management, e.g. TNL association and NGAP interface changes with the old AMF via a first feeder link, and with a target AMF via a second feeder link.

  - A UE indicating support for "Unavailability Period Support" shall support the procedures for feeder link unavailability event.

  - Both the UE and the network indicate support of unavailability period due to discontinuous coverage and/or feeder link unavailability, the UE and AMF apply the following:

    - Unavailability Period Duration is set based on FL unavailability period duration if known,

    - Start of the Unavailability Period is set based on start time of the FL unavailability event if known, and

    - Unavailability Type indicates FL unavailability.

  - For Overload control, see TS 23.501 clause 5.4.13.5. The AMF can determine maximum offset time and provide it to the serving UEs to distribute NAS signalings from/to all serving UEs.

  - For AMF aware feeder link unavailability event, the maximum offset time can be sent via UE Configuration Update command message.

  - For UE aware feeder link unavailability event, the maximum offset time can be sent via Registration Accept message in respond to the Registration Request message from the UE or via SIB info broadcast by the RAN-Sat.

### 6.X.2	Procedures

Figure 6.X.2-1 describes the procedures for Feeder link unavailability with AMF/MME changes.

Figure 6.X.2.1-1: Feeder link unavailability with AMF changes

- The UE receives feeder link unavailability event info from RAN-Sat via SIB info.

The UE determines if FL unavailability event is about to happen based on broadcast SIB info from the RAN-Sat or local configuration.

- The UE sends Registration Request message to old AMF to negotiates FL unavailability event parameters, e.g. feeder link unavailability period duration and/or start time of FL unavailability event, and an indication for triggering re-registration request procedure via NAS messages for enabling UE’s power saving mechanism and keeping UE registered during FL unavailability event in ongoing.

- The RAN-Sat provides information, e.g. update TAI list after FL unavailability event to the old AMF.

T

- he UE receives Registration Accept message including FL unavailability period duration, start of the FL unavailability period, and optional indication for re-registration after FL unavailability event is finished from the old AMF. The RAN-Sat may receive CN assistance information to manage UE’s state during FL unavailability event from the old AMF.

- The UE enters CM-idle state and starts a FL unavailability Period duration timer at the start time of the FL unavailability event in Step 5a; In Step 5b, during feeder link unavailability event, the RAN-Sat may operate in S&F operation mode and then perform FL connection management after FL becomes available.

- After the FL unavailability event is finished, e.g. based on the expiry of the FL period duration timer, the UE sends Registration Request message to the target AMF if a registration indication is received in Step 4.

- This step is the same as TS23.502 Figure 4.2.2.2.2-1, step 4-19. Specifically, the target AMF retrieves UE context from the old AMF via Namf_Communication_UEContextTransfer response message in respond to the Namf_Communication_UEContextTransfer message.

- The AMF sends Registration Accept message to the UE in responding to Step 6.

- The UE initiates Service Request message to re-establish PDU Session(s).

### 6.X.3	Impacts to Services, Entities and Interfaces

The following impacts are foreseen by this solution:

AMF/MME:

-  Determine whether the change of the AMF/MME is needed after feeder link unavailability event and indicates to the UE if re-registration request procedure is need after feeder link unavailability event is completed.

-	Enhance support of discontinuous network coverage for satellite access and the support of Unavailability Period for feeder link unavailability event.

-	provide CN assistance information to RAN-Sat for handling UE’s state during FL unavailability event

RAN-Sat (gNB or eNB):

-	broadcast SIB info regarding feeder link unavailability event timers and parameters.

-	handle UE’s state during FL unavailability event based on CN assistance information from the AMF

UE:

-	Determine whether feeder link availability event is about to happen

-	Negotiate feeder link availability event timers/parameters with the AMF/MME.

-	Enhance support of discontinuous network coverage for satellite access and the support of Unavailability Period for feeder link unavailability event.

# * * * *End change * * * *