SA WG2 Meeting #161	S2-2403201

February 26th – March 1st, 2024, Athens, Greece                 (Revision of S2-2401941)

Source:	OPPO, Xidian University

Title:	FS_UIA_ARC KI#2 New solution for user id authentication

Document for:	Approval

Agenda Item: 	19.8

Work Item / Release:	FS_UIA_ARC /Rel-19

Abstract of the contribution: This paper proposes a new solution for user id authentication for the FS_UIA_ARC TR 23.700-32.

1	Discussion

This paper proposes user id authentication addressing KI#2.

2	Proposal

It is proposed to include the following changes in TR 23.700-32 V0.1.0.

* * * * Start of Changes (All text new) * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> |
| #1 |  | x |
| #2 |  |  |

6.X	Solution #X: solution for user id authentication

6.X.1	Key Issue mapping

6.X.2	Description

The solution has the following assumptions:

-  One SUPI is corresponding to one or more User IDs. The value of User IDs has been negotiated and preconfigured in UE, Core network, and AAA-Server.

-  It is assumed each node performs the following functions for User-ID based authentication

UE: Peer

AMF: Authenticator

AAA-Server: EAP server

-  Reusing EAP based authentication (e.g. MD5 challenge, Cogent Systems Biometrics Authentication EAP) to authenticate User-Id by 3rd party’s AAA-Server. The User-id and the authentication result can be aware by 5GC

6.X.3	Procedures

Figure-6.x.3-1 User ID based authentication after initial registration

(Steps/Parameters for potential impact is in Italic)

- User-A switch on a UE and the UE performs initial registration

- UE send initial registration including SUPI (Concealed as SUCI) and capability of User-Id based authentication

- AUSF triggers the Primary authentication to authenticate the SUPI reported by UE is correct. Specific procedure has been defined in TS 33.501

  - During the registration, AMF retrieves the SUPI’s subscription which indicates whether an User-ID based authentication is required.

NOTE 1: the other omitted messages for 5G registration follows the existing procedure.

5.  Registration accept is sent to UE with AMF allocated 5G-GUTI.

6.  AMF determines to perform User-ID based authentication by sending DL NAS message including EAP-Request (e.g. type=1, identity)

7.  UE sends UL NAS message including EAP-Response (e.g. type=1, identity=user id)

NOTE 2: The step 6 and 7 can be combined into one message

8.  AMF sends AAA Protocol message including GPSI and EAP-Response: identity=user id

9.  More interactions between EAP server and UE may be needed for authentication. E.g., triggers MD5-Challenge to authenticate the user id

10. EAP Server responds authentication result of the user id (success/failure)

11. AMF record the authentication result and send the result to UE

12. AMF sends the authenticated User ID to UDM for storing

Editor’s Note: It needs be further clarified the roaming case and the scenario when User Id is changed

6.X.4	Impacts on services, entities and interfaces

UE

Send capability of User-Id authentication in registration request

Support EAP peer and send EAP message via NAS

AMF

Support EAP authenticator

Support determination of performing User-id authentication, sending EAP message via NAS

UDM

SUPI’s Subscription indicates whether User-Id authentication is required

Store the authenticated User ID

* * * * End of Changes * * * *