3GPP TSG-WG SA2 Meeting #161	S2-2403442

Athens, GR, February 26 – March 1, 2024	(revision of S2-2403156_was S2-2402028r01)

Source:	ETRI

Title:	KI#2, New Sol: Direct subscription of UPF event exposure service for TSC management

Document for:	Approval

Agenda Item:	19.11

Work Item / Release:	FS_UPEAS_Ph2 / Rel-19

Abstract: This paper proposes a solution for direct subscription of UPF event exposure service for TSC management

1. Introduction

In the approved FS_UPEAS_Ph2, Key Issues #2 is to study potential enhancements on UPF event exposure service to optimize the procedures related to UPF data collection.

-  Identify the specific use case and scenarios that require optimizing the procedures related to UPF data collection with enhancements on UPF direct or indirect subscription;

This proposal provides a solution for Direct subscription of UPF event exposure service for TSC management . In Rel-18, UPF event exposure service for TSC management adopted indirect subscription and direct notification for the service as shown Figure 1.

TSN AF or TSCTSF indirectly subscribes UPF event exposure service for TSC management information via PCF and SMF using PCF services and N4 interface. UPF directly reports TSC management information event via Nupf_EventExposure_Notify to the TSN AF or TSCTSF.

This indirect subscription is not optimized. Intermediate nodes (PCF and SMF) only transfer the information transparently using different interfaces (i.e. SBI and N4).

Figure 1: Current indirect subscription and direct notification for UPF event exposure service for TSC management

Therefore, this paper proposes a new solution for direct subscription of UPF event exposure service for TSC management.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-63.

# * * * * Start of change * * * *(all new text)

6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> |
| #x |  | X |

## 6.X	Solution #X: Direct subscription of UPF event exposure service for TSC management

6.X.1	Description

This solution is applicable for a case the UPF employs the NW-TT functionality. According to the latest TS 23.501 specification, the assumptions are as follows.

- 5GS TSN bridge has a single NW-TT entity within UPF.

- Bridge ID is to distinguish between bridge instances within 5GS

- To support TSN, the user plane node ID is Bridge ID.

- To support integration with IETF DetNet, the user plane node ID can be Router ID. The user plane node ID may be pre-configured in the UPF based on deployment.

- TSN AF or TSCTSF has and stores the Bridge ID or Router ID and its address before subscription of UPF event exposure service for TSC management.

Based on the assumption, TSN AF or TSCTSF can directly subscribe UPF event exposure service to exchange TSC management information (i.e. UMIC and/or PMIC).

6.X.2	Procedures

6.X.2.1	Direct subscription of UPF Event Exposure service for TSC management

Figure 6.x.2.1: Direct subscription of UPF Event Exposure service for TSC management

1. TSN AF or TSCTSF gets user plane node ID as Bridge ID or Router ID through PDU session establishment procedure.

2. TSN AF or TSCTSF gets UMIC for the address of user plane node ID through PDU session modification procedure via PCF and SMF.

3. TSN AF or TSCTSF stores user plane node ID and its address.

4.	The consumer NF directly subscribes the UPF event exposure service for TSC management or time synchronization and deterministic networking between TSN AF or TSCTSF and NW-TT via Nupf_EventExposure_Subscribe service operation.

5.	When the subscribed event is occurred, the UPF sends the collected UPF data (i.e., TSC management information) via Nupf_EventExposure_Notify service operation to the consumer NF.

Editor’s Note: The impact of this procedure to the existing procedures of TSN Bridge information exchange and management is FFS.

6.X.3	Impacts on services, entities and interfaces

TSN AF or TSCTSF:

-  Supports direct subscription of UPF event exposure services for TSC management (i.e., Event ID).

UPF:

-	Supports direct subscription of event exposure services for TSC management (i.e., Event ID).

# * * * * End of change * * * *