3GPP TSG-SA2 Meeting #161	S2-2403511

26 Feb-01 Mar 2024, Athens, Greece	(revision of S2-2402087)

# Source:	China Telecom, SK Telecom

# Title:	New WID: TEI19_NetFailTol; Enhancing Network Failure Tolerance

# Document for:	Information

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Enhancing Network Failure Tolerance

Acronym: TEI19_NetFailTol

Unique identifier:

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | x |  |
| No | x | x | x |  | x |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

3	Justification

Scenario 1: Enabling non-roaming UEs to obtain service from the registered PLMN when the UDM/UDR in the registered PLMN fails. The rest of the core network is functional.

In the case of UDM/UDR failure, UDM/UDR related services become not available.

In step 4 of PDU session establishment procedures in clause 4.3.2.1, the SMF shall retrieve Session Management Subscription data from UDM when the SMF does not have it for requested SUPI, DNN and S-NSSAI (e.g. when non-roaming UEs previously authenticated have not established any PDU sessions since registration, or they request a new PDU Session, or they perform EPS to 5GS handover without N26 interface.)

It is proposed that, when Nudm_SDM_Get service is not available, the SMF may use local configuration to complete the PDU Session Establishment procedures. The SMF may determine the local configuration based on the PLMN of the SUPI, DNN and S-NSSAI received from the AMF to consider UE’s characteristics.

In step 7b, the SMF may perform an SM Policy Association Establishment procedure and can use the predefined PCC rules configured in the SMF. It is proposed to allow the PCF to use the local configuration instead of PDU Session policy control data in UDR for the non-roaming UEs previously authenticated.

In step 16c, the SMF shall register with the UDM using Nudm_UECM_Registration for a given PDU Session. It is proposed that, when Nudm_UECM_Registration service is not available, the SMF may delay the registration.

In step 20, the SMF is already allowed not to initiate the SM Policy Association Modification.

It is proposed that, when the SMF determines that Nudm_UECM_Registration and Nudm_SDM_Get services become available, the SMF may retrieve the Session Management Subscription data and register with the UDM.

With the proposed changes, PDU session establishment procedures can be completed for the non-roaming UEs previously authenticated and basic connectivity services can be provided. MT services via UDM (e.g. LCS) cannot be provided.

4	Objective

This new WID aims to enable the following to enhance network failure tolerance.

When Nudm_UECM_Registration and Nudm_SDM_Get services are not available, the SMF may use local configuration to complete the PDU Session Establishment procedures for the non-roaming UEs previously authenticated.

When Nudm_UECM_Registration and Nudm_SDM_Get services become available, the SMF may perform registration with the UDM and retrieve subscription data for the non-roaming UEs previously authenticated.

When Nudr_DM_Query and Nudr_DM_Subscribe services are not available, the PCF may use the local configuration instead of PDU Session policy control data in UDR for the non-roaming UEs previously authenticated.

NOTE 1:	UEs do not perform roaming and continue to receive service from the registered PLMN with UDM/UDR failure.

NOTE 2:	The serving AMF does not change or with intra-AMF mobility, the serving AMF has registered to the UDM before the failure occurs and holds subscription data for UEs.

The expected time for completing this work is 0.5 TUs.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.502 | - Allow the SMF to complete the PDU Session Establishment procedures for the non-roaming UEs previously authenticated, when Nudm_UECM_Registration and Nudm_SDM_Get services are not available - Allow the SMF to perform registration with the UDM and retrieve subscription data when Nudm_UECM_Registration and Nudm_SDM_Get services become available | SA#106 (Dec 2024) |  |

6	Work item Rapporteur(s)

Yinglin Chen, China Telecom, chenyl37@chinatelecom.cn

7	Work item leadership

SA2

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

China Telecom

ZTE

CATT

Inspur

vivo

Huawei

Rakuten Mobile

SK Telecom

China Unicom