SA WG2 Meeting #161	S2-2401940

February 26th – March 1st, 2024, Athens, Greece

Source:	OPPO

Title:	FS_MASSS KI#x New solution for policy enhancement for DualSteer

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS /Rel-19

Abstract of the contribution: This paper proposes a new solution for new policy enhancement for the FS_MASSS TR 23.700-54.

1	Discussion

This paper proposes for policy enhancement for DualSteer addressing KI#x Policy Enhancement.

The following considerations apply to the solution

- Each UE has two USIMs for 3GPP accesses i.e. DualSteer device. The two USIMs belongs to the same MNO.

For DualSteer scenario, usually the VPLMN and HPLMN coverage coexist in a certain area. And for the DualSteer device (with two USIMs), both USIMs belong to the same HPLMN.

For DualSteer scenario, usually the VPLMN and HPLMN coverage coexist in a certain area. And for the DualSteer device (a DualSteer device with two USIMs), both USIMs belong to the same HPLMN. The DualSteer device needs to

1) let one USIM register directly to HPLMN, another USIM register to VPLMN and perform Home Routing to HPLMN; or

2) let one USIM register to a RAT of HPLMN, another USIM register to another RAT of HPLMN; or

3) let one USIM register to 5GS HPLMN, another USIM register to EPS HPLMN.

How to realize the scenarios (1, 2, 3) above belongs to KI#x registration

2)  The UE policy coordination point is in H-PCF, the H-PCF can provision the URSP via 5G and/or EPC following the mechanism defined in R18 eUEPO.

3)  The UE policy enhancement is used only for selection of 3GPP access, which has been established by the UE when finishing PLMN selection and registration. In other words, the UE policy enhancement does not affect existing PLMN selection and registration mechanism.

4)  The URSP policy has been used to steer the traffic in different PDU session, however it has the following limitation which cannot fully fulfil the DualSteer requirements:

-  It is used within a certain USIM when the USIM has finished the PLMN selection and Registration, but in DualSteer case, both of the two USIM may have the URSP rules which matches a certain traffic.

-  The current URSP rule only indicates Access Type preference per 3GPP or non-3GPP access, finer granularity (to differentiate the RAT and core network scenarios mentioned in 2) needs to be considered.

Therefore, the UE policy needs be enhanced to address the aspects above. Considering the backward compatibility and the limitation of current URSP evaluation mechanism, it is proposed to introduce a separate UE policy which is used in combination with the current URSP rule.

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.1.0.

* * * * Start of Changes (All text new) * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> | <Key Issue #y> | …… |
| #x |  |  | “Policy Enhancement” |  |
| #2 |  |  |  |  |

6.X	Solution #X: Access Selection Policy for selecting a registered UE for traffic transmission

6.X.1	Key Issue mapping

Editor's Note:	This clause lists the key issue(s) addressed by this solution.

6.X.2	Description

Editor's Note: This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Sub-clause(s) may be added to capture details.

The following considerations apply to the solution:

1)  Each UE has two USIMs for 3GPP accesses i.e. DualSteer device. The two USIMs belongs to the same MNO.

For DualSteer scenario, usually the VPLMN and HPLMN coverage coexist in a certain area. And for the DualSteer device (a DualSteer device with two USIMs), both USIMs belong to the same HPLMN.

For DualSteer scenario, usually the VPLMN and HPLMN coverage coexist in a certain area. And for the DualSteer device (a UE with two USIMs), both USIMs belong to the same HPLMN. The DualSteer device needs to

1) let one USIM register directly to HPLMN, another USIM register to VPLMN and perform Home Routing to HPLMN; or

2) let one USIM register to a RAT of HPLMN, another USIM register to another RAT of the HPLMN; or

3) let one USIM register to 5GS HPLMN, another USIM register to EPS HPLMN.

How to realize the scenarios (1, 2, 3) above belongs to Key Issue #1.2: Registration and mobility management for DualSteer.

2)  The UE policy coordination point is in H-PCF, the H-PCF can provision the URSP via 5G and/or EPC following the mechanism defined in R18 eUEPO.

3)  The UE policy enhancement is used only for selection of 3GPP access, which has been established by the UE when finishing PLMN selection and registration. In other words, the UE policy enhancement does not affect existing PLMN selection and registration mechanism.

4)  The URSP policy has been used to steer the traffic in different PDU session, however it has the following limitation which cannot fully fulfil the DualSteer requirements:

-  It is used within a certain USIM when the USIM has finished the PLMN selection and Registration, but in DualSteer case, both of the two USIM may have the URSP rules which matches a certain traffic.

-  The current URSP rule only indicates Access Type preference per 3GPP or non-3GPP access, finer granularity (to differentiate the RAT and core network scenarios mentioned in 2) needs to be considered.

Therefore, the UE policy needs be enhanced to address the aspects above. Considering the backward compatibility and the limitation of current URSP evaluation mechanism, it is proposed to introduce a separate UE policy (named ASP, Access Selection Policy) which is used in combination with the URSP policy.

6.x.2.1 New ASP policy for selecting a registered PLMN for DualSteer traffic transmission

A new UE policy called ASP (Access Selection Policy) is introduced, in which the ASP includes the following aspects:

Each ASP rule has a Traffic descriptor: identify the application traffic. The format and value can be the same as the TD in URSP rule. Then, under each traffic descriptor, it has

-  Rule precedence: UE evaluate the ASP rules accordingly

-  PLMN selection preference for the access: indicate the PLMN ID of a 3GPP access via which the application traffic should be transmitted. This entry can have one or more values, UE need to select one of them (no priority list);

An example ASP is as following, when an application traffic appears, it firstly evaluates which Traffic descriptor it can be matched, and then select an PLMN ID for which the UE has registered, if there is no registered PLMN ID in this entry, the UE will further evaluate the next matched traffic descriptor and so on. Then the UE will use the URSP rules corresponding to the PLMN ID

Table ASP policy

| Traffic descriptor | Rule precedence | PLMN preference for the access |
| --- | --- | --- |
| TD-a | 1 | PLMN-1, PLMN-2 |
| TD-b | 2 | PLMN-3 |
| TD-c | 3 | PLMN-2, PLMN-3 |
| …. | … |  |

6.x.2.2 Extension of URSP rules by adding RAT and CN preference

The extension of URSP rule has been indicated as below. In RSD, it adds the description of RAT and CN preference (in red).

Table 1: UE Route Selection Policy Rule

| Information name | Description | Category | PCF permitted to modify in a UE context | Scope |
| --- | --- | --- | --- | --- |
| Rule Precedence | Determines the order the URSP rule is enforced in the UE. | Mandatory (NOTE 1) | Yes | UE context |
| Traffic descriptor | This part defines the traffic descriptors for the policy |  |  |  |
| Application identifiers | Application identifier(s) | Optional | Yes | UE context |
| IP descriptors | IP 3 tuple(s) (destination IP address or IPv6 network prefix, destination port number, protocol ID of the protocol above IP) | Optional | Yes | UE context |
| Non-IP descriptors | Descriptor(s) for non-IP traffic | Optional | Yes | UE context |
| …… |  |  |  |  |
| List of Route Selection Descriptors | A list of Route Selection Descriptors. The components of a Route Selection Descriptor are described in table 6.6.2-2. | Mandatory |  |  |
| …… |  |  |  |  |

Table 2: Route Selection Descriptor

| Information name | Description | Category | PCF permitted to modify in a UE context | Scope |
| --- | --- | --- | --- | --- |
| Route Selection Descriptor Precedence | Determines the order in which the Route Selection Descriptors are to be applied. | Mandatory (NOTE 1) | Yes | UE context |
| Route selection components | This part defines the route selection components | Mandatory (NOTE 2) |  |  |
| SSC Mode Selection | One single value of SSC mode. | Optional | Yes | UE context |
| Network Slice Selection | Either one single value or a list of values of S-NSSAI(s). | Optional | Yes | UE context |
| DNN Selection | Either one single value or a list of values of DNN(s). | Optional | Yes | UE context |
| Non-seamless Offload indication | Indicates if the traffic of the matching application is to be offloaded to non-3GPP access outside of a PDU Session. | Optional (NOTE 3) | Yes | UE context |
| Access Type preference | Indicates the preferred Access Type (3GPP or non-3GPP) when the UE establishes a PDU Session for the matching application. | Optional | Yes | UE context |
| 3GPP RAT and CN preference (NOTE X) | Indicates the preferred 3GPP RAT: NR-TN, NR-NTN, E-UTRA And/or Indicate the preferred CN type: 5GS、EPS |  |  |  |
| NOTE 1: Every Route Selection Descriptor in the list shall have a different precedence value. NOTE 2: At least one of the route selection components shall be present. NOTE 3: When the Subscription Information contains only one S-NSSAI in UDR, the PCF needs not provision the UE with S-NSSAI in the Network Slice Selection information. The "match all" URSP rule has one S-NSSAI at most. NOTE 4: If this indication is present in a Route Selection Descriptor, no other components shall be included in the Route Selection Descriptor. NOTE 5: The SSC Mode 3 shall only be used when the PDU Session Type is IP. NOTE 6: The Route Selection Descriptor is not considered valid unless all the provided Validation Criteria are met. NOTE 7: To support VPLMN specific URSP rules, Time Window and/or Location Criteria in the Route Selection Descriptor may contain VPLMN-specific values. NOTE 8: This component shall be present when the Route Selection Component does neither include the "Non-Seamless Offload indication" nor "ProSe Layer-3 UE-to-Network Relay Offload indication". NOTE 9: If this indication is present in a Route Selection Descriptor, ProSe Layer-3 UE-to-Network Relay Offload indication shall not be included in the Route Selection Descriptor. NOTE X：This indication is only applied when Access Type is selected as “3GPP” |  |  |  |  |

6.X.3	Procedures

Editor's Note: This clause describes high-level procedures and information flows for the solution.

Figure 6.x.3-1: procedure for UE to evaluate ASP and URSP for DualSteer (messages/behaviours with potential new impact are in italic and bold)

Step-1 and 2: The USIM-1 and USIM-2 separately perform PLMN selection and registration as defined in TS23.502 clause 4.2.2.2 and/or any new enhancement to registration in KI#1.2. The AMF and SMF are not necessarily the same for USIM-1 and USIM-2. But the PCF should be the same.

NOTE: Any new enhancement to registration belongs to KI#1.2

Editor’s Note: it is FFS how to select to the same H-PCF for USIM-1 and USIM-2 of a DualSteer device.

Step-3: PCF triggers the UCU procedure, as defined in TS23.502 clause 4.2.4.3, to provision ASP towards v (via either USIM-1 or USIM-2)

Step-4: PCF triggers the UCU procedure, as defined in TS23.502 clause 4.2.4.3, to provision URSP rules separately towards USIM-1 and USIM-2 of the DUALSTEER DEVICE.

Step-5: When a new application traffic appears (e.g. Application-a), the DUALSTEER DEVICE firstly evaluates ASP to find a matched ASP Rule and select the UE (UE-1 or UE-2) which has registered via the required PLMN ID in the matched ASP rule. If no UEs registered to the required PLMN ID, the DUALSTEER DEVICE will further check the next priority ASP rule which matched the Application traffic.

Step-6: The selected UE evaluates its URSP rules with extension of RAT/NAS preference and associates the Application traffic to an PDU session as defined in TS23.503 clause 6.6.2.3. If needed, a new PDU session will be established for the association.

Step-7: When the access of the PDU session is released or not available, the DUALSTEER DEVICE may re-evaluate the ASP and URSP as described in step-5 and 6. The re-evaluation may lead to a traffic switching between two 3GPP accesses to the PDU sessions with the same UPF anchor.

6.X.4	Impacts on services, entities and interfaces

Editor's Note: This clause captures impacts on existing 3GPP nodes and functional elements.

UE

Support the Access Selection Policy, which is used to determine which USIM/UE is used for the traffic transmission for a service.

Support the extension of URSP.

AMF

No

PCF

Support the Access Selection Policy, and the extension of URSP

Support provisioning of Access Selection Policy to UE

* * * * End of Changes * * * *