SA WG2 Meeting #161	S2-2402499

Athens, Greece, February 26 - March 1, 2024

Source:	China Mobile

Title:	New Solution on Registration procedure for Ambient IoT Devices

Document for:	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT /Rel-19

1. Introduction

This solution addresses key issue on Identification, Subscription, Registration and Connection management for Ambient IoT Devices.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13.

# * * * * First change(all new) * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | Key Issue #1 | Key Issue #2 | Key Issue #3 |
| #X | √ |  |  |

6.X	Solution #X: Registration procedure for Ambient IoT Devices

6.X.1	Description

This solution is for Key Issue #2 "Identification, Subscription, Registration and Connection management".

As depicted in Architecture Requirements, the traffic types of DT and DO-DTT will be studied in this stage. The Ambient IoT devices could be driven by network for Topology1 or UE for topology 2 before registering to the network. This proposal proposes one potential mechanism for identification, subscription, registration management and the registration procedures as well.

The principles are given as below:

- A new network function named Ambient IoT NF may be adopted to manage Ambient IoT devices and procedures. If not, this relevant function can be supported by AMF.

- In 5GC, each Ambient IoT device has the unique internal ID which consists of at least MCC, MNC and MSIN. Optionally, the group ID in terms of the client or the type of item may be contained in the device ID.

6.X.2	Procedures

6.X.2.1 Procedures for AF triggered Registration

The following figure presents a procedure of AF triggered registration for Topology 1.

Figure 6.X.2-1 AF triggered Registration Procedure for Topology 1

0a. The Ambient IoT devices may be pre-configured with internal AIoT device ID or group ID.

0b. The UDM may be pre-configured with the Ambient IoT devices profile which contains the device ID or group ID.

- AF triggers registration and sends AF Triggered Registration Request to NEF. The service information such as EPC code list, TID code list and the location information may be included.

- The NEF selects an AMF or Ambient IoT NF that supports Ambient IoT services based on the location information, and maps the location information into TA list or gNB list.

- The NEF retrieves the subscription data from UDM and map the EPC code list with the internal AIoT device ID list or the group ID based on the subscription data and TA list or gNB list.

- The NEF sends AF Triggered Registration Request to the AMF/Ambient IoT NF, including the internal AIoT device ID list, the group ID, the TA list or gNB list.

- The AMF/Ambient IoT NF selects NG-RAN based on the TA list or gNB list.

- The AMF/Ambient IoT NF activates the AIoT devices. The devices access and register to the network via NG-RAN. A receiving limit time may be configured on NG-RAN. Once timeout, the message received after this time will be discarded by NG-RAN.

- The AMF/Ambient IoT NF may interact with UDM and AUSF for Authentication/Security.

- The AMF/Ambient IoT NF creates the UE context for a device or a group of AIoT devices and returns the Response to the NEF.

- The NEF returns AF Triggered Registration Response to the AF.

6.X.2.2 Procedures for UE triggered Registration

For Topology 2, the registration may be triggered by UE which performs as a reader. The UE interact with AMF or Ambient IoT NF via NG-RAN, which could be regarded as the supplement for AF triggered registration procedure.

Editor’s Notes: The procedure for UE triggered registration is FFS.

6.X.3	Impacts on services, entities and interfaces

NEF:

- The NEF supports conversion between the internal AIoT device ID and EPC code.

AMF/Ambient IoT NF:

-The AMF/Ambient IoT NF selects NG-RAN based on the TA list or gNB list and activates the AIoT devices.

UDM:

-The UDM stores the subscription information of Ambient IoT devices in group.

NG-RAN as a reader:

-The NG-RAN performs paging and receives response of Ambient IoT devices in Topology 1.

UE as a reader:

-The UE performs paging and receives response of Ambient IoT devices in Topology 2.

-The UE supports to trigger the registration procedure.

Ambient IoT device:

-The device stores the internal AIoT device ID or the group ID.

# * * * * End of changes * * * *