SA WG2 Meeting #S2-161	S2-2402587

26 February - 1 March, 2024, Athens, Greece	(revision of S2-240xxxx)

Source:	ZTE

Title:	KI#4, resolving the EN on restricting the number of simultaneously active non-3GPP devices

Document for:	Approval

Agenda Item:	19.8

Work Item / Release:	FS_UIA_ARC / Rel-19

Abstract of the contribution: This paper proposes to remove the Editor’s Note on restricting the number of simultaneously active non-3GPP devices in the Key Issue #4.

1. Discussion

There is an EN in the Key issue 4.

Editor's note:	It is FFS whether to include "whether and how the operator restricts the number of simultaneously active non-3GPP devices per SUPI (i.e. per subscription) " in this key issue.

The KI#1/2/3 are used for the human user while the KI#4 is used for non-3GPP device.

In the KI#1/2/3, the “active” means:

A user is considered active if the associated user identifier has been authenticated and authorized to use a linked subscription to access the 5GS.

For the key issue 4 (i.e. non-3GPP device), there is no concept about what is “active” for non-3gpp devices. Does this mean, the non-3gpp devices needs to been authenticated and authorized to use a linked subscription (such as the assumption made for KI#1/2/3)?

But in the Note 1 of KI#4, it said “5G authentication for non-3GPP devices behind 5G-RG is not assumed in this study.”

Because it is unclear what “active non-3GPP devices” refer and there is no 5G authentication for non-3GPP devices, it proposes to remove the EN directly.

2. Proposal

It is proposed to accept the change to 23.700-32.

>>>> Start of Change <<<<

## 5.4	Key Issue #4: Identifying non-3GPP Devices Connecting behind a UE or 5G-RG

### 5.4.1	Description

This key issue will study whether and how 5GC identifies individual non-3GPP devices connecting behind a UE or 5G-RG and whether and how to provide policy control for the traffic associated with individual non-3GPP devices.

The use case that is associated with this key issue is the case where non-3GPP devices behind a UE or 5G-RG need to be identified.

The objective of this key issue is how an identifier is used by the network to control the traffic to/from UE or 5G-RG when the traffic is associated with the non-3GPP devices. This objective differs from existing support for AUN3 devices in TS 23.316 [7] because the objective is to enable the non-3GPP devices to be identified and to use only the subscription of the UE or 5G-RG to access the 5GC (i.e. the UE or 5G-RG should have to maintain only a NAS Context itself and not for each non-3GPP device).

Solutions to this key issue will address:

-	whether and how the 5GC identifies individual non-3GPP devices connecting behind a UE or 5G-RG. (e.g. in order to charge the individual non-3GPP devices),

-	whether and how to provide policy control for the traffic of individual non-3GPP devices connecting behind a UE or 5G-RG. Including whether and how to trigger policy control for the traffic of individual non-3GPP devices via PCF and NEF APIs, including whether and how to support concurrent services with differing QoS requirements launched by the different non-3GPP devices.

NOTE 1:	Changes to the layer 1 or layer 2 protocols of non-3GPP devices are not in scope of this study. 5G authentication for non-3GPP devices behind 5G-RG is not assumed in this study.

NOTE 2:	Conclusions related to an 5G-RG should be shared with the Broadband Forum (BBF) and CableLabs.

NOTE 3:	Solutions for Ethernet PDU Session should work in the presence of randomized MAC addresses.

>>>> End of Change <<<<