3GPP TSG-SA/WG2 Meeting #161	S2-2402752

# Athens, GREECE, February 26 – March 01, 2024

Source:	CATT

Title:	KI#2, New solution – Application layer assisted EAS and local UPF (re)selection

Document for:	Agreement

Agenda Item: 	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract of the contribution: This contribution proposes a new solution for KI#2: Enhancement of EAS and local UPF (re)selection.

- Discussion

As described in the clause 5.2 of TR 23.700-49, the following aspects will be studied for KI#2:

- How to enable 5GC and/or a third party trusted and/or non-trusted Application Function to select the most suitable local UPF and EAS considering end to end delay that includes delay between a candidate N6 interface of the 5GS and a candidate EAS, and/or potentially EAS load.

- Whether and how to define N6 delay information (e.g., RTT).

- Whether considering the EAS load is needed, and if it is needed how the information representing the EAS load would be defined.

- For local UPF and EAS (re)selection, whether and how to take into account potentially EAS load and N6 Delay between the local PSA and EAS, including how to obtain and update the above information.

This paper is to discuss and resolve the above issues.

2	Proposal

It is proposed to agree the following texts in clause 6 of TR 23.700-49.

******* start of 1st change ********

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions |  | Key Issue #2 |  |  |
| #x |  | x |  |  |

6.X	Solution #X: Application layer assisted EAS and local UPF (re)selection

### 6.X.1	Key Issue mapping

The following solution corresponds to the key issue #2 on enhancement of EAS and local UPF (re)selection.

### 6.X.2	Description

The AF provisions the EAS status information (see Table 6.x.1-1). The procedure of EAS Deployment Information provisioning described in clauses 6.2.3.4.2 and 6.2.3.4.3 of TS 23.548 can be reused to provision and manage EAS status information. Based on the EAS status information, SMF can select a suitable EAS and local PSA UPF.

Table 6.x.2-1 Description of EAS Status Information

| Parameter | Description |
| --- | --- |
| EAS load | List of load information for EAS(s) (each EAS identified by the EAS IP address). The load can be current load, available load, maximum load etc. [optional] |
| EAS delay | List of delay information for EAS(s) (each EAS identified by the EAS IP address). The delay represents the process delay at EAS. [optional] |
| N6 delay | Delay between each UPF (identified by UPF ID) and EAS (identified by EAS IP address). [optional] |

### 6.X.3	Procedures

Figure 6.x.3-1: Application layer assisted EAS and local UPF (re)selection

1.	The AF provisions the EAS status information as described in clause 6.x.2.

2.	The steps 1 to 19 of figure 6.2.3.2.2-1 are performed with following differences:

In step 16, based on EAS status information received in step 1, the EAS information received from the EASDF in Neasdf_DNSContext_Notify, and other UPF selection criteria, the SMF selects a suitable EAS and local PSA UPF. The SMF inserts the Local PSA UPF.

In step 17, the SMF sends the updated DNS message handling rule to EASDF via Neasdf_DNSContext_Update Request. The DNS message handling rule includes the selected EAS IP address.

In step 19, the EASDF sends the DNS Response with selected EAS IP address to the UE as indicated in the DNS message handling rule.

### 6.X.4	Impacts on services, entities and interfaces

AF:

-	Provisions EAS status information.

NEF/UDR:

-	Handles and stores EAS status information.

SMF:

-	Manages EAS status information.

-	Selects a suitable EAS and local PSA UPF based on EAS status information.

******* End of 1nd change ********