3GPP TSG-SA2 Meeting #161	S2-2402823

Athens, Greece, February 26 – March 3, 2024	(revision of S2-xxxxxx)

# Source:	Siemens AG

# Title:	New WID on 5GS Support on Loop Prevention for LAN

# Document for:	Approval

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: New WID on 5GS Support on Loop Prevention for LAN

Acronym: TEI19_LP_LAN

Unique identifier:	TBD

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a Feature

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 720005 | (Stage 1 of 5G) New Services and Markets Technology Enablers | Rel-15 work item |
| 800007 | Service requirements for cyber-physical control applications in vertical domains | Rel-16 work item |

# 3	Justification

Loop prevention is a pervasive requirement for Ethernet Transport Services. If, for instance the capillary networks “behind” two UEs connects or if the UE can connect to the same Data Network via two or more PDU Session Anchors, this can result in layer-two loops. Loop prevention protocols such as MSTP suppress such loops. Additionally, such protocols support 5GS media redundancy: if the wireline breaks, every node is still reachable through an alternative path.

Current specifications do not offer active means for loop prevention, the specification does not guarantee that the Ethernet network remains loop-free. 5GS enhancements for support of MSTP (Multiple Spanning Tree Protocol) and RSTP (Rapid Spanning Tree Protocol) need to be explicitly mentioned.

# 4	Objective

The objective of this work item:

WT-1: Support of MSTP for 5G VN scenarios and support of RSTP/MSTP for scenarios of 5GS integrated with Ethernet LAN. This work task includes aspects of:

- Support for MSTP and RSTP configuration when 5GS behaves as a logical IEEE 802.1Q Bridge to the external Ethernet network.

- Support for dynamic loop prevention due to related topology changes in the Ethernet network.

## TU estimates and dependencies

| Work Tas ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT-1 |  | 1 | No | None |

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Ling Zhang, Siemens AG (zhangling@siemens.com)

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

# 9	Supporting Individual Members

Supporting IM name

Siemens AG

Mitsubishi Electric