SA WG2 Meeting #161	S2-2403248

February 26th – March 1st, 2024, Athens, Greece             	(revision of S2-2402162)

Source:	Apple, ETRI, Nokia, Nokia Shanghai Bell, China Telecom, LG Electronics, OPPO, NEC, KDDI

Title:	FS_MASSS Architectural Terms

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS /Rel-19

Abstract of the contribution: This paper proposes Terms for DualSteer for the FS_MASSS TR 23.700-54.

# 1	Discussion

This paper proposes text for the terms section of the FS_MASSS TR 23.700-54.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.1.0.

* * * * Start of Changes * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

DualSteer Device: A device supporting DualSteer traffic steering and switching of user data (for different services) across two 3GPP access networks; it can be a single UE, in case of non-simultaneous data transmission over the two 3GPP access networks, or it is two separate UEs in case of simultaneous data transmission over the two 3GPP access networks.

Editor’s Note: The implications of this definition may be further discussed during solution phase. During the study evaluation phase it will be determined whether there is a need for close coordination between the two UEs in a DualSteer Device at the 3GPP layers, and whether that coordination needs be described in normative or informative manner.

DualSteer traffic steering: the procedure whereby a DualSteer Device selects a 3GPP access network for a new service and transfers all the traffic of that service over the selected 3GPP access network.

DualSteer traffic switching: the procedure whereby a DualSteer device moves all the traffic of a service from one registered 3GPP access network to another 3GPP access network in a way that minimizes service interruption.

* * * * End of Changes * * * *