SA WG2 Meeting #162	S2-2404353

Changsha, China, April 15 –19, 2024	(revision of S2-2402372)

Source:	LG Electronics

Title:	KI#2, Sol#7: Solution updates to resolve EN

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This paper proposes to update Solution#7.

Discussion

TR 23.700-66 includes Solution #7: Policy control for network energy saving for KI#2 "Subscription and policy control to support energy efficiency and energy saving as service criteria" with some ENs.

This paper proposes to update Solution#7 mainly by resolving the following EN.

Editor's note:	How the energy control information as assistance information for policy adjustment satisfies the energy consumption threshold is FFS.

Proposal

It is proposed to agree the following changes into TR 23.700-66.

* * * * Start of 1st Change * * * *

6.7	Solution #7: Policy control for network energy saving

6.7.1	Key Issue mapping

This solution is related to the KI#2: Subscription and policy control to support energy efficiency and energy saving as service criteria. This solution addresses the following issues of KI#2:

-	Whether and how to enhance the existing subscription and policy control framework to support energy related information as service criteria, including:

-	Whether and what new energy related UE subscription information are to be defined, and whether and how to use the energy related UE subscription information.

-	Answer: Energy saving authorization information is added in UE subscription information, authorizing enforcement of energy related policies. Energy consumption threshold may also be included in subscription information to limit the maximum energy consumption.

-	Whether and what new energy related policies are to be defined, and how to perform energy related policy control, e.g. to determine, provision and enforce energy related policies.

-	Answer: This solution re-uses existing AM policy and SM policy and reuses the create/update procedures of the policies, and PDU session release or deactivation information as new SM policy parameter.

-	At what granularity (e.g. network slice, UE, NF, PDU Session, QoS flow, application ID, etc.) the energy related policy control can be performed.

-	Answer: Energy policy control is performed at UE granularity or PDU session granularity in this solution.

-	What network energy related information is required for subscription and policy control and how it is obtained.

-	Answer: Energy consumption threshold and energy consumption assistance information is required. The energy consumption threshold is provided by AF or UDM, and the energy consumption assistance information is collected from 5GC NFs and/or OAM.

6.7.2	Functional Description

The architecture of this solution is described as Figure 6.7.2-1.

Figure 6.7.2-1: Architecture assumption

A new NF, Energy Efficiency Control Function (EECF) is introduced. The EECF includes at least the following functionality:

-	Collecting of the energy consumption assistance information (e.g. data volume, bit rate, etc.) per UE or per PDU session from other 5GS NFs.

Editor's note:	how the detailed procedure of this information is collected and which information besides data volume and bit rate is collected, is FFS.

-	Obtaining energy consumption information (can include ratio of renewable energy and carbon emission information when available) from OAM.

-	Obtaining energy saving authorization information and energy consumption threshold via UE subscription.

Editor's note:	what is this authorization really authorizing. This should be clarified.

-	Generating and sending energy control information to the PCF as assistance information for policy adjustment for energy saving.

This solution performs energy saving in UE or PDU session granularity, by adjusting the AM policy and SM policy. The following existing or new parameters of AM policy or SM policy are to be sent or updated for energy saving consideration:

For AM policy:

-	UE-AMBR: Setting lower UE-AMBR limits the bit rate per UE granularity, to reduce the energy consumption.

-	UE-Slice-MBR: Setting lower UE-Slice-MBR limits the bit rate per UE, to reduce the energy consumption.

-	Restricted RAT type: Restricting usage of specific RAT, to reduce the energy consumption.

Editor's note:	The effectiveness of AM/SM policy for AMBR adjustment to the reduction of energy consumption is FFS.

For SM policy:

-	QoS information: Energy consumption can be reduced by enforcing QoS with lower Session-AMBR, higher PDB, higher PER, etc.

-	PDU session release: Releasing one or more PDU session(s) saves the energy consumption of the PDU session(s).

-	PDU Session deactivation: Deactivating user plane of one or more PDU session(s) saves the energy consumption of the PDU session(s).

NOTE:	It is assumed that the controlled resources (e.g. Non-GBR QoS Flows, slices, PDU Sessions) are used only for the best-effort traffic and the guaranteed traffic is not affected.

In this solution, the EECF generates the energy control information for policy adjustment, based on energy consumption threshold from the AF or UDM, subscription information of the UE and energy consumption assistance information (e.g. data volume, bit rate, etc.) from 5GC NFs and/or OAM.

The EECF sends the energy control information to PCF. The PCF can adjust the parameters in AM and SM policies as described above. The PCF reuses existing AM policy association modification and SM policy association modification procedures to implement the energy saving related policies. It is also assumed that the EECF can also obtain the data volume of one single PDU session through UPF event exposure service, then calculate PDU Session energy consumption.

6.7.3	Procedures

Figure 6.7.3-1: Procedure for policy control for network energy saving

1.	AF creates a request for energy saving, including UE identifier(s), UE address(es), S-NSSAI, DNN, and energy consumption threshold, and sends the request to NEF.

Editor's note:	How does the AF determine the threshold? Is it a customer or the operator?

2.	The NEF invokes Neecf_EnergySaving to send the information provided by AF in step 1 to EECF.

3.	The EECF obtains energy saving authorization information of the target UE(s) via subscribed information from UDM. The EECF may also obtain energy consumption threshold from UDM.

4.	For the target UE(s), or PDU Session(s), the EECF collects energy consumption assistance information (e.g. data volume, bit rate, etc.) from 5GC NFs. The details can be referred to solution of KI#1. For the data volume, the EECF can obtain the data amount of one single PDU session through UPF event exposure service.

5.	Based on the energy consumption assistance information, and the energy consumption threshold provided by AF and/or UDM, the EECF sends energy control information for policy adjustment to the PCF. The energy control information for policy adjustment per target UE may contain one or more of the following information:

A)	Indication that the energy consumption threshold is exceeded or about to exceed

B)	UE-AMBR

C)	UE-Slice-MBR

D)	Restricted RAT type

The EECF sends the information to PCF via Npcf_AMPolicyAuthorization Create/Update service.

NOTE 1:	It is assumed that UE-AMBR, UE-Slice-MBR can be adjusted when the energy consumption threshold is about to exceed while RAT type can be restricted when the energy consumption threshold is exceeded.

6.	Based on the energy consumption assistance information and the energy consumption threshold provided by AF and/or UDM, the EECF sends energy control information for policy adjustment to the PCF. The energy control information for policy adjustment per PDU Session may contain one or more of the following information:

a)	Indication that the energy consumption threshold is exceeded or about to exceed,

b)	Indication to adjust QoS parameters

c)	Indication to trigger PDU Session release or deactivation

The EECF sends the information to PCF via Npcf_PolicyAuthorization Create/Update service.

NOTE 2:	It is assumed that QoS parameters can be adjusted when the energy consumption threshold is about to exceed while PDU Session release or deactivation can be performed when the energy consumption threshold is exceeded.

7.	The PCF reuses steps 2-4 of the AM Policy Association Modification procedure as specified in clause 4.16.2.1.1 of TS 23.502 [3] to update AM policy based on the energy control information received from EECF in step 5, local policy. If the EECF provided only A) in step 5, which AM policy is applied is up to the PCF.

8.	The PCF reuses steps 3-5 of the SM Policy Association Modification procedure as specified in clause 4.16.5.2.1 of TS 23.502 [3] to update SM policy based on the energy control information received from EECF in step 6, local policy. If the EECF provided only a) in step 6, which SM policy is applied is up to the PCF.

NOTE 3:	It is assumed that the controlled resources (e.g. Non-GBR QoS Flows, slices, PDU Sessions) are used only for the best-effort traffic and the guaranteed traffic is not affected.

9.	The SMF triggers N4 Session Modification procedure specified in clause 4.4.1.3 of TS 23.502 [3] to enforce the QoS parameters in the updated SM policy in step 8.

10.	If the updated SM policy in step 8 includes indication of PDU session release, the SMF triggers PDU Session release procedure specified in clause 4.3.4.2 of TS 23.502 [3]. If the updated SM policy in step 8 includes indication of PDU session deactivation, the SMF triggers PDU Session deactivation procedure specified in clause 4.3.7 of TS 23.502 [3].

Editor's note:	Whether there is a need for a new 5GC NF (the name is called as EECF or EMF, will be harmonized) or whether proposed functionality can be provided by existing NFs will be evaluated during evaluation and conclusion phase

Editor's note:	Whether the EECF can be collocated with or part of PCF is FFS.

Editor's note:	How per UE energy consumption can be derived from data on data-rate or volume is FFS (same rate or volume can consume different amount power/energy depending on radio conditions). More details on the PDU session granularity energy related information is FFS.

6.7.4	Impacts on existing services, entities and interfaces

EECF:

-	New network functionalities are as description of clause 6.7.2.

NEF:

-	New service operation Nnef_EnergySaving.

PCF:

-	Updating AM policy and SM policy based on the energy control information for policy adjustment received from EECF, local policy.

UDM:

-	Storing and providing energy saving authorization information and energy consumption threshold in subscription information.

UPF:

-	Reporting the data volume of specific PDU session based on EECF request.

* * * * End of Changes * * * *