3GPP TSG-SA WG2 Meeting #162	S2-2404467

# Changsha, China,  April 15 - 19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0364 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update mobility procedures for MBS assistance information transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN3 sent an LS R3-241145 to SA2 to clarify the selection of NGAP message to provide MBS assistance information from the SMF to the target NG-RAN. In the LS, RAN3 concluded: - RAN node signals to the SMF whether it supports the multicast reception in RRC inactive state. - Based on the signal, the SMF should include MBS assistance information in PDU session modification only when the source RAN node does not support multicast reception in RRC inactive state while the target RAN node supports. To align with RAN3, the conclusions above need to be captured in TS 23.247. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Capture the signal from RAN node to the SMF about whether it supports the multicast reception in RRC inactive state. - Clarify only when when the source RAN node does not support multicast reception in RRC inactive state while the target RAN node supports, the SMF includes the MBS assistance information to the target RAN node. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Misalignment with RAN3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES <<<<

##### 7.2.3.8.1	General

The RAN node signals to the SMF whether it supports the Multicast reception in RRC inactive state via "multicast-supported-with-reception-in-RRC-inactive" in "MBS Support Indicator" as specified in TS 38.413 [15].

The procedures in clauses 7.2.3.2, 7.2.3.3, 7.2.3.4, 7.2.3.6 and 7.2.3.7 apply with the following additions:

-	If the MBS assistance information is available at the SMF for an MBS session that the UE joined, only when the source RAN node does not signal "multicast-supported-with-reception-in-RRC-inactive" support while the target RAN node signals the support, the target RAN node needs the MBS Assistance Information from the SMF.

When the SMF provides the MBS session related information as part of the associated PDU Session context data as specified in the existing procedures, the SMF also includes the MBS Assistance Information in the N2 SM information.

-	For N2-based handover, when the SMF provides the MBS session related information as part of the associated PDU Session context data as specified in the existing procedures, the SMF also includes the MBS Assistance Information in the N2 SM information.

-	For Xn-based handover or Connection Resume, the MBS session information transferred from source NG-RAN towards target NG-RAN also include MBS assistance information for the MBS session if such information is available at the source RAN node as specified in TS 38.300 [9].

NOTE 1:	In deployments where not all the MBS supporting NG RAN nodes support delivery of multicast MBS session data to UEs in RRC_INACTIVE state, a source NG-RAN node not supporting delivery of multicast MBS session data to UEs in RRC_INACTIVE state will not provide MBS assistance information for the MBS session to the target NG-RAN node.

NOTE 2:	It is assumed that a RAN node not supporting delivery of multicast MBS session data to UEs in RRC_INACTIVE state ignores the MBS assistance information for the MBS session.

For a UE that is receiving multicast MBS data in CM-CONNECTED with RRC_INACTIVE state, the scenarios for UE mobility are as follows:

-	UE moves to new cell within the RAN Notification Area (RNA).

-	UE moves outside the current RAN Notification Area but within the current Registration Area (RA).

-	UE moves out of the current Registration Area.

# >>>>END OF CHANGES<<<<