SA WG2 Meeting #162	S2-2404557

15 - 19 April, 2024, Changsha, China

Source:	vivo

Title:	New Solutions: Information provisioning for additional SUPI configuration and network selection

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes a new solution for providing configuration information to DualSteer device for network selection or additional SUPI configuration.

1	Discussion

This paper proposes a new solution for KI#1.4 policy of the FS_MASSS SID (SP-240467).

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.2.0.

* * * * Start of Changes * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of DualSteer Solutions to Key Issues

|  | Key Issues for DualSteer |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | Key Issue #1.1 | Key Issue #1.2 | Key Issue #1.3 | Key Issue #1.4 |
| #1.D | X |  |  | X |
| #1.E |  |  |  | X |

Table 6.0-2: Mapping of ATSSS_Ph4 Solutions to Key Issues

|  | Key Issues for ATSSS_Ph4 |  |
| --- | --- | --- |
| Solution# | Key Issue #2.1 | Key Issue #2.2 |
| #2.1 | X |  |
| #2.2 |  | X |
| #2.3 | X |  |
| #2.4 | X |  |
| #2.5 | X |  |
| #2.6 |  | X |
| #2.7 |  | X |
| #2.8 |  | X |

* * * * Second Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[6]	IETF RFC 9298: "Proxying UDP in HTTP".

[7]	IETF RFC 9484: "Proxying IP in HTTP".

[8]	IETF draft-ietf-masque-connect-ethernet: "Proxying Ethernet in HTTP".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[9]	IETF RFC 9114: "Hypertext Transfer Protocol Version 3 (HTTP/3)".

[10]	IETF draft-ietf-httpbis-connect-tcp: "Template-Driven HTTP CONNECT Proxying for TCP".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[11]	IETF RFC 9297: "HTTP Datagrams and the Capsule Protocol".

[12]	IETF RFC 9000: "QUIC – A UDP based Multiplexed and Secured Protocol".

[13]	IETF RFC 9001: "Using TLS to Secure QUIC".

[14]	IETF RFC 9002: "QUIC Loss Detection and Congestion Control".

[15]	IETF RFC 9221: "An Unreliable Datagram Extension to QUIC".

[16]	IETF draft-ietf-quic-multipath: "Multipath Extension for QUIC".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[17]	3GPP TR 23.700-53: "Study on access traffic steering, switching and splitting support in the 5G system architecture; Phase 3".

[18]	IETF RFC 9369: "QUIC Version 2".

[19]	IETF RFC 9220: "Bootstrapping WebSockets with HTTP/3".

[20]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[2x]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[2y]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

* * * * Second Change (all new text) * * * *

### 6.1.D	Solution #D: Additional SUPI configuration provisioning

6.1.D.1	Description

This solution addresses the Key Issue#1.1 "Subscription aspects to support DualSteer" and Key Issue#1.4 "Policy enhancements for DualSteer".

Currently, if two SUPIs are used, UE shall be configured by user to activate the registration of the additional SUPI, and UE shall interact with user or be configured by user to allow the data transmission over the additional SUPI. For DualSteer, if DualSteer traffic steering or switching every time need to be allowed by user, or if user does not open the steering or switching, the user experience even the performance of DualSteer will be not good enough.

DualSteer as a subscribed service, if additional SUPI can be provided by network to the DualSteer device, the DualSteer service can be automatically activated online without the requirement of user applying the service on operator's business site, which makes the service activation easily.

6.1.D.2	Procedures

Figure 6.1.D.2-1: Additional SUPI configuration provisioning

Precondition: The DualSteer device may not have additional SUPI (SUPI#2) initially.

1.	The DualSteer device performs 5GS registration procedure with SUPI#1 by initiating Registration Request message towards the AMF#1.

2.	After the registration procedure, the UDM performs UE parameters Update procedure as described in clause 4.20 of TS 23.502 [4]. The UDM Update Data in AM subscription data may include Additional SUPI for DualSteer, e.g., due to the two SUPIs share same charging profile. The UDM may interact with AUSF for secure protecting as described in clause 6.15.2 of TS 33.501 [2y].

Editor's note:	Whether existing encryption method (e.g., SUCI or secured packet mechanism) or new is used to encrypt the additional SUPI needs coordination with SA3.

4.	The UDM invokes Nudm_SDM_Notification (UDM Update Data (additional SUPI)) service operation towards the AMF#1.

5.	The AMF#1 forwards the UDM Update Data transparently to the DualSteer device subject to SUPI#1 by sending DL NAS Transport message.

6.	If the additional SUPI is received, when the DualSteer device has SUPI#2, it determines whether the additional SUPI is same as SUPI#2, if yes, then DualSteer option is activated automatically, and following registration procedure will be treated as dual registration for DualSteer, when the DualSteer device does not have additional SUPI, DualSteer option may be activated automatically with the additional SUPI, and it may use the received additional SUPI to perform following registration procedure for DualSteer.

Editor's note:	For the case that DualSteer device initially does not have additional SUPI, the security issue of using network provided additional SUPI to perform dual registration needs coordination with SA3.

7.	The DualSteer device may perform registration procedure with SUPI#2 (i.e., additional SUPI) and treat it as dual registration for DualSteer.

6.1.D.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP services, entities and interfaces.

UDM:

-	Subscription data of UDM Update Data is enhanced to include additional SUPI for DualSteer.

DualSteer device:

-	Enable performing DualSteer feature automatically according to the network provided SUPI.

* * * * Third Change (all new text) * * * *

### 6.1.E	Solution #E: Network selection information provisioning

6.1.E.1	Description

This solution addresses the Key Issue#1.4 "Policy enhancements for DualSteer".

This solution reuses SoR procedure described in Annex C of TS 23.122 [2x].

6.1.E.2	Procedures

Following figure as an example only depicts the call flow that UDM provides SoR information to DualSteer device after registration procedure.

Figure 6.1.E.2-1: Network selection information provisioning for DualSteer

1.	The DualSteer device performs registration procedure with SUPI#2.

2.	The DualSteer device performs registration procedure with SUPI#1.

3.	The UDM prepares the SoR information for SUPI#1, and may prepare the SoR information for SUPI#2.

4.	The UDM provides SoR information related to SUPI#1 towards AMF#1 (i.e., by invoking Nudm_SDM_Notification Request), and may provide SoR information related to SUPI#2 towards AMF#2.

5.	The AMF#1 sends a DL NAS TRANSPORT message towards the DualSteer device subject to SUPI#1. The AMF#1 includes in the DL NAS TRANSPORT message the SoR information received from the UDM. The AMF#2 may also provide SoR information received from the UDM to the DualSteer device subject to SUPI#2.

6.	The DualSteer device and AMF#1/AMF#2 may response as described in Annex C of TS 23.122 [2x].

6.1.E.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP services, entities and interfaces.

UDM:

-	Prepare SoR information for DualSteer device.

* * * * End of Changes * * * *