SA WG2 Meeting #162	S2-2404748

Changsha, China, April 15 – April 19, 2024

# Source:	CATT

Title:	KI#2, Sol #9: Update to resolve ENs

Document for:	Agreement

Agenda Item: 	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract of the contribution: This contribution proposes to update solution #9 to resolve the Editor Notes.

- Discussion

- It is to address the following EN:

Editor's note: Whether End to end delay requirement is sent within EDI or Application Function influence on traffic routing related information as described in clause 5.6.7 of TS 23.501 [2] is FFS.

Per current specs, it is normally use the Application Function influence on traffic routing procedure to provide the requirements (e.g. User Plane Latency Requirement, N6 Traffic Routing requirements) for traffic routing. So it is proposed to provide End to end delay requirement within Application Function influence on traffic routing related information.

- It is to address the following EN:

Editor's note: Whether the processing delay can be provided by the AF or not is FFS.

The application server can have knowledge of processing delay locally, so the processing delay can be provided by the AF directly. Adding a new functionality of EAS load conversion at SMF is not very necessary.

2	Proposal

It is proposed to agree the following changes in clause 6.9 of TR 23.700-49.

******* start of 1st change ********

6.9	Solution #9: local UPF and EAS (re)selection jointly considering N6 delay and EAS load

6.9.1	Description

Per existing design of TS 23.548 [5], the (re)selection of the EAS and local PSA by 5GC is based on the UE location and EAS Deployment Information, which only considers the topology information of UE, UPF and EAS. Nevertheless, dynamic information especially the N6 delay between the local PSA and EAS, and EAS load, which contributes to the End to end delay (i.e. between the UE and EAS), are also important factors that should be taken into account.

Some consideration that EAS load has been considered when the DNS server returns the DNS response back to the client. However, these parameters are considered separately. It is better to have one NF jointly consider these parameters and also get the real time measurement.

Comparing to the solution discussed for KI#1, this solution focus on how the N6 delay and EAS load is used for L-PSA/EAS (re)selection.

6.9.3	Procedures

6.9.3.1	Enhancement on EAS Deployment Information Management

The EAS deployment information provisioned from AF to 5GC as defined in clause 6.2.3.4 of TS 23.548 [5] is used for local UPF and EAS (re)selection. It is proposed to enhance this information as following:

Table 6.9.3.1-1: Additional information of EAS Deployment Information

| Parameters | Description |
| --- | --- |
| … | …. |
| Delay-sensitive indication | the application demands end to end delay requirement [optional]. |
| AF/NEF address information | Target Address for event subscription [optional]. (See NOTE 5). |
| NOTE 4: If the AF is trusted, the AF provides AF address information during the EDI provision process. Otherwise, NEF provides the NEF address information to replace the address information provided by the AF. |  |

6.9.3.2	EAS/L-PSA discovery procedure with EASDF

Normally the EAS load can be converted to the processing delay (e.g. https://ieeexplore.ieee.org/abstract/document/9253665). Hence when the EAS reports how much percentage of load EAS reaches, based on the local configuration, the percentage of load can be converted to the how much processing delay the EAS needs.

Figure 6.9.3.2-1: EAS discovery procedure with EASDF

The AF request in step 1-5 of figure 4.3.6.2-1 in TS 23.502 [3] is used to request End-to-end delay requirement and/or N6 delay requirement for EAS discovery.

The following information may be included in AF request as defined in clause 5.6.7.1 of TS 23.501 [2]:

-	End-to-end delay requirement indicates the value of delay between UE and EAS needs to be satisfied.

-	N6 delay requirement indicates the value of delay between PSA UPF and EAS needs to be satisfied.

Then the following procedure is performed:

1.	Same as step 1 - step 12 of clause 6.2.3.2.2 in TS 23.548 [5].

2.	same as step 13 as described in clause 6.2.3.2.2 of TS 23.548 [5].

3.	same as step 14/15 as described in clause 6.2.3.2.2 of TS 23.548 [5].

NOTE 1:	The SMF can be changed to other NF per the discussion of the KI#1.

4a/4b.	If the matched DNS response message corresponding to the delay sensitive application, i.e. which has provisioned the Delay-sensitive indication as part of EDI, SMF subscribe to the EAS load information and processing delay from AF by sending the received EAS IP (s)/FQDN(s), subscribed event to AF through NEF. The event subscription also includes the Immediate reporting flag and reporting threshold, e.g. load level, delay level.

4c.	SMF subscribe to the EAS load information from AF by sending the received EAS IP (s)/FQDN(s), subscribed event directly. The event subscription also includes the Immediate reporting flag and reporting threshold, e.g. load level, delay level.

4d/4e.	AF notifies SMF of EAS load information corresponding to the received EAS IP (s)/FQDN(s) through NEF immediately. The EAS load and processing delay are reported. The EAS load presents as percentage of EAS Instance load.

4f.	AF notifies SMF of EAS load information corresponding to the received EAS IP (s)/FQDN(s) immediately. The EAS load and processing delay are reported. The EAS load expresses as the percentage of EAS Instance load.

NOTE 2:	The AF can also send a notification when a condition is met, e.g. the load level of the EAS meet the reporting threshold. The details are described in clause 6.9.3.3. It is assumed the AF is willing to provide EAS load information to use the optimization from 5GC.

5.	If the processing delay is not received in step 4d/4e/4f, the SMF performs EAS load-Processing delay conversion, i.e. convert the percentage of EAS load to the processing delay.

Editor's note:	The details of the conversion is FFS.

NOTE 3:	The relationship between EAS load and processing delay can be pre-configured on SMF.

6.	SMF informs candidate local PSA(s) to measure the real-time N6 delay between the L-PSA and the reported EAS IP(s). In details, SMF determine the candidate Local PSA(s) base on the reported EAS IP(s) and Local PSA information. Then, the candidate local PSA(s) obtain the N6 delay by using certain Layer 3 measuring mechanisms respectively, e.g. via ICMP as defined in RFC 792 [7].

NOTE 4:	Step 6 can be executed in parallel with steps 4 or 5.

7.	SMF determines suitable EAS and L-PSA based on the end-to end delay requirement.

NOTE 5:	The delay between UE and PSA UPF is controlled by the PDB in the 5QI. Thus only N6 delay and processing delay are measured for EAS and PSA selection.

8.	Same as step 16 - step 19 as described in clause 6.2.3.2.2 of TS 23.548 [5].

For EAS/L-PSA discovery procedure with local DNS server mechanism, the L-PSA is inserted either per DNS query or per UE mobility before DNS query. For the L-PSA inserted per DNS query, same procedure (i.e. Step 4 -7) as described above can be executed.

6.9.3.3	EAS/L-PSA rediscovery at Edge Relocation

It is assumed that the SMF, which re-discovers the EAS/L-PSA, can be other NF per the discussion of KI#1.

Figure 6.9.3.3-1: EAS/L-PSA rediscovery procedure at Edge relocation

This procedure is used to trigger the EAS (or L-PSA) rediscovery procedure when a new connection between EAS and L-PSA need to be established. This rediscovery can be triggered in three cases:

-	Case 1: SMF triggered EAS rediscovery based on reported EAS load as the end-to end delay requirement is not satisfied.

-	Case 2: SMF triggered L-PSA or EAS rediscovery based on measured N6 delay as end-to end delay requirement is not satisfied.

-	Case 3: AF triggered EAS rediscovery as EAS load balance or maintenance, etc.

1.	Perform EAS and L-PSA discovery as described in clause 6.9.3.2.

Case 1: EAS load trigger EAS rediscovery:

2.	AF notifies SMF of EAS load information corresponding to the EAS IP (s) through NEF or directly when the event condition is met, e.g. the EAS load exceed the load threshold set by SMF. The SMF sets load threshold based on the end to end delay requirement in the AF request.

3.	The same as step 5 of clause 6.9.3.2.

4.	SMF triggers EAS discovery as the end-to end delay requirement are not satisfied.

Case 2: N6 delay trigger L-PSA or EAS rediscovery:

5.	The SMF subscribes the N6 delay measurement event to L-PSA. This event subscription can be per event threshold or periodically. The UPF report the measured real-time N6 delay to the SMF by using certain Layer 3 measuring mechanisms respectively, e.g. via ICMP as defined in RFC 792 [7]. The SMF judges that the N6 delay exceed the related threshold per end-to-end delay requirement provisioned in AF request.

6.	If there are candidate L-PSA(s), e.g. different L-PSA to the same DNAI, the SMF informs the candidate L-PSA(s) to measure the N6 delay to the indicated EAS instance.

7.	If the calculated end-to end delay of the candidate L-PSA(s) can fulfil the end-to end delay requirement, the SMF triggers the L-PSA reselection and following steps are skipped, i.e. Step 8-9 are skipped. Otherwise SMF trigger EAS rediscovery as the end-to end delay requirement are not satisfied.

Case 3: AF trigger EAS rediscovery:

8.	AF trigger EAS rediscovery as EAS load balance or maintenance, etc.

9.	Same as step 2 of clause 6.2.3.3.3 in TS 23.548 [5].

6.9.3	Impacts on services, entities and interfaces

AF:

-	Provide enhanced EDI information as described above.

-	Provide the EAS load information and processing delay to SMF per event subscription.

-	Provide End-to-end delay requirement and/or N6 delay requirement for EAS discovery via AF request.

SMF:

-	Get enhanced EDI information and EAS load information and processing delay from AF.

-	Get End-to-end delay requirement and/or N6 delay requirement from AF.

-	Perform EAS load - processing delay conversion.

-	Instructs UPF to measure the real-time N6 delay.

-	Determine suitable EAS and L-PSA based on the above information.

******* End of 1nd change ********