SA WG2 Meeting #S2-162	S2-2404775

15 - 19 April, 2024, Changsha, China	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0132 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the partial coverage issue |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-03-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the SA2#160AHE meeting, SA2 sends a LS to RAN2 to ask the partial coverage issue (which is not support by RAN2). In this meeting, RAN2 send the LS response (S2-2403886/R2-2401629). In the LS response, RAN2 clarify there is no issue in the SA2 level to support the partial coverage, the RAN2 have determined to remove the NOTE excluding partial coverage in stage 2. So the CR proposes to make the alignment with RAN2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove partial coverage condition in the Located UE and SL Reference UE Discovery and selection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Partial coverage issue is not resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, 5.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.2.2	Located UE Discovery & Selection

When Sidelink Positioning is applied for Target UE in a SL-MO-LR, SL-MT-LR, 5GC-MO-LR or 5GC-MT-LR procedure, Located UE(s) may be discovered and selected:

-	When LMF determines to apply Sidelink Positioning for Target UE, LMF triggers the Target UE to perform discovery of the Located UE.

-	When the Target UE determines to apply Sidelink Positioning, the Target UE triggers the discovery of the Located UE.

The LMF may be pre-configured with the list of candidate Located UEs including e.g. the capability, stationary information and location information of the Located UE(s), e.g. RSU or the operator deployed Located UE. The LMF may provide to the Target UE a list of candidate Located UE(s).

The discovery of Located UEs follows the same principles as specified in clause 5.2.1. The UE can indicate its role "Located UE" in its list of supported roles during discovery, if it is authorized to be a Located UE in a given PLMN as per the Authorization and Provisioning for Ranging/SL positioning service as specified in clause 5.1.

Editor's note:	Privacy aspects of sharing the location of a Located UE (e.g. to any Target UE) and exposing information about such privacy aspects during discovery (e.g. to facilitate Located UE selection by a Target UE) needs to be aligned with SA WG3.

A Target UE may discover and select one or more Located UEs to be used in the Ranging/SL positioning procedures as specified in clauses 5.3 to 5.5.

Multiple candidate Located UEs may be discovered, in that case, the Located UE(s) is selected from the candidate Located UE list. The Located UE(s) is selected based on:

-	Candidate list of Located UE(s), if available.

-	Capabilities of the candidate Located UE(s), e.g. the supported Sidelink Positioning methods.

-	The required positioning QoS of Target UE, positioning QoS supported by the candidate Located UE(s).

-	Whether the serving PLMN of candidate Located UE(s) is same with serving PLMN of Target UE.

-	UE's information, candidate Located UE's location.

When LMF determines SL positioning for target UE and trigger the discovery of the Located UE, LMF can decide that LMF or target UE selects Located UEs. If the decision is LMF selecting Located UEs, Target UE sends the multiple discovered candidate Located UEs to the LMF for the selection. After the LMF determines the selected Located UE(s), the LMF sends the selected Located UE(s) to the Target UE.

In this release of the specification, for UE Positioning assisted by Sidelink Positioning and involving 5GC, the Target UE shall discover and select Located UEs that are in the same serving PLMN of the Target UE as described in clause 5.5.1.

LMF needs to provide located UE for discovery when AF request ranging between two UEs. Additionally, LMF may be provisioned with Located UEs that may be sent to target UE as candidate list of located UEs.

NOTE:	The role of being "Located UE" is dynamic and can change over time, in particular if the Located UE is moving. Hence, the discovery results need to be refreshed if there is a (significant) delay between discovery and initiating of a ranging procedure with a discovered Located UE. How often this is done is up to UE implementation.

Next CHANGE

### 5.2.4	SL Reference UE Discovery & Selection

When Ranging/Sidelink Positioning is applied for Target UE in a SL-MO-LR, SL-MT-LR, 5GC-MO-LR or 5GC-MT-LR procedure, SL Reference UE may be discovered and selected:

-	When LMF determines to apply Ranging/Sidelink Positioning for Target UE, LMF triggers the Target UE to perform discovery of the SL Reference UE.

-	When the Target UE determines to apply Ranging/Sidelink Positioning, the Target UE triggers the discovery of the SL Reference UE.

The discovery of SL Reference UEs follows the same principles as specified in clause 5.2.1. The UE can indicate its role "SL Reference UE" in its list of supported roles during discovery, if it is authorized for Ranging/SL positioning service as specified in clause 5.1.

A Target UE may discover and select one or more SL Reference UEs to be used in the Ranging/SL positioning procedures as specified in clauses 5.3 to 5.5.

When multiple candidate SL Reference UEs are discovered, the SL Reference UE(s) is selected based on:

-	Capabilities of the candidate SL Reference UE(s), e.g. the supported Sidelink Positioning methods.

-	The required positioning QoS.

-	UE's information.

END OF CHANGES