SA WG2 Meeting #162	S2-2404870

15th April – 19th April, 2024, Changsha, China

Source:	Samsung

Title:	KI#1_ New Solution for USS initiated to receive flight assistance information

Document for:	Approval

Agenda Item: 	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes a solution to support for USS to receive flight assistance information from network.

# 1	Discussion

This contribution proposes a solution for addressing USS to receive flight assistance information from network to address key issue #1.

# 2	Proposal

It is proposed to adopt the following new solution in TR 23.700-59.

* * * * Start of Changes * * * *

## 6.1	Solution #1: Support Pre-Mission Planning and In-Mission Monitoring Flight

### 6.1.1	Key Issue mapping

This solution addresses KI#1 aspects.

### 6.1.2	Description

Enhance NEF services to support service exposure and interactions between MNOs and UTM functions for supporting pre-mission flight planning and in-mission flight monitoring for UAVs, is one of the study aspects in KI#1. This solution proposes to enhance NEF services, enable USS/UTM to request NEF assistance for pre-mission flight planning and in-mission flight monitoring.

There can be two options for USS /UTM to request NEF assistance information for pre-mission flight planning.

Option 1: (UAV initiated request)

-	In this option, a particular UAV which may not have the pre-mission flight information, can request USS/UTM to provide the pre-mission flight information after registering and establishing PDU session with network. After receiving the request from UAV then USS/UTM ask network for pre-mission flight planning assistance information.

Option 2: (USS/UTM initiated request)

-	In this option, USS/UTM which has knowledge about the UAV(s) like flight information when a particular UAV is going to fly, the flight path, timing information etc, can request network for pre-mission flight information for a single UAV or hroup of UAVs irrespective of the UAVs are registered or have established PDU Session.

### 6.1.3	Procedures and Parameters

#### 6.1.3.1	Procedure for NEF Assist Pre-mission Flight Planning

Option 1: (UAV initiated request)

Figure 6.1.3.1-1: Procedure for NEF Assist Pre-mission Flight Planning

1.	The UAV (or UAV-C) establishes a PDU Session for communication with the USS/UTM as described in clause 5.2.3 of TS 23.256 [2].

2.	The UAV (or via its paired UAV-C) requests Pre-mission flight planning service from USS/UTM. The request message includes identifier of the UAV (e.g. GPSI, CAA-Level UAV ID), information of the starting and ending points for the flight, requirements on the flight route (e.g. on time), and may include candidate flight route(s) if available.

3.	The USS/UTM derives information for the Pre-mission flight planning request and decides to request assistance information from NEF.

NOTE 1:	The content of Pre-mission flight planning service information derived at USS is out of scope.

NOTE 2:	The solution uses request/response since considers such approach is more suitable for this scenario. Pre-mission flight planning, after receiving the planned route from USS, the UAV is expected to plan by following the route. There is no need to keeps receiving the notifications. When it is required to keep receiving notification during flight, the solution belongs to in-mission monitoring.

4.	The USS/UTM sends Pre-mission flight planning assist request to NEF. The request message includes identifier of the UAV (e.g. GPSI), information of the starting and ending points for the flight, requirements on the flight route (e.g. on time), candidate flight route(s) (received from the UAV or local derived at the USS/UTM), and accuracy level of predictions relevant to the flight planning.

5.	The NEF maps parameters included in the request from the USS/UTM to information used by the 3GPP system (e.g. map the geographical area into an area of interest that is represented by a list of Cell IDs, gNB IDs or TAIs). The NEF determines services needed for the request and relevant NFs, e.g. NEF service on UAV tracking and mode (UAV location reporting mode, UAV presence monitoring mode, List of Aerial UEs in a geographic area), NWDAF analytics service (Movement Behaviour analytics), GMLC service (e.g. Ranging/Sidelink Positioning location).

6.	If the NEF cannot satisfy the requirements on flight planning (e.g. cannot get predictions relevant to the flight planning from NWDAF with the required accuracy level), the NEF responses to the USS/UTM to reject the Pre-mission flight planning assist request, and may include the detail reason.

7.	If being rejected in step 6, the USS/UTM may response to the UAV (or its paired UAV-C) to reject the Pre-mission flight planning request, may include the detail reason if available.

8.	The NEF performs steps 2-5 in clause 5.3.4 of TS 23.256 [2], and filter list of UAVs in the areas of interest.

9.	The NEF subscribes/requests for notification on Movement Behaviour analytics provided by NWDAF as defined in clause 6.21.4 of TS 23.288 [6]. The subscribe/request message include identifier of the UAV (e.g. GPSI) obtained in step 4. The other parameters included in the request are described in clause 6.1.3.3. The list of UAVs in the areas of interest output from step 8 can be used as inputs for the NEF to request the NWDAF on Movement Behaviour analytics.

10.	The NEF may request GMLC service(s), e.g. Ranging/Sidelink positioning location as defined in clause 6.20.3 of TS 23.273 [7]. The list of UAVs in the areas of interest output from step 8 can be used as inputs for the NEF to request the GLMC service on Ranging/Sidelink Positioning location.

11.	The NEF generates assistance information for pre-mission flight planning based on the information and analytics from steps 8-10. The assistance information may be the best matched route among the ones provided from the USS/UTM in step 4, or potential flight route(s) if candidate flight route(s) is not provided in step 4.

12.	The NEF responses to the USS/UTM with the assistance information.

13.	The USS/UTM decides flight planning by using the assistance information and flight planning mechanism which is out of scope.

14.	The USS/UTM sends response to the UAV (or its paired UAV-C) with the planned flight route.

NOTE 3:	USS/UTM are used to represent the consumer of the enhanced NEF services in Figure 6.1.3.1-1, the consumer can also be other entities, e.g. TPAE.

Option 2: (USS/UTM initiated request)

The call flow is similar to the option 1 with the below difference:

1 & 2. This Step 1 and 2 is not applicable.

3. The USS/UTM is already having the knowledge for UAVs for which pre-flight assistance information is needed.

4. Pre-mission flight planning assistance request can be for a single UAV or group of UAVs.

14. This Step 14 is not applicable.

#### 6.1.3.2	Procedure for NEF Assist In-mission Flight Monitoring

Figure 6.1.3.2-1: Procedure for NEF Assist In-mission Flight Monitoring

NOTE 1:	The NFs in Figure 6.1.3.2-1 represents all the other NFs beside GMLC and NEF/UAS NF involved in the procedure.

1.	The USS/UTM sends In-mission flight monitoring assist request to NEF. The request message includes identifier of the UAV (e.g. GPSI) and information of the flight route, monitoring mode (e.g. UAV location monitoring, fight route monitoring, flight environment monitoring, Ranging/Sidelink positioning location monitoring), report requirements (e.g. report format (event triggered or periodically), assistance information (e.g. the monitoring results, information generated from the monitoring results)), and may include planned fight route if fight route monitoring, list of UAVs in the areas of interest if flight environment monitoring, preset value for distance if Ranging/Sidelink positioning location monitoring.

2.	NEF maps the parameters in the request from the USS/UTM to information used by the 3GPP system. The NEF determines services needed for the request and relevant NFs, e.g. NEF service on UAV tracking (UAV location reporting mode, UAV presence monitoring mode, List of Aerial UEs in a geographic area), GMLC service (Ranging/Sidelink Positioning location).

3.	If the NEF cannot provide the requested assistance information for In-mission flight monitoring, the NEF responses to the USS/UTM to reject the In-mission flight monitoring assistance request, may include the detail reason if available.

4.	For UAV location monitoring, the NEF executes steps 2-5 in clause 5.3.2 of TS 23.256 [2] to obtain the location of the target UAV.

5.	For fight route monitoring, the NEF executes steps 2-4 in clause 5.3.3 of TS 23.256 [2]. Upon to the request in step 1 for reporting, the NEF may compare the monitored flight route with the planned fight route (if provided in step 1) and generate assistance information on whether the UAV is flight in right route.

6.	For flight environment monitoring, the NEF executes steps 2-5 in clause 5.3.4 of TS 23.256 [2] to obtain the information of aerial UEs in the same geographic area of the target UAV.

7.	For Ranging/Sidelink positioning location monitoring, the NEF requests GLMC service for Ranging/Sidelink Positioning location results as described in clause 6.20.4 of TS 23.273 [7], the procedure of SL-MT-LR for periodic, triggered Location Events (steps 1-20 for initiation the monitoring, steps 21-31 for monitoring periodic). The list of UAVs in the areas of interest output from step 6 can be used as inputs to step 7 for NEF to request GLMC service for Ranging/Sidelink Positioning location, if the UAVs is unknown in the areas of interest.

8.	The NEF notifies the USS/UTM the monitoring results (e.g. the location of the target UAV, indication on whether the UAV is flight in right route, the target UAV presence in areas of interest, the information of aerial UEs in the same geographic area of the target UAV, the distance of other UAVs and the target UAV and whether the distance is smaller than a preset value).

Steps 4-8 may be repeated for report monitoring results periodically.

NOTE 2:	USS/UTM are used to represent the consumer of the enhanced NEF services in Figure 6.1.3.2-1, the consumer can also be other entities, e.g. TPAE.

#### 6.1.3.3	Parameters in Request for Movement Behaviour Analytics

The USS acting as an Application Function communicates with the NEF which corresponds to the NF consumer in clause 6.21.4 of TS 23.288 [6].

Editor's note:	The final list of parameters is FFS.

The USS can either subscribe to notifications from the NEF (i.e. a Subscribe-Notify model) or request a single notification from the NEF (i.e. a Request-Response model). The USS request contains the following parameters:

-	Analytics ID = "movement behaviour";

-	Target of Analytics Reporting: any UE;

-	Analytics Filter Information:

-	Area of Interest (AOI): restricts the scope of the movement behaviour analytics to the provided area. The AOI may be described as shown in clause 5.5 of TS 23.273 [7];

-	Optionally, the list of analytics subsets that are requested among those specified in clause 6.21.3 of TS 23.288 [6];

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Optionally, preferred level of accuracy of the analytics;

-	Optionally, preferred level of accuracy per analytics subset (see clause 6.21.3 of TS 23.288 [6]);

-	Optionally, preferred granularity of location information: "longitude and latitude level";

-	Optionally, preferred orientation of location information: ("horizontal", "vertical", "both"); and

-	Optionally, maximum number of objects.

### 6.1.4	Impacts on services, entities and interfaces

UAS NF/NEF:

-	Enhance for supporting pre-mission flight planning assistance service.

-	Enhance for supporting in-mission flight monitoring assistance service.

NWDAF:

-	Enhance the inputs and the outputs of Movement Behaviour Analytics.

USS/AF:

-	Enhance for supporting UAV/UAV-C trigger pre-mission flight planning service.

-	Enhance for supporting in-mission flight monitoring service.

Editor's note:	It is FFS other possible impacts related to services, entities and interfaces.

* * * * End of Changes * * * *