3GPP TSG-WG SA2 Meeting #162	S2-2405294

Changsha, China, April 15th – 19th, 2024

Source:	Huawei, HiSilicon, China Telecom, Intel

Title:	KI#3, new solution: support traffic routing between local-DN and central-DN via the existing UP path of the PDU session

Document for:	Discussion/Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract: It is proposed to add a merged solution for PDU session based solutions to reach common understanding.

1. Discussion

The key issue#3 of TR 23.700-49 is to study the support of traffic routing between local DN and central DN. Several solutions agreed in SA2#161 meeting proposed to route traffic between local DN and central DN via the tunnel of the PDU session (sol#17, 18, 19, 20, 23). These solutions share some common understandings on the following issues:

-	Traffic between local DN and central DN should be routed via the tunnel of PDU session.

-	Information provided by AF to indicate 5GC to enable traffic routing between local DN and central DN with the assistance of IP replacement.

For some issues, these solutions have not reached a common understanding on how to do the IP replacement.

To address this KI more efficiently, it is proposed to add a new solution to merge the common understandings together and add ENs for issues that need further discussion.

2. Text Proposal

It is proposed to add the following solution to the TR 23.700-49.

# * * * * First change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |
| --- | --- | --- | --- |
|  | 1 | 2 | 3 |
| #1: Edge computing handling by I-SMF | X |  |  |
| #2: Edge computing handling by local SMF | X |  |  |
| #3: Reducing impact of DNS message handling on central SMF for EAS (re)discovery based on offload to L-SMF | X |  |  |
| #4: Enhanced EC Architecture with SMF selecting local SMF storing EC related information | X |  |  |
| #5: Enhanced EC architecture with AMF selecting local SMF | X |  |  |
| #6: Local management of EAS Deployment Information with local SMF | X |  |  |
| #7: EAS deployment information report from L-UPF | X |  |  |
| #8: Selecting an EAS server leveraging analytics |  | X |  |
| #9: Solution of local UPF and EAS (re)selection jointly considering N6 delay and EAS load |  | X |  |
| #10: L-PSA and EAS (re)selection based on N6 one-way and two-way delay measurement |  | X |  |
| #11: Provision weight factor of DNAIs from AF |  | X |  |
| #12: NWDAF and SMF-based EAS and local UPF (re)selection |  | X |  |
| #13: EAS Discovery taking account of EAS load in EASDF |  | X |  |
| #14: EAS selection considering DNS historical handling records |  | X |  |
| #15: The local EASDF assist for the EAS and local UPF (re)selection based on the AF provided N6 delay and EAS load information |  | X |  |
| #16: Local UPF and EAS (re)selection considering access network delay and N6 delay information by 5GC or AF |  | X |  |
| #17: EC Traffic Routing between local part of DN and central part of DN with IP replacement in EAS |  |  | X |
| #18: Supporting traffic routing between local DN and central DN within a PDU Session |  |  | X |
| #19: Traffic Routing between local DN and central DN over session breakout model |  |  | X |
| #20: EC Traffic Routing between local part of DN and central part of DN via PDU session |  |  | X |
| #21: Solution to traffic routing between local and central part of DN via tunnel(s) |  |  | X |
| #22: Establishment of connectivity between the local DN and central part of DN based on OAM |  |  | X |
| #23: Traffic steering between different parts of a DN |  |  | X |
| #X: support traffic routing between local-DN and central-DN via the existing UP path of the PDU session and IP replacement |  |  | X |

# * * * * Next change * * * * (all new text)

6.x	Solution #<X>: Support traffic routing between local-DN and central-DN via the existing UP path of the PDU session and IP replacement

6.x.1	Key Issue mapping

This solution is proposed to solve KI#3. It is the merged solution on using the PDU session to convey the traffic between local DN to central DN the existing UP path of the PDU Session.

6.x.2	General Description

This solution resolves KI#3 about traffic routing between L-DN and central DN based on the scenario that the application traffic is required to be processed at different locations.

Based on AF traffic influence request information, the 5GC determines to enable the traffic routing between L-DN and central DN via the tunnel of PDU session.

- For the UL traffic, the UE generates traffic and send it to the EAS (i.e. target IP address is EAS IP address), the traffic is routed to EAS to be processed firstly. Then after processed by EAS, the EAS generates the traffic and send it to the AS (i.e. the cloud server) for further processing.

- For the DL traffic, the AS (cloud server) generates traffic based on the received UL traffic and send it back to the EAS.

- For UL/DL traffic between EAS and AS, L-PSA also perform IP replacement (if required).

For the traffic sent from EAS to cloud AS, there are three options (option 1: sol#18; option 2: sol#17; option 3:sol#19) on how to handle the user plane traffic at L-PSA, including the IP replacement as described in Table 6.x.2-1, and traffic rules configured at the UL CL/BP as described in Table 6.x.2-2. UL traffic is the traffic from EAS to AS and DL traffic is the traffic from AS to EAS.

Table 6.x.2-1 Traffic between local DN and central DN for each option

| Options | UL Traffic generated by EAS | How L-PSA associate the UL traffic with PDU session | How L-PSA performs IP replacement for UL traffic | DL Traffic generated by AS | How PSA associate the DL traffic with PDU session | How L-PSA performs IP replacement for DL traffic |
| --- | --- | --- | --- | --- | --- | --- |
| Option 1 (sol#18) | Source IP: EAS IP Target IP: AS IP | Based on IP 5-tuple (NOTE 3) | Replace the source IP to UE IP2. (NOTE 4) | Source IP: AS IP Target IP: UE IP 2 | Based on UE IP 2 (target IP address) | Replace the target IP to EAS IP. |
| Option 2 (sol#17) | Source IP: UE IP Target IP: EAS IP or AS IP | Based on UE IP (source IP address) | None. | Source IP: AS IP Target IP: UE IP | Based on UE IP (target IP address) | None. |
| Option 3 (sol#19) (NOTE 1) | Source IP: EAS IP 2 (NOTE 2) Target IP: UE IP | Based on UE IP (target IP address) | Replace the Source IP to UE IP address, and Target IP to AS IP address. | Source IP: AS IP 2 Target IP: UE IP | Based on UE IP (target IP address) | Replace the target IP to EAS IP. Replace the source IP to AS IP. |
| NOTE 1: For option 3, to align with the existing mechanism, it is suggested not to perform IP replacement at ULCL/BP. NOTE 2: EAS IP 2 is an IP address assigned by EAS, and is different from the IP address EAS used to communicate with UE. NOTE 3: The IP 5-tuple is the IP 5-tuple of traffic between EAS and AS (i.e. EAS IP address, port number and AS IP address, port number) NOTE 4: UE IP 2 is an IP address assigned by PSA, and is different from the UE IP included in the UL traffic. NOTE 5: It is assumed that there is a tunnel configured between the L-PSA and EAS. NOTE 6: AS IP 2 is an IP address assigned by AS, and is different from the IP address of AS used to communicate with UE. |  |  |  |  |  |  |

Table 6.x.2-2 Traffic routing rule configured at ULCL/BP for each option

| Options | Traffic from L-PSA | Traffic from PSA |
| --- | --- | --- |
| Option 1 (sol#18) | Target IP is UE IP address: Route to UE (RAN). Others: Route to PSA. | Target IP is UE IP address: Route to UE (RAN). Others: Route to L-PSA. |
| Option 2 (sol#17) | Target IP is UE IP address: Route to UE (RAN). Others: Route to PSA. | Source IP is AS IP: Route to L-PSA for specific source AS IP Others: Route to UE (RAN). |
| Option 3 (sol#19) | Target IP is UE IP address: Route to UE (RAN). Others: Route to PSA. | Source IP is AS IP 2: Route to L-PSA Others: Route to UE (RAN). |

NOTE 1:  Option 2 and 3 assume the EAS can do IP replacement for the UL and DL traffic, as such there is no need to do IP replacement in 5GC.

NOTE 2: Option 3 assumes the AS can assign two IP addresses, one is for directly communicate with UE and the other is for communicate with EAS.

6.x.3	Procedures

2. Packet detection and IP replacement configuration

1. AF request for local traffic routing

UE

SMF

PCF

ULCL/BP

L-PSA

PSA

EAS

AF/NEF

AS

(cloud server)

3. traffic routing rule configuration

5.Traffic from L-DN to central DN

6.Traffic from central DN to L-DN

4. Notify AF the user plane has been configured

0b. Traffic from UE

7.Traffic from EAS to UE

0a. PDU session establishment

Figure 6.X.2-1: High level procedure for supporting traffic routing between local DN and central DN via PDU session

Traffic from UE to EAS via the existing procedure.

- The UE establishes a PDU Session as described in clause 4.3.2.2.1 of TS 23.502 [3]. The UE does the DNS query and finds the EAS. The UL CL/BP is inserted per existing procedure as defined in clause 6.2.3 of TS 23.548[5]. UE send the traffic to the EAS, i.e. the target IP address is EAS IP address, and UL CL/BP route the traffic to L-PSA.

Traffic from EAS to central AS (i.e. Cloud server) is routed via the tunnel of PDU session

- The AF requests the SMF to perform traffic routing between local DN and central DN via the existing AF traffic influence procedure.

Following information are included in the information sent from AF to PCF: indication for traffic routing between local DN and central DN, traffic flow description for UL and DL (e.g. IP 5-tuples), target PDU session (e.g. associated UE IP address), QoS requirement of the indicated traffic flow.

Editor’s note: Whether other IEs should also be included in the AF request is FFS.

- Based on AF request in step 1, the SMF configures UPF(s) (i.e. L-PSA and/or PSA) to detect traffic related to one dedicated PDU session, and performs IP replacement at L-PSA. The information for SMF to instruct to L-PSA/PSA is per the information described in Table 6.x.2-1. This step includes the following handling:

For UL traffic between local DN and central DN sent by EAS:

2a. The SMF configures the L-PSA to detect the indicated traffic flow and associate the indicated the traffic flow with the PDU session per the received traffic flow description and target PDU session information.

2b. The SMF configures the L-PSA to perform IP replacement for UL traffic.

For DL traffic between local DN and central DN sent by AS:

2c. the SMF configures the PSA to detect the indicated traffic flow and associate the indicated the traffic flow with the PDU session per the received traffic flow description and target PDU session information.

2d. The SMF configures L-PSA performs IP replacement for DL traffic.

NOTE 1: If UE IP 2 is used as described in Table 6.x.2-1, SMF also needs to request another UE IP address from PSA before step 2b.

- The SMF configures the traffic routing rule on ULCL/BP to route traffic between L-DN and central DN. The information for SMF to instruct to UL CL/BP is detailed in Table 6.x.2-2.

- The SMF notifies AF the traffic routing path between L-DN and central DN has been configured via PCF and/or NEF as described in clause 4.3.6.3 of TS 23.502 [3].

- Traffic from L-DN (i.e. after processed by EAS) is sent to central DN via the tunnel of PDU session (i.e. via L-PSA, ULCL/BP and PSA).

The L-PSA determines which PDU session to convey the traffic per N4 rule configured in step 2. And the UL-CL/BP routes the traffic to PSA per the N4 rule configured in step 3.

- Traffic from central DN (i.e. after processed by cloud server) sent to L-DN via the tunnel of PDU session (i.e. via PSA, ULCL/BP and L-PSA).

The PSA determines which PDU session to convey the traffic per the N4 rule configured in step 2. And UL-CL/BP route the traffic to L-PSA per the N4 rule configured in step 3.

- Traffic sent from EAS to UE.

The L-PSA determines which PDU session to convey the traffic per UE IP address. And UL-CL/BP route the traffic to RAN per the N4 rule configured in step 3.

NOTE 2: 	All the traffic is within the PDU Session, thus operators can identify it with SDF template in a PCC rule and apply appropriate QoS parameter as described in clause 6.3 TS 23.503 [4]. The PCF can generate QoS parameter considering the QoS requirement required by AF.

NOTE 3: 	All the traffic is within the PDU Session, thus operators can identify it with SDF template in a PCC rule and apply appropriate charging key and charging method (e.g. neither of online or offline charging) as described in clause 6.6 TS 23.503 [4]. There can also be Sponsor Identifier if it is charging for 3rd party as described in clause 6.6 TS 23.503 [4].

6.x.4	Impacts on services, entities and interfaces

SMF (applies for all three options):

- Generate traffic routing rule on ULCL/BP and PDR, FAR on L-PSA/PSA based on AF influence as described above.

- Inform AF the routing path has been configured.

NOTE X: Details of the traffic routing rule, PDR, FAR are different for three options and described in UPF impacts.

L-PSA:

-	detect traffic between local DN and central DN (based on IP 5-tuple for option1, based on UE IP for option 2,3) and route the traffic to ULCL/BP.

-	For option 1,3: Perform IP replacement for traffic between local DN and central DN.

ULCL/BP:

-	Route traffic from EAS to AS based on Table 6.x.2-2.

PSA:

-	For option 1: assign UE IP 2 for IP replacement.

AF (applies for all three options):

-	inform 5GC to configure the traffic between local DN and central DN.

# * * * * End of changes * * * *