SA WG2 Meeting #162	S2-2405436

15 April – 19 April, 2024, Changsha, China       (Revision of S2-2404959, 5393, Merge of S2-2404382)

Source:	InterDigital Inc., Deutsche Telekom, ZTE

Title:	Key Issue #2, Interim Conclusion Principles

Document for:	Approval

Agenda Item: 	19.8

Work Item / Release:	FS_UIA_ARC / Rel-19

Abstract of the contribution: This paper proposes conclusion principles for key issue #2 based on the informal collection of company views prior to SA2 #162.

# 1	Discussion

This paper proposes conclusion principles for key issue #2 based on the informal collection of company views prior to SA2 #162.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-32 V0.2.0.

* * * * Start of Changes – All New Text * * * *

## 8.X	Key Issue #2 Conclusions

The following principle is agreed for the “how the network restricts the usage of User Identifiers, including in roaming scenarios” aspect of key issue #2.

- Existing PCC Rules framework can be used to implement rules for a user and provide service differentiation (e.g. by limiting the data rates that are accessible to the user). The PCF can:

  - obtain information from the User Profile to derive PCC Rules for a user, OR

  - be configured by the AF, or obtain information from the UDR that was configured by an AF, to influence the derivation of PCC Rules for a user.

Editor’s note: 	It is FSS whether the PCF obtains information directly from the user profile or is configured by another NF or AF.

Editor’s note: 	It is FSS how the network restricts other aspects of user activities (e.g. the number of simultaneously active User Identifiers per UE).

Editor’s note: 	The above conclusion principles are only tentatively agreed.

* * * * End of Changes * * * *