3GPP SA WG2 Meeting #166-AdHoc-e	S2-2500162r06

Electronic Meeting, 20-24 January 2025

# Title:	[Draft] Reply LS on multi-modality awareness

# Response to:	Reply to SA2 LS on multi-modality awareness (R2-2409272/S2-2500014) and to LS on multi-modality awareness (SP-241921/S2-2500074)

# Release:	Release 19

# Work Item:	NR_XR_Ph3-Core

Source:	SA2

To:	RAN2; RAN3

Cc:	SA4, RAN, SA

Contact Person:

Name:	Sebastian Speicher

E-mail Address:	speicher AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.501 CR5893

1. Overall Description:

SA2 would like to thank SA, SA4, RAN2 and RAN3 on their LSs on multi-modality awareness at RAN. Regarding multi-modality awareness, RAN2 agreed that the multi-modality information (e.g., MMSID), can be used for joint admission control and QoS flow to DRB mapping. RAN3 agreed that it is technically feasible to enhance NGAP to provide the MMSID to NG-RAN node. SA4 has indicated that it is only the application that knows whether all its constituting data flows are critical, and not 5GS.

Based on the above, SA2 would like to provide the following feedback:

- Currently, MMSID is provided by the AF and it is not guaranteed that any 2 AFs will not use the same MMSID unless an SLA is in place between Operator and Application Service providers.

- Some companies have expressed their concern on using MMSID for joint admission control based on the following SA4 feedback, while some companies do not have the concern.

“From an XR service perspective, it is generally desirable that the gNB should try to grant all associated QoS Flows for the XR service whenever possible. However, not all applications require all QoS flows to be granted. If the gNB cannot grant all associated QoS Flows, only the application layer can determine how to handle the case when the QoS flow for one modality cannot be established e.g. due to RAN resource limitations. For instance, it may be acceptable to an application if the QoS flow for video cannot be established with the requested QoS requirements but it may not be acceptable if the QoS flow for audio cannot be established.  The conditions under which particular QoS flows are considered essential for an application to continue are application specific and cannot be determined by 5GS.  “

SA2 has not discussed traffic synchronization by packet level synchronization and coordinated PDU Set Discard across different multi-modal flows.

2. Actions:

To RAN2 and RAN3:

ACTION: 	SA2 kindly requests RAN2 and RAN3 to take the above information and approved CR into account and to provide any additional feedback on the exact use of MMSID as NR_XR_Ph3-Core work progresses at earliest possible within Meeting SA2#167.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #167		17-21 February 2025	Athens, Greece

TSG-SA2 Meeting #168		7-11 April 2025	Goteborg, Sweden