3GPP SA WG2 Meeting #167	S2-2501495

Athens, Greece, 17-21 February 2025

# Title:	[Draft] LS response on OSAppID usage by AppToken use case

# Response to:	LS from GSMA TSG network Slicing Group (S2-2501377)

# Release:	Release 19

# Work Item:	TEI19

Source:	SA2

To:	GSMA TSG Network Slicing Group

Cc:	SA3

Contact Person:

Name:	Stefano Faccin

Tel. Number:

E-mail Address:	sfaccin AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	N/A

1. Overall Description:

SA2 thanks GSMA TSG Network Slicing Group for the LS. SA2 has discussed the material provided and the questions submitted, and has the following responses:

- Do the existing 3GPP specifications allow the network to use application identities that are generated by an application backend to populate URSP in step 2?

SA2 Response: 3GPP specifications do allow URSP rules population influenced by an Application Function (3rd party backend)

Regarding the remaining questions, SA2 provides a series of comments and observations:

## User Privacy: GSMA states that:

“The App Token is a piece of information that the UE OS can obtain from specific applications and allows to uniquely identify an application across the complete ecosystem (application developer, MNO, subscriber UE) whilst protecting the end user privacy. This means that the UE OS does not need to use the information it has about the identity of an application when comparing against the OSAppID field from the URSP rules received from the MNO, but to use an anonymous identifier received from the MNO through TS.43 interface for that comparison.”

It is unclear to SA2 how the AppToken protects user privacy:

- The actual identity of the application requesting connectivity to the OS cannot be hidden from the OS, since it is the application itself that sends the AppToken to the TS.43 client running in the OS

- The actual identity of the application requesting connectivity to the OS cannot be hidden from the MNO, since if the application provider via its backend requests the MNO to create URSP rules with the “anonymized” AppID, then the MNO knows the application associated to the token.

It seems that if there is a privacy or security concern to be addressed, further study is required by SA3 to identify the threat and the validity of this or alternative solutions.

## Signalling Impact Versus MNO Awareness of the app: Analyzing the message flow from the material sent from GSMA, it seems that when the 3rd party app needs connectivity, it triggers the OS to validate the AppToken with the OES, which means the OES (i.e. the MNO) is aware the application is starting and thus it is not protecting the user privacy wrt which application is being used. Moreover, this requires additional signalling everytime the 3rd party app requires connectivity, and it seems the 3rd party app needs to have the AppToken verified before it can connect to the network, which may lead to additional delay with impact on the user experience. Can GSMA clarify exactly what privacy the mechanism is intended to protect considering the additional signalling being introduced?

## Impact on 3GPP NFs: The solution may have impact on the PCF: if the URSP rules are configured on a per subscription basis, PCF would need to know whether the UE is configured with the anonymized App_Token or not, and thus an interaction between the entitlement server and PCF would be required, but this aspect is not addressed at all in the proposal. Has GSMA considered the need for interaction between the entitlement server and the PCF?

## Time Validity of AppToken: The proposal is unclear as to whether the AppToken has a limited time validity or it has no expiration. This would need to be clarified to understand the impact on the creation and handling of URSP rules, and it has implications on the UE side in terms of how many timers would need to be run (one per app?). Also, can an App_token be invalidated after being issued? If not, why?

## Impact on applications: it is evident that applications need to be modified in order to enable the application to pass the AppToken to the OS. Is this the intention and the assumption? Can GSMA explain why they do consider feasible having to rewrite application software to enable the AppToken?

2. Actions:

To GSMA:

ACTION: 	Kindly asks GSMA to take the above into account and provide answers to the questions above.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #168		7-11 April 2025	Goteborg, Sweden

TSG-SA2 Meeting #169		19-23 May 2025	Fukuoka, Japan