3GPP TSG-SA2 Meeting #167	S2-2501677

# 17 - 21 February, 2025, Athens, Greece 		(revision of S2-2500658)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1357 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on the contents of VFL related services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the VFL procedures and the HFL service operation description in TS 23.288, this CR is proposed to update the contents of VFL related services to align with the VFL procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to describe the contents of VFL inference service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The contents of VFL related services are not clear and missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.4.2, 6.2H.X(new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | [The following contents are added in rev 1] Update clause 6.2H.2.4.2 to address the following ENs. Editor's note: It is FFS additional parameters are needed to send from the VFL server to VFL client. Editor's note: It is FFS additional parameters are needed to send from the VFL client to VFL server. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.2H.2.4.2	Inference procedure for vertical federated learning when untrusted AF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when untrusted AF is acting as VFL server

The inference procedure when untrusted AF is acting as VFL server may be triggered by a request or subscription from a 5GC consumer NF or internal service logic of the AF acting as VFL server. If triggerd by internal service logic of the AF acting as VFL server, the steps 0, 1,7 and 8 are skipped. The inference procedure triggerd by internal service logic of the AF acting as VFL server is out of 3GPP scope.

0.	Same as step 0 in clause 6.2H.2.4.1.

1.	Same as step 1 in clause 6.2H.2.4.1, to NEF using Nnef_Inference_subscribe. NEF forwards the subscription request to AF using Naf_Inference_subscribe.

Editor's note:	When the AnLF determine the VFL server AF is FFS. For example, during the training phase.

Editor's note:	The service name between the AnLF and untrusted AF as VFL server is FFS.

2.	Same as step 2 in clause 6.2H.2.4.1, to NEF using Nnef_VFLInference_subscribe including parameters as described in clause 6.2H.X. An untrusted AF includes the external NWDAF ID and sends the request to the NEF.

NEF converts any received external identifiers to internal identifiers and forwards the subscription request to NWDAF using Nnwdaf_VFLInference_subscribe including parameters as described in clause 6.2H.X.

3.	Same as step 3 in clause 6.2H.2.4.1.

4.	Same as step 4 in clause 6.2H.2.4.1.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

5.	Same as step 5 in clause 6.2H.2.4.1, to NEF using Nnwdaf_VFLInference_Notify including parameters as described in clause 6.2H.X.

NEF converts any received internal identifiers to external identifiers and forwards the subscription notify to the untrusted AF using Nnef_VFLInference_Notify including parameters as described in clause 6.2H.X.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

6.	Same as step 6 in clause 6.2H.2.4.1.

7.	Same as step 7 in clause 6.2H.2.4.1, to NEF using Naf_Inference_Notify.

NEF converts any received internal identifiers to external identifiers and forwards the subscription Notify to NWDAF using Nnef_Inference_Notify.

8.	Same as step 8 in clause 6.2H.2.4.1.

# * * * * Next change (all new text) * * * *

### 6.2H.X	Contents of VFL Inference service

The consumers of the VFL inference services may provide the input parameters in Nnwdaf_VFLInference service or Naf_VFLInference service or in Nnef_VFLInference service as listed below:

-	Analytics ID: identifies the analytics for which the ML Model is requested to be used for inference.

-	(Only for Inference subscribe service) A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from the VFL client with the subscription.

-	VFL Correlation ID: identifies the VFL procedure (i.e. training and inference).

-	Target of VFL inference: indicates the object(s) for which data for VFL inference is requested, i.e. a list of SUPIs, group of UEs identified by a list of Internal-Group-Ids or External-Group-Ids.

-	(Only for Inference subscribe) Subscription Correlation ID.

-	[Optional] VFL inference filter.

-	[Optional] Inference target period: indicates the time interval for inference.

The VFL client provides to the consumer of the VFL inference service operations the output information in as listed below:

- (Only for Inference notify) Notification Correlation Information.

- Intermediate results: the output of the client local model, which is used for the VFL server to generate the VFL inference results.VFL correlation ID.

- Analytics ID.

# * * * * End of changes * * * *