SA WG2 Meeting #167	S2-2501889

Athens, GR, 17th Feb – 21st Feb, 2025

Source:	Huawei, HiSilicon

Title:	Discussion on UUAA Unsubscribe Procedure

Document for:	Discussion

Agenda Item:	8.7

Work Item / Release:	Rel-17

Abstract of the contribution: This paper discusses the LS from CT4 on UUAA Unsubscribe Procedure.

Discussion

CT4 sent the LS on UUAA Unsubscribe procedure as below.

CT4 defines two service operations (i.e., AuthenticateAuthorize and Notification) for the UAS NF/NEF API in TS 29.256, which follows the stage 2's specification TS 23.256. CT4 notices that AMF and SMF can trigger the unsubscribe operation during the UUAA-MM and UUAA-SM procedures, as specified in TS 23.256. In clause 5.2.2.2, it reads: If the UE is deregistered as per clause 4.2.2.3 of TS 23.502 [3], then the AMF shall unsubscribe to UAS NF and then UAS NF/NEF may clear the UUAA-MM context and update USS. And in clause 5.2.3.2: If the PDU session is released as per clause 4.3.4 of TS 23.502 [3] then the SMF shall unsubscribe to UAS NF/NEF and then UAS NF/NEF may clear the UUAA-SM context and update USS. However, the Unsubscribe service operation for Nnef_Authentication service is not defined in neither TS 23.256 nor TS 29.256, and the UUAA context in the UAS NF/NEF may not be properly cleared. CT4 kindly asks SA2 to check whether Unsubscribe service operation for Nnef_Authentication service is needed in TS 23.256 and provide feedback.

Observation-1: The key point of the text referenced in the LS is how to let UAS NF/NEF know UE is deregistered or PDU session is released.

Observation-2: The question asked by CT4 is whether using a Unsubscribe service operation of Nnef_Authentication service for UUAA context removal in this case.

Clause 4.4.1.2.2.1 of 3GPP TS 23.256 specifies that the Naf_Authentication service can be used to implicitly subscribes to USS, and the implicit subscription is implicitly released by USS when the corresponding authentication session is removed.

“When creating an authentication session, the UAS NF/NEF implicitly subscribes to USS about notification related with the authentication/authorization (e.g. re-authenticate, update authorization data or revoke the UUAA authorization). This implicit subscription is implicitly released by USS when the corresponding authentication session is removed (e.g. in the case of re-authentication failure and USS indicating to release network resource, or in the case of authorization revocation).”

Clause 4.4.1.1.2 of 3GPP TS 23.256 specifies that the Nnef_Authentication service can be used to implicitly subscribes to UAS NF/NEF, and the implicit subscription is implicitly released by UAS NF/NEF when the corresponding authentication session is removed.

“When creating an authentication session, the AMF/SMF implicitly subscribes to NEF about notification related with the authentication/authorization (e.g. re-authenticate, update authorization data or revoke the UUAA authorization). This implicit subscription is implicitly released by UAS NF/NEF when the corresponding authentication association is removed (e.g. in the case of re-authentication failure and USS indicating to release network resource, or in the case of authorization revocation).”

Observation-3: Current clause 4.4.1.2.2.1 and 4.4.1.1.2 reflects that AMF/SMF implicitly subscribes to UAS NF/NEF, and UAS NF/NEF implicitly subscribes to USS, i.e., there is no Subscription service operation of Nnef_Authentication or Naf_Authentication defined in TS 23.256.

Observation-4: The corresponding authentication association can be removed in the case of re-authentication failure and USS indicating to release network resource, or in the case of authorization revocation. The implicit subscription is implicitly released, which is not related with UE registration status or PDU session status.

Observation-5: When corresponding authentication association is not removed, but UE is deregistered or the PDU session is released, it can be regarded as an abnormal case.

Based on description in Clause 4.3.2.3 of TS 23.502 for Secondary authorization/authentication, UE unreachable event needs to be informed to DN-AAA server which then can decide to keep the PDU Session or request to release the PDU session.

During Secondary Re-authentication, if the SMF receives an indication from the AMF that the UE is unreachable then it informs the DN-AAA Server that UE is not reachable for re-authentication. Based on this indication from SMF, the DN-AAA Server may decide to keep the PDU Session or request to release the PDU session.

Observation-6: Similar with UE unreachable in Secondary authorization/authentication scenario, UE deregistration or PDU session release event can be notified to UAS NF/NEF and USS, which then can release UUAA context and implicitly terminate implicit subscription from AMF/SMF and UAS NF/NEF. The event notification does not rely on new service operation or procedure, it can be assumed that the subscription to deregistration event and PDU session release event is implicitly performed by the USS and UAS NF when responding to the UUAA authentication request.

Proposal-1: It is proposed to clarify in TS 23.256 that the UE deregistration or PDU session release event can be notified to the UAS NF/NEF and USS, which can then release the UUAA context and implicitly terminate the implicit subscription from the AMF/SMF and UAS NF/NEF.

Proposal

It is proposed to clarify in TS 23.256 that the UE deregistration or PDU session release event can be notified to the UAS NF/NEF and USS, which can then release the UUAA context and implicitly terminate the implicit subscription from the AMF/SMF and UAS NF/NEF.