3GPP SA2#167

S2- 2502022

Athens, GR, Feb 17-21, 2025

Source:	 	IIT Kharagpur

Title:		Energy-Efficient User Plane for 6G Core Service-Based

Architecture Leveraging Nndwaf_MLModelProvision Service

Agenda item:             19.4.2

Document for:	Discussion

Contact:		Nairit Das (nairit.das@kgpian.iitkgp.ac.in),

Sudip Misra (sudipm@iitkgp.ac.in), and

Aneek Adhya (aneek@gssst.iitkgp.ac.in)

# Abstract

This contribution discusses the following topics:

- Inclusion of an energy efficiency module (EEM) within the NWDAF which executes a predictive algorithm to select the most energy-efficient UPF for new incoming flows.

- Inclusion of “Nnwdaf_MLModelProvision” service into the “Procedure for UE Communication analytics” through appropriate service flows by integrating the EEM service flow sequences as detailed in figure 5.7.7-1 titled "Procedure for UE Communication Analytics", in 3GPP TS 29.552 v19.0.0 (2024-09).

In this contribution, we propose a system and mechanism to realize a green energy-efficient 6G core by efficiently selecting a UPF among a plurality of UPF nodes to be allocated for each new application flow. Specifically, we propose the introduction of an Energy Efficiency Module (EEM) within the Network Data Analytics Function (NWDAF), which is responsible for gathering historical energy consumption data for each UPF associated with the network. Using a Long Short Term Memory (LSTM), the probable energy consumption for each UPF is predicted in the next instant. The UPF predicted to consume the least energy is selected to be the anchor UPF. The contribution also proposes the inclusion of the “Nnwdaf_MLModelProvision” service into the “Procedure for UE Communication analytics” through appropriate service flows by integrating the EEM service flow sequences as detailed in figure 5.7.7-1 titled "Procedure for UE Communication Analytics", in 3GPP TS 29.552 v19.0.0 (2024-09).

# Discussion

2.1 Background

The User Plane Function (UPF) is an essential user plane Network Function (NF) within the present Fifth-Generation (5G) and upcoming Sixth-Generation (6G) core Service-Based Architecture (SBA) responsible for processing, routing, and transmitting user data. UPF acts as a connection to external Internet Protocol (IP) networks and serves as a reference point for User Equipment (UE). Of special interest is the data analytics function in 6G, namely the Network Data Analytics Function (NWDAF), which consolidates several types of data from other network functions such as the Access and Mobility Function (AMF), Session Management Function (SMF), and UPF, and provides insights to network operators in the form of output results by executing different data analytics and/or ML algorithms. The NWDAF operates in the form of a subscribe/unsubscribe model, whereby other NF consumers may choose to subscribe to specific analytics services from NWDAF, while the latter may, in turn, choose to subscribe for different kinds of data from different NFs. On the other hand, an effective and energy-efficient selection of a UPF node to act as the anchoring UPF for a new flow request from a UE is a significant concern. Improving energy efficiency reduces operational expenses for telecom service providers and aligns with the United Nations' Sustainable Development Goals (SDGs) by minimizing the carbon footprint. In this light, it is proposed to utilize specific services of the NWDAF, and in particular, introduce an Energy Efficiency Module (EEM) within the NWDAF, which helps in efficient UPF selection based on a predictive analysis of energy consumption by each of the UPFs associated with the network.

2.2 Proposed Methodology

2.2.1: Overview

With high-speed traffic flow through 6G core networks, the efficient selection of a UPF node to which a new flow would be allocated to improve energy efficiency is a critical issue. Towards this, the contribution proposes a system, architecture, and methodology for efficient UPF selection to allocate to a new user application flow based on the predictive energy consumption analysis using the services from the NWDAF [2]. The NWDAF is an essential component of the 5G core SBA, as specified in the third-generation partnership project (3GPP) technical document TS 23.288 [1]. Specifically, the contribution proposes the introduction of an Energy Efficiency Module (EEM) to be integrated with the existing NWDAF that executes learning algorithms over the historical energy consumption data for each UPF node. This data includes the amount of energy consumed by each UPF for processing all the flows that are allocated to them. We also propose the procedure to exchange the relevant set of messages between the NWDAF and the other NFs, primarily the SMF and the UPF, to facilitate the implementation of the EEM module. The proposed contribution is aimed at improving the “Procedure for UE communication analytics [2] with the integration of Nnwdaf_MLModelProvision service [2] [3]” within the future 6G service level agreement (SLA) framework as defined in the 3GPP technical document TS 29.552 v19.0.0 (2024-09) [2]. When a new flow request arrives, connection establishment occurs in the control plane, and the SMF is used to anchor a particular UPF to the request. In order to achieve this, the EEM continuously executes a time series-based learning algorithm that predicts the energy consumption in the next interval slot for all UPFs based on the previous sampled values of energy expended for each UPF. The NWDAF receives information from the SMF on the amount of energy consumption by each UPF registered with the network for all the flows that the UPF manages. This information includes comprehensive historical data (sampled at a regular time interval) of all the flows that a given UPF manages, including details such as the nature and type of flow application, data rate, flow start time, and flow completion time. For each such UPF, the EEM collects the data and computes the energy expended for each flow. The UPF, which is predicted to consume the least energy in the next interval, is chosen to be the anchor UPF for the new flow, and accordingly, the UPF is assigned. This contribution proposes a Long Short-Term Memory (LSTM) based predictive algorithm; however, the EEM is proposed to support any other suitable algorithm that may provide time-series-based predictive analysis. Furthermore, we have suggested modifications to the signaling flow handled by different NFs (including NWDAF). The model training logical function (MTLF) that operates inside NWDAF offers various services to facilitate machine learning (ML) services. We propose an integration of the Nnwdaf_MLModelProvision service [2] [3] to enable LSTM-based predictive capability.

Observation 1: The enhancement of energy efficiency and energy saving in 5GS can be achieved by selecting an energy-efficient UPF as an anchoring UPF among the plurality of UPF in 5GC.

Proposal 1: An Energy Efficiency Module (EEM) within NWDAF is required to facilitate the

energy-efficiency in 5GC to select the anchoring UPF.

Proposal 2: Using a predictive learning algorithm, e.g. LSTM (not limited to), can select the best energy-efficient UPF based on historical energy consumption data.

2.2.2: EEM

The proposed NWDAF-Assisted Energy-Efficient 6G Core User Plane Architecture (Fig. 1) is described below.

Fig. 1 NWDAF-Assisted Energy-Efficient 6G Core User Plane Architecture.

Fig. 1 shows a diagram for the representative system model architecture of the proposed energy-efficient user plane for the 6G core. 6G core consists of a plurality of UPFs, which act as connection points to external IP networks and act as a reference point for UE. Within the 6G core, UPF routes data packets between (UEs) through the radio access network (RAN) via the N3 interfaces (as specified by 3GPP TS 23.501) [4] and the backend destination servers via the N6 interfaces. The UPFs are connected via appropriate service-based interfaces (SBI) with other NFs of the 6G control plane, specifically the NWDAF through the SMF. Residing within the NWDAF is an energy-efficient module (EEM) that is responsible for executing relevant logic with respect to selecting UPFs for every new incoming UE data flow. Specifically, the EEM within NWDAF selects UPF, which is predicted to consume the least amount of energy in a successive time interval. The EEM obtains relevant data and parameters about each UPF from the NWDAF. The proposed EEM within NWDAF is unique and novel in that it is proposed to be included within the 6G service-based architecture. The different UEs correspond to different 6G applications, including but not limited to augmented reality (AR), virtual reality (VR), extended reality (XR), social metaverse, internet of things (IoT), and massive machine-type communication, among others.

Proposal 3: The architecture for the NWDAF-assisted energy-efficient 6G core User Plane is illustrated in Fig. 1.

2.2.3: LSTM within EEM

Table I below illustrates the energy consumption data of a UPF for a certain flow, together with the associated residence period, namely period Time In and Time Out. The energy consumption value is determined according to equation (8) referenced in section 2.2.3.1: Energy Model. Table II serves as the input dataset for the LSTM model. It comprises the energy consumption value of a UPF, encompassing the idle energy consumption for a specific sampling interval (e.g., 5 minutes), in addition to the timestamp value. Specifically, Table II is obtained from Table I by the proposed EEM by sampling the total energy consumed by the UPF for all the flows that the UPF processes every 5 minutes (or a specific duration as set by the network administrator). From this data, as shown in Table II, the LSTM predicts the possible energy consumption value for the next instant based on the historical energy consumption of the previous few instances. Both Table I and II shown here are with respect to a single UPF, but the same set of tabular information is maintained by the EEM for all the UPFs registered with the network.

Table: I- Energy consumption table with time in and time out for different flows without idle energy consumption

| Date | Time In | Time Out | Energy Consumption |
| --- | --- | --- | --- |
| 01-01-2024 | 06:00:00 | 06:00:45 | 2.204505 |
| 01-01-2024 | 06:01:00 | 06:01:41 | 2.019763 |
| 01-01-2024 | 06:02:00 | 06:02:54 | 2.678184 |
| 01-01-2024 | 06:03:00 | 06:04:14 | 3.67188 |
| 01-01-2024 | 06:04:00 | 06:04:44 | 2.187944 |
| 01-01-2024 | 06:05:00 | 06:06:10 | 3.42923 |
| 01-01-2024 | 06:06:00 | 06:07:01 | 2.946422 |
| 01-01-2024 | 06:07:00 | 06:07:44 | 2.02983 |
| 01-01-2024 | 06:08:00 | 06:09:04 | 2.702432 |
| 01-01-2024 | 06:09:00 | 06:10:05 | 3.215095 |
| … | … | … | … |

Table: II- Energy consumption values with five-minute intervals with the idle energy consumption for five-minute intervals.

| Interval Start | Total Energy |
| --- | --- |
| 01-01-2024 06:30 | 2952009.914 |
| 01-01-2024 06:35 | 2952012.856 |
| 01-01-2024 06:40 | 2952010.285 |
| 01-01-2024 06:45 | 2952009.917 |
| 01-01-2024 06:50 | 2952004.384 |
| 01-01-2024 06:55 | 2952015.239 |
| 01-01-2024 07:00 | 2952009.929 |
| … | … |

2.2.3.1: Energy Model

In references [6] and [7], A. Vishwanath et al. describe incremental energy per bit of network equipment like core routers, edge routers, and Ethernet switches. P0 denotes the idle power consumption of networking devices (e.g., a core router); Pc signifies the power consumption at a data rate of C bits/sec, and the maximum power consumption is expressed as Pt for Ct bits/sec. We can express the slope m, which represents incremental energy per bit, as

|  | m = (Pt - P0) / Ct | (1) |
| --- | --- | --- |

According to [6] taking Ct= 4480 Gbps, Pt= 12300W and P0= 9840W, the calculated m as 0.5 nJ/bit. We have assumed that the UPF runs on this type of networking device (for instance, the core router CRS-3). The network administrator can adjust the value of 'm' in accordance with the device specifications. Our energy usage framework is as follows:

|  | e = r × m × t+ k0 | (2) |
| --- | --- | --- |

The total amount of energy consumption for a particular flow, denoted by "e," is calculated by multiplying the data rate (r) (in bits/sec), the energy per bit (m) (in nJ/bit), and the time the flow was in the device (t) (in seconds). Moreover, the idle energy consumption is represented by k0 (J).

2.2.4: Sequencing

In this contribution, our solution works on the “Nnwdaf_MLModelProvision” service by proposing necessary modifications to the service flow sequences as detailed in figure 5.7.7-1 titled "Procedure for UE Communication Analytics", in 3GPP TS 29.552 v19.0.0 (2024-09) [2].

Fig. 2. Proposed procedure for UE communication analytics with Nnwdaf_MLModelProvision service.

Fig. 3. Proposed procedure for UE communication analytics with Nnwdaf_MLModelProvision service.

In this contribution, our solution works on the “Nnwdaf_MLModelProvision” service by proposing necessary modifications to the service flow sequences as detailed in figure 5.7.7-1 titled "Procedure for Fig. 3 displays the proposed placement of Nnwdaf_MLModelProvision signaling flows within the “Procedure for UE Communication Analytics” signaling hierarchy (highlighted by an oval box). Specifically, we propose to incorporate “Nnwdaf_MLModelProvision_Subscribe request” and “Nnwdaf_MLModelProvision_Subscribe response” into the sequence numbers (1b) and (1c) of Fig. 3 respectively. We include “Nnwdaf_MLModelProvision_Notify request” with sequence numbers (13a) and (20a), and “Nnwdaf_MLModelProvision_Notify response” with sequence numbers (13b) and (20b), respectively. As per signaling flows of (1b) and (1c), to acquire the ML model information pertinent to UE communication analytics (provided by NWDAF), the NF service consumer may invoke “Nnwdaf_MLModelProvision_Subscribe request” and the NWDAF may respond with “Nnwdaf_MLModelProvision_Subscribe response”. Upon the execution of steps (1b) and (1c), the NWDAF may initiate “Nnwdaf_MLModelProvision_Notify request” and “Nnwdaf_MLModelProvision_Notify response” as given in (13b) and (13c). These services are invoked once again by NWDAF in (20b) and (20c). The proposed sequential placement of the signaling flow between NWDAF and NF realize the analytic services of NWDAF within the future 6G core.

Observation 2: NWDAF provides “Nnwdaf_MLModelProvision” services to realise ML model provision.

Proposal 4: Leveraging “Nnwdaf_MLModelProvision” services to get the user communication analytics and the possible placing of this particular services in between the NF consumer and NDWAF.

This possible placing of the user communication analytics is shown the figure 5.7.7-1 titled "Procedure for UE Communication Analytics", in 3GPP TS 29.552 v19.0.0 (2024-09).

Proposal 5: Availing the “Nnwdaf_MLModelProvision” services, the predictive ML algorithm LSTM (not limited to) can predict the most energy-efficient UPF as an anchoring UPF among the plurality of UPF.

Proposal 6: In order to obtain the UE communication analytics with predicted energy consumption data, the NF may invoke the “Nnwdaf_MLModelProvision_Subscribe request” and “Nnwdaf_MLModelProvision_Subscribe response” into the sequence numbers (1b) and (1c).

Proposal 7: If step 1(b) and 1(c) are performed, the NWDAF may invoke “Nnwdaf_MLModelProvision_Notify request” with sequence numbers (13b) and “Nnwdaf_MLModelProvision_Notify response” with sequence numbers (13c) respectively, 20(b) and 20(c) is same as step 13(b) and step 13(c).

2.2.5: Results and Discussions

The proposed EEM within NWDAF and its predictive capability using LSTM has been verified experimentally on a synthetically generated energy consumption dataset for an UPF (Tables I and II) to display its proof of concept. Table III shows the hyper-parameters of the LSTM model used to evaluate our system and mechanism.

Table III: LSTM Model hyper-parameters

| Sl. No. | Parameters | Values |
| --- | --- | --- |
| 1 | Learning rate | 0.00001 |
| 2 | Epochs | 125 |
| 3 | Steps | 70 |
| 4 | Batch size | 64 |
| 5 | Number of neurons per layer | 200 |
| 6 | Number of hidden layers | 2 |
| 7 | Optimizer | Adam |

Fig. 4: Epochs curve vs. train and validation loss.        Fig. 5: Train and test prediction using LSTM model.

With respect to the training, testing and validation results of the LSTM model, Fig. 4 depicts the training loss and validation loss trajectories according to the number of epochs. It is evident that an increase in the number of epochs correlates with a reduction in losses. The training mean absolute error (MAE) score is 1.24, and the test MAE score is also 1.24.

Fig. 5 depicts the prediction plot for both the training and testing periods, alongside the actual data of the LSTM model. The blue represents the real data, while green and red denote the training and testing phase data, respectively. The testing data indicates predictions that closely align with the actual energy values, suggesting that our model is functioning correctly.

2.3 Requirements for incorporating EEM

- The contribution is expected to realize an intelligent AI/ML-enabled 6G core towards reducing the carbon footprint with predictive energy consumption towards efficient UPF selection.

- Towards the United Nations SDG, the contribution aims to provide a green 6G core architecture and design

- The proposed contribution and its constituent solution is intended to be standardized to be incorporated into the next 3GPP Release and relevant TS documents.

2.4 Standardization

- Introduction of an energy consumption predictive analytic service through an energy efficiency module (EEM) incorporated with the existing NWDAF.

- Mechanism to implement the “Nnwdaf_MLModelProvision” service into the “Procedure for UE Communication analytics” through appropriate service flows by integrating the EEM service flow sequences as detailed in figure 5.7.7-1 titled "Procedure for UE Communication Analytics", in 3GPP TS 29.552 v19.0.0 (2024-09).

# Conclusion

The contribution is intended to incorporate an energy efficiency module (EEM) into the current NWDAF modularity, focusing on the energy-efficient user plane for 6G core networks. A predictive algorithm with LSTM has been proposed to estimate the future energy consumption of the UPFs registered with the network, based on the historical energy consumption for each of the UPFs. This historical data is gathered from the UE communication analytics, and the UPF predicted to consume the least energy in the next instant is selected to act as the anchoring UPF (among several UPFs) for a new application flow. The "Nnwdaf_MLModelProvision" services offered by the NWDAF facilitate the implementation of machine learning models into 6G core networks. This contribution additionally suggested the suitable positioning of the “Nnwdaf_MLModelProvision” services within figure 5.7.7-1, entitled "Procedure for UE Communication Analytics," in 3GPP TS 29.552 v19.0.0 (2024-09). This paper provides the following observations and proposals:

Observation 1: The enhancement of energy efficiency and energy saving in 5GS can be achieved by selecting an energy-efficient UPF as an anchoring UPF among the plurality of UPF in 5GC.

Proposal 1: An Energy Efficiency Module (EEM) within NWDAF is required to facilitate the

energy-efficiency in 5GC to select the anchoring UPF.

Proposal 2: Using a predictive learning algorithm, e.g. LSTM (not limited to), can select the best energy-efficient UPF based on historical energy consumption data.

Proposal 3: The architecture for the NWDAF-assisted energy-efficient 6G core User Plane is illustrated in Fig. 1.

Observation 2: NWDAF provides “Nnwdaf_MLModelProvision” services to realise ML model provision.

Proposal 4: Leveraging “Nnwdaf_MLModelProvision” services to get the user communication analytics and the possible placing of this particular services in between the NF consumer and NDWAF.

This possible placing of the user communication analytics is shown the figure 5.7.7-1 titled "Procedure for UE Communication Analytics", in 3GPP TS 29.552 v19.0.0 (2024-09).

Proposal 5: Availing the “Nnwdaf_MLModelProvision” services, the predictive ML algorithm LSTM (not limited to) can predict the most energy-efficient UPF as an anchoring UPF among the plurality of UPF.

Proposal 6: In order to obtain the UE communication analytics with predicted energy consumption data, the NF may invoke the “Nnwdaf_MLModelProvision_Subscribe request” and “Nnwdaf_MLModelProvision_Subscribe response” into the sequence numbers (1b) and (1c).

Proposal 7: If step 1(b) and 1(c) are performed, the NWDAF may invoke “Nnwdaf_MLModelProvision_Notify request” with sequence numbers (13b) and “Nnwdaf_MLModelProvision_Notify response” with sequence numbers (13c) respectively, 20(b) and 20(c) is same as step 13(b) and step 13(c).

# References

[1] “Architecture enhancements for 5G System (5GS) to support network data analytics services”, 3GPP TS 23.288, version 19.0.0, 2024.

[2] “5G System; Network Data Analytics signalling flows; Stage 3”, 3GPP 29.552, version 19.0.0, 2024.

[3] “5G System; Network Data Analytics Services; Stage 3”, 3GPP 29.520, version 19.0.0, 2024.

[4] “System architecture for the 5G System (5GS)”, 3GPP 23.501, version 16.6.0, 2020.

[5] “Study of enablers for Network Automation for 5G”, 3GPP TR 23.791version 16.2.0, 2019.

[6] A. Vishwanath, F. Jalali, R. Ayre, T. Alpcan, K. Hinton and R. S. Tucker, "Energy consumption of interactive cloud-based document processing applications," 2013 IEEE International Conference on Communications (ICC), Budapest, Hungary, 2013, pp. 4212-4216, doi: 10.1109/ICC.2013.6655224.

[7] A. Vishwanath, F. Jalali, K. Hinton, T. Alpcan, R. W. A. Ayre and R. S. Tucker, "Energy Consumption Comparison of Interactive Cloud-Based and Local Applications," in IEEE Journal on Selected Areas in Communications, vol. 33, no. 4, pp. 616-626, April 2015, doi: 10.1109/JSAC.2015.2393431.