3GPP TSG-WG SA2 Meeting #167	S2-2502035

Athens, GR, 17th Feb – 21st Feb, 2025	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Discussion on the update of S2-2501134

Document for:	Discussion

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract: It proposes to discuss the update of the endorsed paper S2-2501134.

1. Introduction

The paper on the triggering for VFL training (S2-2501134) has been endorsed. However, the following details need further discussion.

- Relationship on triggers between Training and inference

- Whether VFL Training termination Flag in the termination message is required is determined after settling down the service operation.

2. Discussion

- Relationship on triggers between Training and inference

In the general case, there are only two cases triggering the model training:

- Based on configuration or operator policy, the model is trained (in advance).

- As described in clause 6.2A.0 of TS 23.288, an NWDAF containing MTLF may determine that further training for an existing ML Model is needed when it receives the ML Model subscription or the ML Model request

Figure 6.2A.1-1: ML Model for analytics subscribe/unsubscribe

In the VFL case, a) and b) are still applied. However, it is agreed that in VFL process there will be no separate between the training entity and inference entity, so the NWDAF Service consumer (i.e. AnLF) cannot get the model directly. So the AnLF can only get the Analytic Output in this case, which is for inference.

Observation 1: The trigger for normal training also applies to VFL training.

Observation 2: The output of the above trigger b) to the AnLF is analytic output but not the model, i.e. it is in fact inference request.

Proposal 1: It is proposed to agree there no extra trigger for training and move step 0 in clause 6.2H.2.3.1 of S2-2501134 into inference procedure.

Another special thing in VFL is that the VFL Server could be AF. S2-2501134 proposes to use a new MTLF pretending as VFL Server. However, the MTLF is using Naf_VFLTraining Subscribe service to trigger the VFL training, which will not introduce new services compared to the AnLF when the NWDAF is VFL Server. VFLTraining service is only used by VFL participants now while the MTLF is not part of VFL process.

Observation 3: The role of MTLF when the AF is VFL Server needs more discussion.

Proposal 2: Only capture the option based on step 0a,0b in clause 6.2H.2.3.1 of S2-2501134 in the inference procedure. But for the details, leave them as maintenance.

In detail, the proposal is as following:

There are still only two triggers for VFL training, no matter whether AF or NWDAF is the VFL server.

Table 1 Trigger for VFL training

|  | NWDAF as VFL Server | AF as VFL Server |
| --- | --- | --- |
| Model training triggered via internal logic | No need for explicit call flow. | No need for explicit call flow. |
| Model training triggered by inference request | See inference (Table 2) | See inference (Table 2) |

And the inference request is always triggered when an AnLF receives Analytic request.

Table 2 Inference request

|  | NWDAF as VFL Server | AF as VFL Server |
| --- | --- | --- |
| AnLF itself is VFL Server | Supported already in step 2 of clause 6.2H.2.4.1 of TS 23.288. Need to add description that if not trained, also start training. | / |
| AnLF itself is not VFL Server | Find an MTLF support such Analytic ID. For this Analytic ID, MTLF is the VFL server. This is related to case 0c of the endorsed paper. Need to add description that if not trained, also start training. | Find an MTLF support such Analytic ID. For this Analytic ID, AF is the server. And the MTLF will pretend to support such Analytic ID. This is related to case 0a+0b of the endorsed paper. Need to add description that if not trained, also start training. |

- Whether VFL Training termination Flag in the termination message is required is determined after settling down the service operation.

It has already agreed to use Subscribe/notify also for VFL training, i.e. same as HFL. For the termination flag, it is used to clearly define the difference between an unsubscription due to error/failure and a unsubscription due to successful end of training. There is another proposal to use cause code of the subscription. However, it is preferred to keep similar format as HFL.

Proposal 3: Capture VFL Training termination Flag in step 8 of clause 6.2H.2.3.1.

3. Conclusion and proposal(s)

Proposal 1: It is proposed to agree there no extra trigger for training and move step 0 in clause 6.2H.2.3.1 of S2-2501134 into inference procedure.

Proposal 2: Only capture the option based on step 0a,0b in clause 6.2H.2.3.1 of S2-2501134 in the inference procedure. But for the details, leave them as maintenance.

Proposal 3: Capture VFL Training termination Flag in step 8 of clause 6.2H.2.3.1.

Please also see details in S2-2502036 to see the update of inference procedure.