# 3GPP TSG-WG SA2 Meeting #167																S2-2502064

# 17 – 21 February, 2025, Athens, Greece

Source:	OPPO

Title:	Issues on LMF relocation for multiple LCS-UPP connection maintanence

Agenda item:	9.6.2 (5G_eLCS_Ph3)

Document for:	Decision

1. Introduction

As proposed in S2-2500370, multiple LCS-UPP connections towards different LMFs should be allowed to avoid conflicts of principles in LMF selection and also LMF internal logic.

However, if there are more than one LCS-UPP connections for one UE, the UE should have to decide which message to be sent on which LCS-UPP connection. The traffic routing issues for multiple LCS-UPP connections are discussed as follows.

2. Discussion

As specified in 6.11.4 of TS 23.273, LPP messages could be transferred via user plane. As specified in TS 37.355, LPP messages including assistance information transfer, capability transfer and location information transfer can be used to fulfil a complete positioning procedure without other extra interaction. For LMF triggered positioining procedure in LCS-UPP connection, the UE can always use the same LCS-UPP session ID as the DL message sent by LMF. And the UE can bind the UL messages to the same LCS-UPP connection according to source address and LCS-UPP session ID of the DL message. But for UE triggered positioning procedure, the UE cannot decide which LCS-UPP connection should be used when there are multiple connections.

Observation #1: For LMF triggered positioning procedure, there is no routing selection issue. For UE triggered positioning procedure, the UE cannot select proper LCS-UPP connection.

In order to avoid routing selection issue, proposal #1 is:

Proposal #1: The 5GC-MO-LR should not be triggered via user plane.

When LMF relocation occurs, the old LCS-UPP connection is released. The UE side maintained mapping between LCS-UPP connection (e.g. LMF’s user plane IP address) and deferred routing identifier (e.g. LCS UPP Session ID) cannot be used any more. If there are multiple connections maintained at the UE side, the UE cannot decide which connection should be used for which LCS-UPP session.

Observation #2: In LMF relocation scenario, routing selection at the UE side exists if multiple LCS-UPP connections are maintained.

In order to assist the UE re-establish the binding relationship between the LCS-UPP Session and proper LCS-UPP connection, proposal #2 is:

Proposal #2: The target LMF should notify the UE with the defered routing identifier and related user plane information (e.g. LMF’s user plane IP address) to the UE after context transfer.

3. Proposal

Proposal #1: The 5GC-MO-LR should not be triggered via user plane.

Proposal #2: The target LMF should notify the UE with the defered routing identifier and related user plane information (e.g. LMF’s user plane IP address) to the UE after context transfer.

Proposals above are provided in S2-2502065 and S2-2502066.