SA WG2 Meeting #167	S2-2502261

# 17 - 21 February 2025, Athens, Greece

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1377 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on intermediate training result and intermediate model training information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-12-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clarify the contents of intermediate training result and intermediate model training information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the content inside gradient information and loss information, adding two notes saying that they are up to implementation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CT3 may be confused and can not conduct stage 3 work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

6.2H.2.3.1	Training Procedure for Vertical Federated Learning when NWDAF is acting as VFL server

The figure 6.2H.2.3.1-1 below shows the training procedure for Vertical Federated Learning when NWDAF is acting as VFL server.

Figure 6.2H.2.3.1-1: Training procedure for Vertical Federated Learning when NWDAF is acting as VFL server

Editor's note:	How the NEF assists the VFL training process as well as whether the service operations going via NEF is using the existing or new service operation are FFS.

Editor's note:	The details of the services in the procedure and whether VFL Training Start Flag is needed are FFS.

Editor's note:	It is FFS whether sample/feature information is required to be provided or updated in each training.

Editor's note:	Whether and how to include interoperability information in the VFL training procedure is FFS.

Editor's note:	Whether and how to define the trigger of VFL training is FFS.

Editor's note:	Whether and how to transfer the confirmation in VFL Preparation Phase at the beginning of VFL Training Phase is FFS.

1.	The NWDAF acting as VFL server determines the VFL clients that participate in VFL procedure in the VFL clients discovery and preparation phase as described in the clause 6.2H.2.1 and clause 6.2H.2.2.

NOTE 1:	VFL Server can determine to start the training based on local configuration and agreement among vendors and/or application providers participating in the same group for specific VFL task(s).

Steps 2-6 are repeated until the training termination condition is reached.

2.	To start the VFL training, the VFL server sends a request to start the training to all selected VFL clients The request includes VFL correlation ID, at least the parameters negotiated during the preparation phase, Optionally, the VFL Server includes: Analytic filter information, maximum response time (i.e. the maximum time between VFL clients receive intermediate model training information and send back intermediate training result).

Editor's note:	Whether the parameters negotiated in the preparation phase are provided at the end of the preparation phase or at the start of the training is FFS.

If the VFL procedure continues in subsequent iterations, the VFL server sends a request for a new VFL training iteration containing the VFL correlation ID and intermediate model training information to each of the VFL clients for next round of VFL training.

2a.	The VFL server sends a Nnwdaf_VFLTraining_Subscribe to the selected NWDAF VFL clients(s).

2b.	The VFL server sends a Naf_VFLTraining_Subscribe to the selected trusted AF VFL clients(s).

2c.	For each selected untrusted AF VFL clients, the VFL server sends a Nnef_VFLTraining_AFClient_Subscribe to the NEF handling that AF.

2d.	For each selected untrusted AF VFL clients, the NEF sends a Naf_VFLTraining_Subscribe to that AF. The NEF may also translate the analytic filter information if needed, e.g. TAIs into geographical area.

NOTE 2:	In this release, the same NF associated with a VFL Server or VFL Client capability during the VFL training for a VFL correlation ID is also the same NF during the VFL inference.

Editor's note:	Additional Parameters to be provided in the request are FFS.

Editor's note:	It is FFS whether and how the local ML model is obtained by VFL Client in VFL training process.

3.	[Optional] Each VFL client collects its local data by using the current mechanism if the VFL client has no local data already available. The data used by each VFL Client is collected as per alignment information.

4.	During VFL training procedure, each VFL client further trains the local ML model associated with the same VFL Correlation ID based on their own collected or available data and when applicable (e.g. after the first round of training) and possible intermediate model training information distributed by the VFL server in the previous training iteration. Each VFL Client computes and reports the client intermediate training result of the local ML model to the VFL server.

NOTE 3:	The intermediate model training information and intermediate training result are constructed in per sample granularity.

NOTE X: The contents of intermediate training result depend on the type of ML Model or algorithm used in VFL training and are up to implementation.

Editor's note:	It is FFS and may depend on the service design: When the clients report the client intermediate training result, it also includes the corresponding VFL correlation ID.

Editor's note:	Whether and how to exchange intermediate training result among VFL Clients is FFS.

5.	Each VFL client reports the computed client intermediate training result of the local ML model to the VFL server.

5a.	A NWDAF VFL client sends a Nnwdaf_VFLTraining_Notify.

5b.	A trusted AF VFL client sends a Naf_VFLTraining_Notify to the VFL server.

5c.	An untrusted AF VFL client sends a Naf_VFLTraining_Notify to the NEF.

5d.	For each untrusted AF VFL client , the NEF converts any external identifiers to internal identifiers and sends a Nnef_VFLTraining_AFClient_Notify to the VFL server.

6.	The VFL server may collect the local data and generate its own local intermediate training result. The NWDAF acting as VFL Server computes the intermediate model training information (e.g. gradient information or loss information that may contain loss function or loss value) based on the VFL Client(s) intermediate training result(s) received in step 4, its own local intermediate results and the label. The intermediate model training information is used for updating the models of VFL clients. Different intermediate model training information may be computed for different VFL clients and for the VFL Server itself.

NOTE X: The contents of loss information and gradient information depend on the type of ML Model or algorithm used in VFL training and are determined by implementation.

The VFL server may also compute the ML model metric (e.g. ML model accuracy) based on all the intermediate training result received from VFL clients and the label.

Editor's note:	Whether weight of the VFL Client is computed by VFL server is FFS.

Editor's note:	Whether VFL server and VFL clients share feature information is FFS.

7.	[Optional] The NWDAF acting as VFL server evaluates (e.g. based on the convergence of a loss function or loss value and/or if the pre-set iteration number is reached) whether VFL Training process converged. If the VFL Server evaluates the VFL Training process did not converge, the NWDAF acting as a VFL Server determines another round of VFL training is required and repeats step 2 - 6. If the VFL Server evaluates the VFL training process converged, it determines the VFL Training is completed. In this case, the VFL Server terminates the current VFL training process via step 7.

The VFL training termination decision may be also made as follows:

Based on the consumer request, the VFL server sends VFL status report to the consumer. The status report may include model metric (e.g. ML model accuracy).

Editor's note:	The content of the VFL status report is FFS.

Editor's note:	Whether VFL server sending convergence report to the VFL client and what is convergence report are FFS.

The consumer decides whether the current model can fulfil the requirement, e.g. ML model metric is satisfactory for the consumer and determines to stop or continue the training process. The consumer continues the training process or stops the training process.

Based on the subscription request sent from the consumer, the VFL server updates or terminates the current VFL training process.

Editor's note:	Whether the ML model metric (e.g. ML model accuracy) defined for HFL can be re-applied to VFL is FFS.

8.	The VFL server sends VFL training termination message to VFL Client if it decides to terminate the VFL training process, the termination message contains VFL Correlation ID.

8a.	The VFL server sends a Nnwdaf_VFLTraining_Unsubscribe t to the selected NWDAF VFL clients(s).

8b.	The VFL server sends a Naf_VFLTraining_Unsubscribe to the selected trusted AF VFL clients(s).

8c.	For each selected untrusted AF VFL clients, the VFL server sends a Nnef_VFLTraining_AFClient_Unubscribe to the NEF handling that AF.

8d.	For each selected untrusted AF VFL clients, the NEF sends a Naf_VFLTraining_Unsubscribe to that AF.

9.	The VFL Server, stores VFL correlation ID, the local trained ML Model, the mapping information of the VFL correlation ID to the following parameters: Analytics ID related to the VFL training process, locally trained Model. Additionally, the VFL server stores the VFL client information, which may be used to determine associated VFL client in the VFL inference.

Each VFL client stores VFL correlation ID, the locally trained ML Model, the mapping information of the VFL correlation ID to locally trained Model

NOTE 4:	The VFL correlation ID and the stored mapping information are used later for inference as described in Clause 6.2H.2.4.1.

Editor's note:	Whether VFL Training termination Flag in the termination message is required	is determined after settling down the service operation.

NOTE 5:	If untrusted AF is involved in VFL Clients, the message between NWDAF acting as VFL Server and the untrusted AF is via NEF.

* * * End of change* * *