3GPP TSG-SA2 Meeting #167	S2-2502394

Athens, Greece, February 17 – 21, 2025

Title:	[Draft] LS on NR Femto cell shared by PLMN and PNI-NPN

Response to:	-

Release:	Rel-19

Work Item:	5G_Femto, NR_WAB_5GFemto-Core

Source:	SA2

To:	RAN3

Cc:	-

Contact person:	Myungjune Youn

m.youn@lge.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

Overall description

SA2 discussed access control description agreed in RAN3, so captured in RAN3 BL CR to TS 38.300 (R3-247936).

- The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG. - The NR Femto may activate a cell shared by both PLMN and PNI-NPN, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell. - The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

Among above three options, second option requires further impact in stage-2 and stage-3 specifications because current specifications do not allow a UE to access a CAG cell when the UE does not support CAG e.g. as described below.

// From TS 24.501 ##### 5.5.1.2.8 Abnormal cases on the network side The following abnormal cases can be identified: …… j) Based on operator policy, if the initial registration request from a UE not supporting CAG is rejected due to CAG restrictions, the network shall reject the initial registration with a 5GMM cause value other than the 5GMM cause #76 (Not authorized for this CAG or authorized for CAG cells only). ……

SA2 did not identify use case regarding a cell shared by both PLMN and PNI-NPN. On the contrary, according to TS 23.501 clause 5.30.3.4, the UE would always be rejected by the AMF if it does not have valid CAG subscription. SA2 also did not identify any reason to change the principle for NR Femto use cases.

Therefore, SA2 would like to understand whether RAN3 has any specific reason to introduce the case of a cell shared by both PLMN and PNI-NPN, or RAN3 can accept aligning with SA2’s design as in TS 23.501 clause 5.30.3.4.

Actions:

To RAN3:

SA2 kindly asks RAN3 to provide answer for above question.

Date of Next TSG-SA2 Meetings:

SA WG2#168	07 – 11, April 2025	Goteborg, Sweden

SA WG2#169	19 – 23, May 2025	Fukuoka, Japan