3GPP SA WG2 Meeting #167	S2-2502435

# Athens, Greece, 17-21 February 2025								                 (was S2-2502194)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1531 | rev | 3 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving ENs on Avatar communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To resolve following Editor’s Notes capatured in clause AC.11.3.2.1: Editor's note: Whether MF download the avatar representation directly from BAR or via DC AS is FFS. There are misalignments between Figure AC.11.3.2.1-1 and related description. There are misalignments between Figure AC.11.3.2.2-1 and related description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified the communication between MF and BAR for Avatar representation downloading. Modify the description of procedures in clause AC.11.3.2.1 and AC.11.3.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incompletion of IMS Avatar communication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.11.3.2, AC.11.3.2.1, AC.11.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES <<<<

### AC.11.3.2	UE centric procedure

The UE centric mode includes two scenarios:

-	Sending UE centric: When the UE initiates an avatar communication with the peer UE, the UE that owns the avatar representation should download the avatar representation from BAR, if the Avatar representation is not locally available. The sending UE performs avatar animation and sends the animated video stream to the peer UE.

-	Receiving UE centric: When the UE initiates an avatar communication with the peer UE, the UE that owns the avatar representation should sends the avatar ID to the peer UE. The peer UE downloads the avatar representation from BAR based on the received avatar ID, performs the avatar animation, and displays the animated video stream.

NOTE 1:	It is assumed that both UEs support IMS data channel and at least one UE support avatar animation capability among the two UEs in an avatar communication session.

NOTE 2:	UEs could transmit additional data using application data channel and/or audio/video to each other to support the UE centric rendering. The usage of those data is out of scope of this specification.

# >>>>NEXT CHANGE<<<<t

#### AC.11.3.2.1	Sending UE centric procedure

Figure AC.11.3.2.1-1: Procedures of Sending UE centric IMS Avatar communication

Figure AC.11.3.2.1-1 depicts a typical call flow procedure of sending UE centric IMS Avatar communication. The main steps in the call flow are as follows:

1.	IMS audio/video session and bootstrap data channel have been established. UE1 downloads avatar communication application. The avatar ID list is downloaded as described in clause AC.11.1. Then UE1 chooses an avatar ID and runs the avatar communication application. If the avatar representation is locally available in UE1, steps 2-6 are skipped.

2.	P2A application data channel(s) are established for avatar representation transmission. The rendering mode is set to 'UE centric' by the DC AS. The DCSF instructs the MF to establish MDC2 connection either to the BAR or the DC AS.

NOTE:	In this step, the UE fetches the associated URL associated with the requested Avatar ID.

3.	UE1 requests downloading Avatar representation from BAR via MF using the associated URL, and UE1's identity (i.e., MSISDN) through the established application data channel. The MF, based on the instruction received in step 2 fetches the Avatar represention from the BAR or  DC AS using the resource URL for that purpose.

4-5.	The BAR authorizes the request of downloading Avatar representation and responses with avatar representation data to the MF.

NOTE:	The MF downloads UE1's avatar representation from BAR directly or indirectly via DC AS based on the MDC2 connection established in step 2.

6.	The MF sends the avatar representation data to UE1.

7.	UE1 performs avatar animation with the selected Avatar representation.

8.	UE1 transmits the animated video stream over RTP to UE2.

# >>>>NEXT CHANGE<<<<

#### AC.11.3.2.2	Receiving UE centric procedure

Figure AC.11.3.2.2-1: Procedures of Receiving UE centric IMS Avatar communication

Figure AC.11.3.2.2-1 depicts a typical call flow procedure of receiving UE centric IMS Avatar communication. The main steps in the call flow are as follows:

1.	IMS session and bootstrap data channel have been established. UE1 downloads avatar communication application. The avatar ID list is downloaded as described in clause AC.11.1. Then UE1 chooses an avatar ID and runs the avatar communication application.

2.	P2A2P application data channel(s) are established between UE1/UE2 and DC AS. The rendering mode is set to 'UE centric' by the DC AS. The DCSF instructs the MF to establish MDC2 connection either to the BAR or the DC AS.

NOTE:	In this step, the UE fetches the associated URL associated with the requested Avatar ID.

3.	UE1 decides to request peer UE to perform avatar animation based on its status such as power, signal, computing power, internal storage, etc.

4.	UE1 performs avatar animation negotiation with the DC AS and UE2. The negotiation includes usage of the UE1's avatar ID and the animation data types (e.g. text and/or facial expression) supported by UE1.

Editor's note:	The negotiation procedure needs to be further defined in SA WG4, and cooperation with SA WG4 is needed.

5.	UE2 requests downloading avatar representation from BAR via MF using the associated URL received from UE1 through the established application data channel in step 2. The MF, based on the instruction received in step 2 fetches the Avatar represention from the BAR or  DC AS using the Resource URL for that purpose.

6-7.	The BAR authenticates and authorizes the request of downloading avatar representation using UE1's avatar ID and responses with UE1's avatar representation data. The DC AS should authorize the UE2 download request based on the negotiation result from step 4.

NOTE:		The MF downloads UE1's avatar representation from BAR directly or indirectly via DC AS based on MDC2 connection established in step 2.

8.	The MF sends UE1's Avatar representation data to UE2.

9.	Based on the avatar animation negotiation result in step 4, UE1 may send metadata (e.g. facial expression) to UE2 through the established application data channel or audio/video through RTP channel between UE1 and UE2.

10.	UE2 performs avatar animation with the UE1's avatar representation based on the received metadata or audio/video from UE1.

# >>>>END OF CHANGES<<<<