3GPP TSG-SA WG2 Meeting #167	S2-2501457

Athens, Greece, 17th Feb 2025 – 21st Feb 2025	(revision of S2-2500227)

Agenda item:	19.4.2

Source:	Nokia, Deutsche Telekom

Title:	Discussion on Energy Saving Indicator

WI/Release:	EnergySys/Rel-19

Document for:	Discussion and Decision

# 1	Discussion

At SA2#166 in 23501_CR5739r11 in S2-2413019 it was agreed to introduce in the 5GS an Energy Saving Indicator that is stored in the UDM as AM subscription data and can be provided to the NG-RAN and to the PCF (to be confirmed).

In this discussion paper we propose that this shall be specified as a scalar with values that can be associated to operator-defined behaviors in the NG-RAN and/or end to end.

One example could be helpful:

For instance, in a PLMN there could be several types of energy saving policies which the Energy Saving indicator could trigger and different subscriptions may be associated to different behaviors, for instance:

- The energy saving behavior affect only the way the NG-RAN handles the UE from a resource scheduling standpoint so that energy saving is achieved.

- The energy saving behavior affects the UE-AMBR policies at AM-PCF

- The energy saving behavior affects the RFSP value at AM-PCF

- A combination of 1 and 2

- A combination of 1 and 3

- A combination of all of the above in 1-3

If we just defined a single value, UEs could only be associated to a single behavior, and this may not be desirable.

Also, allowing for multiple values is future proof, if e.g. the

Nokia would recommend that at least 8 values are allowed, but 16 or 256 could be easily supported and enable easy encoding from a stage three standpoint, hence 16 values seem a reasonable figure to go for.

# 2	Conclusions

It is proposed that the Proposal here below are agreed.

Proposal#1: The Energy saving indicator is a scalar that could have e.g. up to 16 values (as proposed in the CR in S2-2501458).

Proposal#2: The AMF sends this value to NG-RAN and AM-PCF.

The supporting companies can be flexible on whether SA2 specifications will identify the maximum number of values, or this can be left to stage 3. The benefit of specifying this in stage 2 is that it provides guidance to stage 3 groups and avoid the usual flow of LSs needed to coordinate the value across CT and RAN WGs.