3GPP TSG-WG SA2 Meeting # 167	S2-2501597

# Athens, Greece, 17 - 21 February 2025									(revision of S2-xxxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6087 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PWS handling with Additional ULI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2, VMR |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | PWS handling with Additional ULI is missing in this spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the usage of Additional ULI for PWS in clause 5.35A.6 and cluase 5.49.4 with reference to TS 23.502. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing linkage between PWS handling and Additional ULI in this spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35A.6, 5.49.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.35A.6	Providing cell ID/TAC of MBSR for services

The TAC and cell ID broadcasted by the MBSR cell(s) are configured and reconfigured e.g. upon mobility as specified in TS 38.401 [42].

After the MBSR is authorized as defined in 5.35A.4, when a UE is served by a cell of this MBSR, the IAB-donor-CU may provide 'Additional ULI' in addition to User Location Information, in N2 messages. The 'Additional ULI' is the ULI of the IAB-UE. The AMF may consider the 'Additional ULI' when it determines UE location and manages the UE location related functions (e.g. Mobility Restrictions).

When the AMF provides user location information to other NFs (e.g. LMF as specified in clause 5.9 of TS 23.273 [87]) for a UE connected via MBSR, the AMF may also send the Additional ULI received via N2 messages.

The AMF and IAB-donor-CU use Additional ULI to support the PWS function as described in clause 5.16.1 of TS 23.502 [3].

# * * * * Next change * * * *

### 5.49.4	Support of Additional ULI

The TAC and cell ID broadcasted by the MWAB-gNB are configured and reconfigured e.g. upon MWAB mobility as specified in TS 38.401 [42]. The MWAB-gNB provides these in the User Location Information (ULI) to the AMF serving the UE accessing the MWAB.

The MWAB-gNB shall provide the Additional User Location Information (Additional ULI) that reflects the location of the MWAB-UE to the AMF when it sends the User Location Information (ULI) over N2 messages.

Editor's note:	For the case that the BH PLMN and the MWAB broadcasted PLMN are different the exact content and handling of the Additional ULI will be defined based on RAN feedback.

If serving cell change report is required by AMF as specified in clause 4.10 of TS 23.502 [3], the MWAB-gNB shall report Additional ULI (and ULI) for the UE even when only serving cell of the MWAB-UE changes.

The AMF may consider the 'Additional ULI' when it determines UE location and manages the UE location related functions (e.g. Mobility Restrictions).

Based on operator policy, the AMF may send the Additional ULI when the AMF provides user location information it has received over the N2 interface to a NF (e.g. the LMF as specified in clause 5.19.1 of TS 23.273 [87]).

The AMF and MWAB-gNB use Additional ULI to support the PWS function as described in clause 5.16.1 of TS 23.502 [3].

# * * * * End of changes * * * *