TSG-SA WG2 Meeting #167	S2- 2502773

17 – 21 February 2025, Athens, Greece                        (revision of S2- 2502032 , S2-2500787)

# Title:	Revised WID on Energy Efficiency and Energy Saving

# Source:	Samsung, Lenovo (Rapporteurs)

# Document for:	Approval

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Energy Efficiency and Energy Saving

Acronym:	EnergySys

Unique identifier:	1050112

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X | X |  |
| No | X | X |  |  |  |
| Don’t know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| x | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| EnergyServ | SA1 | 1000033 | Energy Efficiency as service criteria |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 940037 | Enhancements of EE for 5G Phase 2 | The Rel-18 WI in SA5 may be potentially related to the proposed SA2 study. |
| 870022 | Enhancements on EE for 5G networks | The Rel-17 WI in SA5 may be potentially related to the proposed SA2 study. |
| 810023 | Energy efficiency of 5G | The Rel-16 WI in SA5 may be potentially related to the proposed SA2 study. |
| 981137 | Network energy savings for NR | The Rel-18 work in RAN1 and RAN2 may be potentially related to the proposed SA2 study, depending on the agreed Rel-19 scope. |

# 3	Justification

Energy consumption is a significant source of operations costs for Mobile Network Operators (MNOs) and depending on the energy generation mix that is used to power networks, it can also have impact on the environment. There has been increasing work in 3GPP on improving energy efficiency and energy saving. Some solutions studied how to optimize energy consumption by adapting the network itself, e.g. activating and deactivating parts of the network. Such change to the network topology and components of the network could be performed transparent to the network architecture in some cases, or it may have implications in other cases, e.g. reselection of the appropriate network functions.

Stage 1 requirements for energy as service criteria have been identified in Rel-19 EnergyServ by SA1. While the goal of energy efficiency is to reduce energy for the same services, the goal of energy usage control as service criteria will be to supervise services in an energy-aware manner, ensuring the services are offered as intended by service providers, network operators or subscribers, with energy constraints and no performance consequences. Several functional requirements have been identified that promise increased control over energy usage to achieve service objectives for mobile network operators, service providers and their customers in the SA1 Rel-19 EnergyServ. Examples of these functional requirements include the ability to control energy usage based on the operator policies such as ‘energy credit limits’ and ‘maximum energy usage rate’ applying to services provided for a UE or group of UEs.

Also, 3GPP has already undertaken work in different working groups to provide recommendations on energy saving and introduce actual OAM or Radio Network enhancements to save energy. SA plenary has issued also a 3GPP-wide recommendation on considering Energy efficiency as an important design criterion for the technical solutions 3GPP defines in their specifications (see SP-211621). SA2 will also be required to investigates options for improved system behaviour targeting energy saving and energy efficiency.

This work item is to complete the design of stage 2 solutions and document the normative specifications, based on the conclusions of the FS_EnergySys study.

# 4	Objective

This work aims to specify required enhancements on 5GS to improve energy efficiency and to support energy saving in the network, for the following aspects, based on the conclusions in the TR 23.700-66:

- WT #1. The objective of this WT is to specify the basic elements for supporting the collection and calculation of energy saving and efficiency and the exposure of network energy related information. The following enhancements will be specified:

1)	A new network functionality is defined to collect and calculate the energy related information and exposes to the authorized consumer subject to operator’s policy:

-	If the authorized consumer is AF, the granularities include: per UE, per UE per application, per PDU session;

-	If the authorized consumer is 5GC NFs, the granularities include: per application, per UE, per-UE-per-QoS flow, per PDU session;

NOTE 1:	The new network functionality can be deployed standalone or co-located with other NFs under SA2 responsibility.

-	The energy related information for the granularities above include:

-	Energy consumption information;

-	Renewable energy information.

2)	The consumer NFs including 5GC NFs and /or AF, may request the energy consumption information exposure with reporting request e.g. Periodic reporting or Threshold based reporting.

3)	The new functionality supporting the calculation of the Energy Consumption information includes the following aspects:

a)	OAM provides the NF/Node-level energy consumption information at the gNB(s) and UPF(s) serving the UE to the new functionality.

b)	OAM: provides the overall data volume of the gNB to the new functionality;

c) The NF/Node-level information of a) and b) received from OAM, could be used by the new functionality for all the UEs serving by the NF/Node.

NOTE 2:	There is coordination between SA2 and SA5 to help OAM provides NF/Node-level EC information in normative phase.

d)	UPF: provides the overall data volume of the UPF;

e)	UPF: provides the data volume for the QoS flow or the SDF.

f) When the gNB and/or the (I-)UPF(s) which serving the UE are changed, the serving gNB ID and UPF ID will be sent to the new functionality through AMF/SMF.

NOTE 3: The measurement time period T needs to be aligned over the UPF/NG-RAN nodes for which information is collected, i.e. the start/stop of the time periods is time-synchronized.

4)	The new functionality determines the E2E energy consumption based on energy consumption per the granularities above at serving NF (i.e. NG-RAN and UPF).

5)	In the current release of the specification, only the energy-related information of user plane communication (not control plane signalling) is supported.

NOTE 4:	The exposure and collection of renewable energy information depend on the coordination with SA WG5.

NOTE 5:	In this Release, the gNB neither supports per-UE-per-PDU session nor per-UE-per-QoS flow level energy consumption reporting.

NOTE 6:	This work will coordinate with SA WG5 for OAM and charging part.

- WT #2. The objective of this WT is to specify the enhancements for subscription to enable network energy savings as service criteria based in WT#1. The following enhancements will be specified:

- The definition of energy saving subscription information per UE that is stored as part of the subscription data in the UDM/UDR, to assist the network to perform energy saving strategies for the UE

- The detailed procedures for the PCF to receive UE subscription data related to the energy related information to make policy decisions (reusing the existing parameters)

NOTE 8:	Further details and what energy related information can be taken into account for policy control will be defined during the normative work.

NOTE 9:	Detailed parameters and associated policy procedures will be discussed during the normative work.

# 5	Expected Output and Time scale

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | System architecture for support of Energy Efficiency and Energy Saving | TSG SA #106 (Dec., 2024) |  |
| 23.502 | Procedures for support of Energy Efficiency and Energy Saving | TSG SA #106 (Dec., 2024) |  |
| 23.503 | Policy control for support of Energy Efficiency and Energy Saving | TSG SA #106 (Dec., 2024) |  |

# 6	Work item Rapporteur(s)

Primary Rapporteur: Jungshin Park (Samsung), shin02.park@samsung.com

Secondary Rapporteur : Konstantinos Samdanis (Lenovo), ksamdanis@lenovo.com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Potential RAN issues are covered by RAN WGs (e. g. RAN2, RAN3)

Potential charging and OAM issues are covered by SA5.

Potential security issues are to be covered by SA3.

# 9	Supporting Individual Members

Supporting IM name

AT&T

CATT

China Mobile

Deutsche Telekom

ETRI

Hisilicon

Huawei

Intel

InterDigital

KDDI

KPN

Lenovo

LG Electronics

LG Uplus

MediaTek

NEC

NTT DOCOMO

OPPO

ORANGE

Rakuten Mobile

Samsung

SK Telecom

Telefonica

Toyota

Vivo

ZTE