3GPP TSG-SA WG2 Meeting #168	S2-2503122

# Gothenburg, Sweden, 07 – 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6182 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Energy Consumption Model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-03-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current energy consumption model/formulation, documented in Annex T of TS 23.501, does not distinguish the energy consumption used for serving the traffic and the idle mode energy consumption that leads to the robustness issue mentioned in S2-2501605. To address the issue, as discussed and illustrated in S2-2503121, it is proposed to exclude (deduct) the (estimation of) static/idle power consumption of the node (which is provided by OAM) from the total energy consumption of the node (which is also measured by OAM). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposing alternative model for calculation of energy consumption. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inaccurate estimation of energy consumption |  |  |  |  |  |  |  |  |
| Clauses affected: |  | T.3 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

Annex T (informative):
Network Energy consumption information calculation

# T.3	Calculation of Energy Consumption considering idle mode

To reduce the influence of idle mode energy consumption of the node, which is not consumed to serve UE traffic, on the information provided by EIF on the finer granularities to the consumer, as an alternative approach, EIF may calculate the energy consumption for required granularities as explained in T.2 where the node-level energy consumption EgNB and EUPF are replaced by E'gNB and E'UPF which are computed as follows:

- E'gNB=EgNB-EgNB,0

- E'UPF=EUPF-EUPF,0

Where EgNB and EUPF are the current node-level energy consumption values collected from OAM and the EgNB,0 and EUPF,0 are the node-level energy consumption in the idle mode.

NOTE X:	To consider dependency of the idle mode energy consumption of the node on the scaling level of the node, EIF may periodically requests EgNB,0 and EUPF,0 from the OAM.

********** End of Changes **********