3GPP TSG SA WG2 #168	            S2-2503167

07 - 11 April, 2025, Goteborg, Sweden 	(revision of S2-2502136)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1598 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#3: Updates on the ADC Interworking via DC AS procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The step 6 says “The DCSF changes P2P application DC to P2A application DC”. However, it is not clarified what operation should be done at DCSF and IMS AS to change the ADC session type from P2P to P2A. Also, it is not clear what if the IMS AS cannot change SDP attributes different with the UE requested (e.g. UE sent P2P SDP offer, but IMS AS responses P2A SDP answer). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The DCSF indicates to IMS AS that the P2P ADC needs to be changed to P2A ADC when the IMS DC interworking action is required. - The IMS AS changes the P2P ADC offer from UE to P2A ADC offer by modifying the endpoint type from ‘UE’ to “Server’. - The IMS AS provides P2A ADC accept answer to the UE. - When the IMS AS does not support change of ADC session type, then the IMS AS can reject the P2P ADC request. - In step 10, the box extended to cover UE#1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

AC.7.9.3	Signalling Procedure of Application Data Channel Interworking via DC AS

Figure AC.7.9.3-1 depicts a signalling flow diagram for establishing an application data channel from DCMTSI UE to DC application server, providing interworking of application data channel to MTSI UE via DC application server.

The originating DCMTSI UE sends the data channel application media to the DC AS, which performs the necessary interworking actions on the application media for the terminating MTSI UE.

Figure AC.7.9.3-1: Application Data Channel set up signalling procedure for IMS data channel interworking with MTSI UE via DC application server

The steps in the call flow are as follows:

1.	DC AS subscribes via the NEF to be notified when any application DC for a DC application that requires interworking is established.

1a.	The DC AS subscribes to the NEF by using Nnef_ImsEE_Subscribe Request (event ID: DC Interworking Required, DC Interworking Information). The DC Interworking Information includes the application ID(s) of the DC application which requires interworking with MTSI UE.

1b.	The NEF responses to DC AS by using Nnef_ImsEE_Subscribe Response.

1c.	The NEF locates the IMS AS(s) via NRF.

1d.	The NEF subscribes to the IMS AS as requested by DC AS in step 1a, by using Nimsas_ImsEE_Subscribe Request.

1e.	The IMS AS responses to NEF by using Nimsas_ImsEE_Subscribe Reponse.

2.	IMS session and bootstrap data channel have been established. Selected DC application(s) have been downloaded to UE#1.

The DCSF knows the UE#2 or terminating network does not support IMS DC based on procedure in clause AC.7.9.1. Hence, the bootstrap data channel is established only between the UE#1 and the originating network.

3-5.	The UE#1 initiates a P2P application DC. Steps 1-2 of clause AC.7.2.1 applies.

6.	After receiving the session event notification from the IMS AS, the DCSF determines that IMS DC interworking for the requested DC application is to be performed based on the application information, i.e. application id. the remote network or UE#2 not supporting IMS DC, or not willing to use IMS DC. The DCSF changes P2P application DC to P2A application DC.

6a.	The DCSF informs to the IMS AS that IMS DC interworking action is required for the requested DC application and the UE#2 side Information. The UE#2 side Information may include the SIP URI of the UE#2 or Called ID. The DCSF indicates that the P2P application DC needs to be changed to P2A application DC.

The IMS AS modifies the endpoint type of the application data channel from ‘UE’ to ‘Server’ for the application data channel to change the P2P application DC request to a P2A application DC request.

NOTE: If the IMS AS cannot change SDP attributes different with the UE requested, the IMS AS can reject the P2P application data channel establishement request and the UE#1 may initiate P2A application data channel establishment. The detail will be described in Stage 3.

6b.	The IMS AS notifies to the NEF that the DC Interworking Required event is detected with the UE#2 side Information by using Nimsas_ImsEE_Notify Request.

6c.	The NEF responses to the IMS AS by using Nimsas_ImsEE_Notify Response.

6d.	The NEF notifes to the DC AS that the DC Interworking Required event is detected with the UE#2 side Information by using Nnef_ImsEE_Notify Request.

6e.	The DC AS responses to the NEF by using Nnef_ImsEE_Notify Response.

7.	The DCSF instructs the MF via the IMS AS to establish the media resource of the P2A application DC and the DC connection is established between UE and the DC AS. During P2A connection establishment, the DC AS takes into account the information received in step 6d. Steps 5-14 of clause AC.7.2.2 applies.

8-9.	IMS AS sends the re-INVITE to remote network side and UE#2, which does not include bootstrap data channel information and application data channel media components in the SDP.

10.	Steps 17-26 of clause AC.7.2.2 applies. The SDP answer for application DC sent to the UE#1 indicates that P2A application data channel establishment is accepted.

11.	UE#1 uses the established application data channel to provide DC application specific information e.g. sends the file (e.g. a photo) to the DC AS, which will be sent to UE#2 from DC AS.

12.	DC AS performs interworking actions based on the notification received in step 6d and the data received in step 11.

NOTE:	How the DC AS performs interworking actions to provide DC application specific information to the MTSI UE is out of scope of this specification. The DC AS can for example generate a hyperlink associated with the file received in step 11 and sends the hyperlink to UE#2 through a SMS, where the user of UE#2 can download the file using the hyperlink if he/she wants.

# * * * * End of changes * * * *