3GPP TSG-WG SA2 Meeting #168	S2-2503279

# 07 - 11 April, 2025, Goteborg, Sweden

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0551 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction on Multi-hop UE-to-UE Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There exits editorial issues on Multi-hop UE-to-UE Relay |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 4.2.8A, change '5G ProSe UE-to-UE Relay' to '5G ProSe Multi-hop UE-to-UE Relay'. - Remove 'IP type' from the title of 6.7.5.2 - Rename the title of 6.7.5.2.2, correct the referenced clause |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inaccurete description on Multi-hop UE-to-UE Relay |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.8A, 6.7.5.2, 6.7.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.2.8A	5G ProSe Multi-hop UE-to-UE Relay reference architecture

Figure 4.2.8A-1 shows the Layer-3 5G ProSe UE-to-UE Relay reference architecture. The 5G ProSe End UEs communicate with each other via a set of interconnected 5G ProSe Multi-hop UE-to-UE Relays.

Figure 4.2.8A-1: Reference architecture for Layer-3 5G ProSe Multi-hop UE-to-UE Relay

Based on capability and configuration the 5G ProSe UE can act as a 5G ProSe End UE, a 5G ProSe Multi-hop UE-to-UE Relay or both. Each 5G ProSe End UE and the 5G ProSe Multi-hop UE-to-UE Relay may have subscriptions from the same PLMN or different PLMNs.

-	For IP PDU type, the 5G ProSe Multi-hop UE-to-UE Relays form a MANET network as defined in IETF RFC 7181 [35]. A 5G ProSe UE acting as 5G ProSe Multi-hop UE-to-UE Relay shall support the protocols defined in IETF RFC 7181 [35] and IETF RFC 6130 [36] on the PC5 interfaces towards other 5G ProSe Multi-hop UE-to-UE Relays. In addition, the 5G ProSe Multi-hop UE-to-UE Relay shall support a dedicated MANET Discovery Info message on the PC5 interfaces towards other 5G ProSe Multi-hop UE-to-UE Relays for propagation of DNS information, as described in clause 5.14.3.2.

NOTE:	The MANET Discovery Info message will be specified in Stage 3.

# * * * * Second change * * * *

6.7.5.2	Communication via 5G ProSe Layer-3 Multi-hop UE-to-UE-Relays

6.7.5.2.2	Layer-2 link establishment via 5G ProSe Layer-3 Multi-hop UE-to-UE Relays (Non-IP Type)

Figure 6.7.5.2.2-1: Layer-2 link establishment via 5G ProSe Layer-3 UE-to-UE Relay for Non-IP Type PDU

1.	5G ProSe Layer-3 End UE1 and 5G ProSe Layer-3 End UE2 are authorized for multi-hop UE-to-UE Relay service as End UE and are provisioned with parameters for discovery and connection setup with other UEs via multi-hop UE-to-UE Relay services.

5G ProSe Layer-3 UE-to-UE Relays are authorized for multi-hop UE-to-UE Relay service as Relay UE and are provisioned with parameters for discovery and connection setup with other UEs and relay UEs via multi-hop UE-to-UE Relay services (as described in clause 5.1.5.1). The provisioned parameter may include parameters such as RSC (Relay service Code)(s), list of PLMN, user info of UE for application which are allowed at multi-hop relay connection.

2.	It is assumed that the source 5G ProSe Layer-3 End UE1 selects a relay path to the Target End UE2 from the discovery procedure which is captured in clause 6.3.2.6.3.

3.	The source 5G ProSe Layer-3 End UE1 initiates the unicast Layer-2 link establishment procedure with the 5G ProSe Layer-3 UE-to-UE Relays by sending a Direct Communication Request message to the UE-to-UE Relay. The parameters included in the Direct Communication Request message are described in clause 6.4.3.10.2.

If there is already a PC5 link with the same RSC been established between the End UE and the UE-to-UE Relay or between UE-to-UE Relays, a Link Modification Request message is sent instead of Direct Communication Request message. The parameters included in the LMR message are described in clause 6.4.3.10.2.

4.	(Optional) When the security protection is enabled, the source 5G ProSe Layer-3 End UE sends the parameters as described in clause 6.4.3.10.2 to the 5G ProSe Layer-3 UE-to-UE Relay.

If 5G ProSe Layer-3 UE-to-UE relay detects the Ethernet MAC address of source 5G ProSe Layer-3 End UE is already used by another 5G ProSe Layer-3 End UE, then the 5G ProSe Layer-3 UE-to-UE Relay rejects the direct link establishment indicating that the MAC address is not unique.

5.	A UE-to-UE Relay sends a Direct Communication Request or Link Modification Request message to the next UE-to-UE Relay according to the path information in the received Direct Communication Request or Link Modification Request message.

The Source Layer-2 ID of the Direct Communication Request message is self-assigned by the UE-to-UE Relay and the Destination Layer-2 ID is the unicast Layer-2 ID associated with the User Info ID of the next hop UE-to-UE Relay. This association may be determined by UE-to-UE relay during discovery procedure in step 2.

6.	(Optional) When the security protection is enabled, the UE-to-UE Relays may establish security association as described in clause 6.4.3.10.2.

7.	A UE-to-UE Relay may know it is the last Relay in the path (e.g. according to the User Info ID of Relays in the received Direct Communication Request/Link Modification Request message). The UE-to-UE Relay sends a Direct Communication Request/Link Modification Request message to the target 5G ProSe Layer-3 End UE.

The Destination Layer-2 ID is the unicast Layer-2 ID of target End UE or a broadcast Layer-2 ID. Unicast Layer-2 ID is used if the Layer-2 ID of the Target End UE is known to the UE-to-UE Relay.

8.	(Optional) When the security protection is enabled, the UE-to-UE Relay (here, U2U Relay 2) may establish security association with the target 5G ProSe Layer-3 End UE as described in clause 6.4.3.10.2.

9.	The target End UE sends a Direct Communication Accept message to the UE-to-UE Relay it has successfully established security with. The DCA message may include the path information.

10-11.	After receiving a DCA message, the UE-to-UE Relay (e.g. Relay2) sends a DCA message to the next UE-to-UE Relay (e.g. Relay1) or Source End UE it has successfully established security with. The UE-to-UE Relay may decide the next UE-to-UE Relay (e.g. Relay1) according to the received DCA message or from which it previously received a corresponding DCR message.

NOTE 1:	In step 6, U2U Relay2 may detect the Ethernet MAC address of source 5G ProSe Layer-3 End UE is already used by another 5G ProSe Layer-3 End UE.

NOTE 2:	In step 9 and step 10, U2U Relay1 and U2U Relay2 may detect the Ethernet MAC address of target 5G ProSe Layer-3 End UE is already used by another 5G ProSe Layer-3 End UE.

For Ethernet communication, the 5G ProSe Layer-3 UE-to-UE Relay maintains the association between PC5 links and Ethernet MAC addresses received from the 5G ProSe Layer-3 End UE.

For Unstructured traffic communication, for each pair of source and target 5G ProSe Layer-3 End UEs, the 5G ProSe Layer-3 UE-to-UE Relay maintains the 1:1 mapping between two PC5 links to the (next hop to) source 5G ProSe Layer-3 End UE and the (next hop to) target 5G ProSe Layer-3 End UE.

# * * * * End of change * * * *