SA WG2 Meeting #168	S2-2503420

April 7 - 11, 2025, Goteborg, SE

Source:	China Mobile

Title:	Architectural assumptions and Architectural Requirements

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_ISAC/Rel-20

Abstract of the contribution: This paper proposes the architectural assumptions and architectural requirements for FS_ ISAC TR 23.700-14.

1	Discussion

This paper proposes the architectural assumptions and architectural requirements based on content of the FS_ISAC SID (SP-250401).

2	Proposal

It is proposed to include the following changes in TR 23.700-14.

* * * * Start of Changes (All new)* * * *

4.1	Architectural Assumptions

The architecture in this study should be based on the following assumptions:

-	The following two connectivity topologies between sensing RAN and 5GC are to be studied:

-	Topology 1: Support the transfer of sensing data and control plane message between RAN and 5GC directly.

NOTE 1: whether the NF(s) supporting the transfer of sensing data and control plane message is split or not need further be discussed.

-	Topology 2: Support the transfer of control plane message between RAN and 5GC reusing the existing N2 interface; Support the transfer of sensing data between RAN and 5GC referring to N3 interface.

- The architecture enhancement for support Non-3GPP sensing is excluded in Rel-20.

- Roaming scenario is not supported for ISAC service in Rel-20.

NOTE 2: Privacy protection and other security aspects will be coordinated with SA3.

NOTE 3 Charging and OAM aspects will be studied by SA5.

NOTE 4: Implications to RAN or RAN dependent aspects will be coordinated with RAN WGs.

NOTE 5: Support gNB based sensing in high priority. In addition, support UE based sensing if TU is enough in Rel-20.

4.2	Architectural Requirements

The following architectural requirements are applicable to this study:

- The architecture enhancement of support for ISAC Services needs to take the compatibility of multiple sensing modes into account.

- Support of the security aspects needs to consider the nature of the ISAC service (e.g., user’s privacy, area correlation).

* * * * End of Changes * * * *