TSG SA WG2 Meeting #168	S2-2503542

07 - 11 April, 2025, Goteborg, SE

Source:	Lenovo

Title:	New KI: service authorization and revocation for sensing services

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing-ARC / Rel-20

Abstract of the contribution: Proposes KI description for service authorization and revocation

1	Background

According to the SP-250401[1], the study on Integrated Sensing and Communciation targeted at Rel-20 supports 5GS architectural and functional extensions so that sensing service can be supported. One of the objectives that this study attempts to address is:

-	WT-2: Service authorization and revocation. This includes investigating mechanisms for authorization and revocation sensing service requests from the service consumer.

In this paper, service authorization and revocation for sensing service are discussed.

2	Discussion

2.1 Service authorization

In general, NEF performs service authorization for AF request using OAuth-based authorization mechanism in TS 33.501[2]. PCF may further determines whether the AF request is allowed and notifies the NEF if the request is not authorizaed.

In Rel-19 AIoT topic, the service authorization for AIoT mainly includes two parts:

-	NEF performs AIoT AF request authorization based on the service level agreement (SLA) between the 3rd party AF and the 5GS of the mobile network operator, the operator policy and local configuration as in TS 33.501 [2].

-	AIOTF may perform authorization of AIoT service requested by the AF, by retrieving the AF authorization data stored in the AIoT Data Management (ADM).

That is, service authorization for AIoT includes two parts, e.g., SLA part (e.g., NEF performs) and service specific part (e.g., AIoTF performs).

For sensing service, service authorization may also include both SLA part (e.g., NEF performs) and service specific part (e.g., Sensing Function (SF) performs). It is FFS whether or not to stored AF Authorization data for sensing. And whether a new Network Function for storing AF Authorization data for sensing is needed or not should be further discusssed.

Proposal 1: It is proposed to study which network function(s) is/are responsible for service authroization for sensing.

2.3 Service revocation

There’re mainly two options for service revocation for AF request in the existing specification.

- Option#1 is described in clause 4.15.6.6 of TS 23.502 [3], e.g., AF may send Nnef_AFsessionWithQoS_Revoke request to NEF in order to revoke the AF request. The NEF authorizes the revoke request and triggers the Ntsctsf_QoSandTSCAssistance_Delete and/or Npcf_PolicyAuthorization_Delete service operation for the AF request.

- Option#2 is described clause 6.2.1.2 of TS 23.503 [4], e.g., AF may provide Temporal validity condition and Spatial validity condition for specific AF request. It is straightforward that either the violation of Temporal validity condition or Spatial validity condition triggers the revocation of sensing service requests from the service consumer (e.g., AF on behalf of 3rd party Application server).

In general, the above service revocation for AF request can be reused for sensing service revocation. It is FFS whether any enhancement is needed for sensing sevice revocation.

Proposal 2: It is proposed to study whether and how to enhance the existing service revocation for AF request for sensing service.

3 Reference

[1] SP-250401, New SID on Study on Stage 2 for Integrated Sensing and Communication, Xiaomi

[2] TS 33.501, 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Security architecture and procedures for 5G system

[3] TS 23.502, 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Procedures for the 5G System (5GS); Stage 2

[4] TS 23.503, 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Policy and charging control framework for the 5G System (5GS); Stage 2

4 Text Proposal

It is proposed to capture the following changes to TR 23.700-14.

* * * * Start of Change * * * *

## 5.X	Key Issue X: <Service authorization and revocation for sensing service>

### 5.X.1	Description

This key issue will address service authorization and revocation. This includes investigating mechanisms for authorization and revocation sensing service requests from the service consumer. The key issue will study the following aspects:

-  Study which network function network function(s) is/are responsible for service authroization for sensing?

-  Whether and how to enhance the existing service revocation for AF request for sensing service?

* * * * End of Change * * * *