3GPP TSG-SA WG2 Meeting #168	S2-2503626

Goteborg, Sweden, 07 – 11 April 2025

Source:	ETRI

Title:	TR 23.700-14: New key issue for WT-5 Sensing result exposure

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing-ARC / Rel-20

Abstract: This contribution proposes a key issue for WT-5: Sensing result exposure.

# 1. Discussion

The FS_Sensing-ARC SID (SP-250401) identifies WT-5 as follows:

- WT-5: Mechanisms for providing sensing associated information and result (including sensing result and contextual information) exposure, for one time, periodic or event based reporting.

In this work task, it is required to study exposure methods for sensing results and contextual information, identify responsible reporting entities, address security/privacy aspects, and determine the types of information to be exposed.

# 2. Proposal

It is proposed to add the following text to TR 23.700-14.

* * * Start of Change * * * *

## 5.X	Key Issue #X: Mechanisms for providing contextual information and sensing result exposure

### 5.X.1	Description

This key issue studies mechanisms necessary for exposing sensing results and associated contextual information to authorized sensing service consumers, supporting various reporting methods.

This key issue addresses the following aspects:

- How sensing results and associated contextual information are exposed to authorized sensing service consumers (e.g., AF).

- Which entity should be responsible for managing and supporting different reporting methods (e.g., one-time, periodic, or event-based reporting).

- How to handle security and privacy aspects of the sensed targets (or objects) when exposing sensing results and contextual information.

- What specific information (e.g., location, velocity, object type) should be exposed as sensing results, and what associated contextual information (e.g., map information, area information, time of capture) should be provided along with the results.

* * * End of Change * * * *