SA WG2 Meeting ##168	S2-2503858

07 - 11 April, 2025, Goteborg, Sweden

Title:	[Draft] Reply LS on support for multiple non-3GPP device identifiers

Release:	Rel-19

Work Item:	UIA_ARC

Source:	SA2

To:	CT1

Cc:	CT3

Contact Person:

Name:	Yuanxing Xue

E-mail Address:	xueyx@chinasatnet.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

The agreed SA2 CR 5963 (S2-2501225) to 23.501 states:

For the traffic of non-3GPP devices requiring differentiated QoS:

-	...(skip text)

If the PCF indicates to the SMF that the corresponding Non-3GPP Device Identifier is not available for the UE as specified in clause 6.1.3.31 of TS 23.503 [45], the SMF rejects the PDU Session Modification with a cause code to notify the UE that the Non-3GPP Device Identifier is not available for the UE.

From the marked yellow text, there is no consensus in CT1 on:

- whether the multiple non-3GPP device identifiers are allowed in a single PDU session modification procedure;

- whether the SMF rejects the Non-3GPP Device Identifier or rejects the whole PDU SESSION MODIFICATION REQUEST message.

If only a single non-3GPP device identifier is allowed in the PDU SESSION MODIFICATION REQUEST message, the UE/5G-RG must initiate multiple PDU SESSION MODIFICATION REQUEST messages for each non-3GPP device that is connected behind the UE/5G-RG. CT1 discusses that this approach may lead to increased signalling overhead within the system if the UE/5G-RG needs to request QoS differentiation for multiple devices in a short time. For example, consider a scenario where many UE/5G-RG’s in an area are powered cycled and each needs to request QoS differentiation for multiple non-3GPP devices.

The CT1 group would like to highlight that, CT1 also found that TS 29.512 allows the SMF to report multiple non-3GPP device identifiers to the PCF in one message, as referenced in TS 29.512, Table 5.6.2.3-1.

| n3gDevInfos | map(Non3gppDeviceInfo) | O | 1..N | Contains information about the non-3gpp device(s) behind the UE using the PDU Session of the UE. The key of the map is the "n3gDevId" attribute of the Non3gppDeviceInfo data type. | Non3gppDevice |
| --- | --- | --- | --- | --- | --- |

CT1 requests Stage-2 to consider the scenario where multiple non-3GPP device identifiers are removed from the subscription in the UDR and the network therefore needs to stop QoS differentiation for the multiple devices. A key concern is identifying the most efficient mechanism to convey this updated information to the UE.

Given these considerations, CT1 requests SA2 to provide answers to the following questions:

Q1: Whether the multiple non-3GPP device identifiers are allowed in a single PDU SESSION MODIFICATION REQUEST message?

SA2 Answer: Single non-3GPP device identifier in a single PDU SESSION MODIFICATION REQUEST message is the basic way of working. There will be Maximum quantity control on devices behand UE/5G-RG, to reduce the significant signalling overhead when UE/5G-RG cycled restart.

Q2: For the Non-3GPP device identifier not available scenario, whether the SMF rejects the Non-3GPP device identifier or rejects the whole PDU SESSION MODIFICATION REQUEST message?

SA2 Answer: SMF rejects the Non-3GPP device identifier with a reasonable cause code, and does not reject the whole PDU SESSION MODIFICATION REQUEST message.

Q3: For the scenario of removal of more than one non-3GPP device identifier from the subscription in the UDR, whether a single or multiple non-3GPP device identifiers should be permitted in a single PDU SESSION MODIFICATION COMMAND message?

SA2 Answer: Each PDU session has its own unique context and parameters, and separate modification commands are usually issued for each session to ensure the accuracy and granularity of the modifications, a single non-3GPP device identifier should be permitted in a single PDU SESSION MODIFICATION COMMAND message.

2. Actions:

To CT1 group.

ACTION: 	SA2 kindly requests CT1 to take above information into account in normative works.

3. Date of Next SA2 Meetings:

SA2#169                           19th May- 23rd May 2025............................. Fukuoka, Japan

SA2#170                           25th Aug.- 29th Aug. 2025............................Goteborg, Sweden