3GPP TSG-SA2 Meeting #168	S2-2503894

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6069 | rev | 3 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on ECN marking for L4S |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently the text on ECN marking for L4S by NG-RAN says dedicated QoS Flow(s) are used for carrying L4S enabled IP traffic. This is used with the understanding that a QoS Flow is exclusively for carrying L4S enabled IP traffic. The term dedicated QoS Flow(s) is misleading here as it can be interpreted that ECN marking for L4S may not be enabled for a QoS flow associated with the default QoS Rule in a PDU Session. However, under the assumption that only L4S enabled IP traffic is mapped to this Qos Flow, this is possible. In clause 5.37.3.1 NOTE 3 describes that enabling a QoS flow for ECN marking for L4S can be triggered dynamically based on detection of packet with ECT(1) or CE marking. A clarification that this detection can make use of the existing N4 session management procedures between SMF and UPF is required |  |  |  |  |  |  |  |  |
| Summary of change: |  | The text in clause 5.37.3.2 is modified to state that QoS Flows enabled for ECN marking for L4S are used exclusively for carrying L4S enabled IP traffic.Also a NOTE is added to state that ECN marking for L4S can be enabled for a QoS flow associated with default QoS rule if only L4S traffic is mapped to that QoS flow. In 5.37.3.1, NOTE 3 is expanded to say that detection of L4S traffic can be carried out using the usage reporting procedure between SMF and UPF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Text remains ambiguous |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.3.1, 5.37.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

#### 5.37.3.1	General

L4S (Low Latency, Low Loss and Scalable Throughput) is described in IETF RFC 9330 [159], IETF RFC 9331 [160] and IETF RFC 9332 [161]. It exposes congestion information by marking ECN bits in the IP header of the user IP packets between the UE and the application server to trigger application layer rate adaptation.

In 5G System, ECN marking for L4S may be supported. ECN marking for L4S is enabled on a per QoS Flow basis in the uplink and/or downlink direction and may be used for GBR and non-GBR QoS Flows. ECN marking for the L4S in the IP header is supported in either the NG-RAN (see clause 5.37.3.2 and TS 38.300 [27]), or in the PSA UPF (see clause 5.37.3.3).

NOTE 1:	Based on operator's network configuration and policies, SMF decides whether NG-RAN or PSA UPF based ECN marking for L4S is used.

In the case of ECN marking for L4S by PSA UPF, the NG-RAN is instructed to perform congestion information monitoring and report to the PSA UPF the congestion information (i.e. a percentage of packets that UPF uses for ECN marking for L4S) of the QoS Flow on UL and/or DL directions via GTP-U header extension to PSA UPF and accordingly, the PSA UPF may mark the UL and/or DL direction packets of the QoS Flow.

NOTE 2:	As for any QoS Flow, QoS rules in the UE and PDRs in the PSA UPF control which packets are bound to the L4S enabled QoS flow. The Packet Filter Set in the QoS rule or PDR can use packet filter(s) in clause 5.7.6.2 (e.g. match packets with ECT(1) or CE (See RFC 9331 [160]) together with IP 5 tuple) to steer traffic to an L4S enabled QoS Flow.

NOTE 3:	A QoS Flow may be enabled with ECN marking for L4S requirement e.g. statically when a PDU session is established based on configuration in SMF or PCF, or dynamically upon detection of L4S traffic, or by requests by an AF.

NOTE 4:  SMF can also be configured for enabling ECN marking for L4S based on dynamic detection of L4S traffic. SMF instructs UPF by means of solicited application reporting (as described in 5.4.11 of TS 29.244 [65] using parameters for N4 Session management described in clause 5.8.5.1). The Application Identifier in this message can refer to packet detection filter in the UPF configured to detect L4S traffic (e.g. match packets with ECT(1) or CE (See RFC 9331 [160])) in the IP header on a QoS flow associated with default QoS rule. Upon a report of L4S traffic detection from the UPF, SMF triggers a setup/modification of a QoS Flow enabled for L4S. The non-L4S traffic mapping to QoS Flow continues as before but N4 rules are installed so the detected L4S traffic is mapped now to the QoS Flow dynamically enabled for L4S. Under specific configuration this procedure can also detect L4S traffic on QoS Flows associated with other packet filters.

NOTE 5:	To support this functionality, the UE needs to support L4S feedback as described in IETF RFC 9330 [159], which is not in the scope of 3GPP.

When serving PSA UPF or NG-RAN is changed e.g. due to inter-NG-RAN handover or PSA UPF relocation, target NG-RAN and target PSA UPF, if supported, should continue to perform ECN marking for L4S for the QoS Flow. However, if not available (i.e. ECN marking for L4S is not supported in both, target NG-RAN and target PSA UPF), AF should be notified that ECN marking for L4S can no longer be performed if ECN marking for L4S had been enabled for the QoS Flow based on AF request. When ECN marking for L4S is supported again either in target NG-RAN or in target PSA UPF, AF should be notified that ECN marking for L4S can be performed again if ECN marking for L4S had been enabled for the QoS Flow based on AF request.

Next Change

#### 5.37.3.2	Support of ECN marking for L4S in NG-RAN

ECN marking for L4S may be supported in NG-RAN as specified in TS 38.300 [27].

To enable ECN marking for L4S in NG-RAN, QoS Flow(s) dedicated for carrying only L4S enabled IP traffic are used.

NOTE:	To enable ECN marking for L4S in NG-RAN on a QoS flow, it needs to be ensured that only L4S enabled IP flows are transferred on this QoS Flow. Non-compliance will have detrimental impact on the performance of the L4S enabled IP flow.

The SMF may be instructed, based on either dynamic or predefined PCC rule, to provide an indication for ECN marking for L4S to NG-RAN for a corresponding QoS Flow(s) in UL and/or DL directions. In the absence of such PCC rule, the use of ECN marking for L4S in NG-RAN on a QoS Flow is controlled by a coordinated configuration in NG-RAN and 5GC.

NOTE 2:	In home-routed roaming scenarios, a coordinated configuration in NG-RAN and 5GC would require an appropriate SLA between the HPLMN and the VPLMN.

The criteria based on which NG-RAN decides to mark ECN bits for L4S is NG-RAN implementation specific.

In the case of inter NG-RAN UE mobility, if the ECN marking for L4S has been enabled on source NG-RAN, but the target NG-RAN does not support ECN marking for L4S, then the SMF may, if supported, enable ECN marking for L4S in PSA UPF as defined in clause 5.37.3.3.

End of changes