3GPP TSG-SA WG2 Meeting #168	S2-2503980

# 7-11 April, 2025, Goteborg, Sweden	(Revision of S2-2503761)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1641 | rev | 1 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on bootstrap DC removal |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2025-03-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It is stated in current specification that a bootstrap DC in a session shall not be removed during the session. This description is not correct because if a session with bootstrap data channels is diverted to a UE or a IMS network that does not support IMS data channel, the bootstrap data channels targeting to the diverted-to UE shall be removed, as specified in TS 24.186 [105]. The description in the specification needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the description of bootstrap DC removal to clarify the bootstrap DC shall be removed if a session with bootstrap data channels is diverted to a UE or a IMS network that does not support IMS data channel. - clarify that IMS data channel interaction with supplementary services is supported as specified in TS 24.186 [105] in order to avoid duplicating 24.186 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.6, AC.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## AC.7.6	Termination of IMS Data Channel

When a bootstrap data channel is established successfully, the bootstrap data channel shall be terminated in one of the following cases:

-	when the IMS session is released;

-	when conditions to terminate bootstrap DC due to interaction with supplementary services as specified in TS 24.186 [105] are met.

The application data channel shall be terminated along with the release of the call or be terminated by closing this application separately. In the latter case, the UE triggers a SDP negotiation to release the application data channel.

The termination of application data channel shall not have any impact on the audio or video within the IMS sessions.

# * * * Second changes * * * *

# AC.1	General

This annex describes IMS architecture enhancements to support data channel services.

Service based interfaces applicable to data channel services are specified in clauses AA.2.4 and AA.2.5.

The architecture in this annex supports separation of signalling function and media function supporting data channel services. A network function for data channel management signalling, Data Channel Signalling Function (DCSF), is specified. The network function to handle data channel media can be provided by introducing a new network function called Media Function (MF), which interacts with the IMS AS via the service-based interface DC2. MF provides media capabilities in support of IMS DC and Augmented Reality.

NOTE:	In this Release of the specification the IMS AS supporting data channel services is collocated with the TAS.

The IMS architecture supporting data channel services also supports exposure of IMS DC. The architecture and procedures of IMS DC event exposure are specified in Annex AD. The architecture and procedures of IMS DC capability exposure are specified in Annex AG.

In this Release of the specification IMS Data Channel is supported in the case of non-roaming and home routed roaming scenarios.

Multiplexing of multiple IMS Data Channel streams with different endpoints onto a single SCTP association is not supported in this Release of the specification.

IMS data channel interaction with supplementary services is supported as specified in TS 24.186 [105].

# * * * End of changes * * * *