SA WG2 Meeting #168	S2-2504020

Goteborg, Sweden, 07 – 11 April 2025	(revision of S2-2502939)

Source:	LG Electronics, Futurewei, NTT DOCOMO, InterDigital, Ericsson, ZTE, Lenovo, OPPO, CATT, Huawei, Nokia, MediaTek Inc.

Title:	Clause 5.4 updates

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The paper proposes to update clause 5.4.

1. Discussion

[1] Regarding the following EN in clause 5.4 of TS 23.369, it is considered that 'approximate number of AIoT Devices' and 'approximate D2R message size' are useful for AIoT RAN in performing service operations, e.g. radio resource allocation.

Editor's note:	How the AIOTF determines the assistance information (e.g. approximate number of AIoT Devices, approximate D2R message size) without input from the AF is FFS.

Therefore, it is proposed that the AIOTF determines these information based on local configuration if not provided by the AF.

[2] Regarding the following EN in clause 5.4 of TS 23.369, "Time interval as AIoT aggregation assistance information" was added to clause 8 Conclusions of TR 23.700-13 at SA2#167.

Editor's note:	Other assistance information may be added later if necessary.

Therefore, it is proposed to add "Time interval as AIoT aggregation assistance information" to clause 5.4.

2. Proposal

It is proposed to capture the following changes into TS 23.369v0.2.0.

* * * * Start of Changes * * * *

5.4	Assistance information provided to AIoT RAN node

The AIOTF provides the following assistance information to the AIoT RAN together with the service operation requests:

a)	AIoT service type (e.g. Inventory, Command).

b)	Optionally, approximate number of AIoT devices based on AF request.

c)	Optionally, approximate D2R message size based on AF request.

d)	Optionally, time interval as AIoT aggregation assistance information.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	It is FFS where detailed description on time interval as AIoT aggregation assistance information will be captured, i.e. in this clause or in another clause.

If not provided by the AF, bullets b) and c) in the above assistance information provided by the AIOTF may be based on local configuration based on SLA between the AIoT service provider represented by an AF and the operator.

The assistance information is used by the AIoT RAN for performing service operations, e.g. radio resource allocation by using bullets b) and c), AIoT Device responses aggregation by using bullet d).

* * * * End of Changes * * * *