3GPP TSG-SA WG2 Meeting #168	S2-2504479

Gothenburg, Sweden, April 07–11, 2025

Source:	Ericsson

Title:	Energy Optimization Trigger for PCF based Policy Control

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_ph2/Rel-20

Abstract of the contribution: Slice or 5GC level EnC triggers for PCF policy control.

# 1. Introduction/Discussion

In release-19 standardization, it was agreed that the current attribution model of energy consumption (EnC) per UE, per PDU and per Service Data Flow (SDF) per UE is very much dependent on the load of UP and gNB nodes where the UE data traffic is carried. Therefore, the current granular EnC is not good enough as criteria for service adjustment trigger. This contribution proposes a solution to consider more accurate triggers for service adjustment to enable network energy saving.

# 2. Text Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-67.

# * * * * First change * * * *

## 6.X	Solution #XX: Policy Control Considering Slice or 5GC Energy Consumption

### 6.X.1	High-level solution principles

### The solution follows the following principles:

- Existing 5G MM, SM and BDT procedure can be enhanced for energy saving based on coarse-grained energy consumption (EnC) information.

- Slice EnC can be used to trigger service adjustments on all or some slice users

Note: The energy-related information at UE or NF level used in KI#2 and KI#3 should be based on robust and accurate enough estimates to be fit for purpose.

### 6.X.2	Functional Description

This solution addresses KI#1 and KI#2.

Existing 5G MM, SM and BDT procedure can be enhanced for energy saving based on more coarse-grained energy consumption (EnC) information. Using coarser grained EnC information like the slice EnC to trigger service adjustments on all or some slice users offers a robust mechanism and actions may be applied on a sufficient number of users/sessions to have noticeable energy savings.

The AM policy, SM policy and/or BDT policy created by the PCF could consider the requirements of per slice EnC. The slice level EnC thresholds related policy can be added in slice control policy data defined in 5.2.15 Resource: SlicePolicyControlData, of TS 29.519.

The PCF retrieves the slice level EnC from the OAM system (using SA5 defined services) and compares the current consumption with thresholds defines in the policy data. Once EnC exceeds the thresholds, the PCF performs AM policy, SM policy adjustment or BDT policy (re-)selection. The actions can be applying alternative lower UE-AMBR for the UEs, applying alternative lower Session-AMBR for session for the PDU sessions etc. The adjustment can be applied to new UE registration or new PDU session (PDU Session establishment) or applied to existing UE registrations and PDU sessions (PDU Session modification). The PCF may consider UE energy saving indicator subscription and only apply service adjustment to UEs which allows energy saving operation instead of all UEs in a slice.

NOTE: Policy modifications should not violate Slice SLAs (when applicable).

NOTE: whether the solution is to become part of normative work will be based on evaluation of the efficacy of the proposed selection of UEs to be affected by service adjustment and whether the service adjustments provide energy saving in an application-agnostic manner.

### 6.X.3	Procedures

### 6.X.3.0	General

The PCF can apply service adjustment based on the triggers mentioned in 6.X.2 for various procedure. The section only describes the enhancement for PCF initiated SM Policy Association Modification procedure as an example on how the procedure is look like.

#### 6.X.3.1	PCF initiated SM Policy Association Modification

The PCF may initiate the SM Policy Association Modification procedure when the EnC per slice exceed thresholds.

Figure 6.X.3.1: PCF initiated SM Policy Association Modification

A SM Policy Association is already established in PCF as described in clause 4.16.4 TS 23.502 [x] before the SM Policy Association Modification procedure can be triggered.

1.	The PCF retrieves the slice policy related to EnC control trigger. The policy should contain the EnC threshold for triggering and revoking service adjustment or may optionally contains time limit for the service adjustment duration.

2.	The PCF retrieves the slice EnC for each slice that is in used from the OAM system periodically (e.g. per 15 minutes).

3. PCF compares the EnC value to the EnC threshold for triggering service adjustment upon receipt of the EnC.

4. Once the EnC for the slice or 5GC exceed the EnC thresholds, the PCF decides to apply service adjustment for PDU sessions or SDFs in the slice or 5GC.

5. The PCF may select the PDU session or SDF from UEs with energy saving indicator which allows energy saving operations. For each affected PDU session where service adjustment can be applied like applying lower Session-AMBR or lower SDF GBR:

5a. The PCF issues a Npcf_SMPolicyControl_UpdateNotify request to the SMF with possibly updated policy information about the PDU Sessions or SDFs.

5b. The SMF acknowledges the PCF request with a Npcf_SMPolicyControl_UpdateNotify response.

6. The PCF monitors the slice or 5GC EnC and compares EnC to the thresholds as described in step 2 and step  3. The PCF decides to revoke the service adjustment when

- the EnC for the slice or 5GC fall below the EnC threshold for revoking service adjustment

7. For each PDU session which requires revoking service adjustment

7a. the PCF issues a Npcf_SMPolicyControl_UpdateNotify request with revoked policy information about the PDU Sessions or SDFs to the SMF.

7b. The SMF acknowledges the PCF request with a Npcf_SMPolicyControl_UpdateNotify response.

### 6.X.4	Impacts on existing services, entities and interfaces

UDR:

-	Store and provide slice level energy control policy data.

PCF:

- Collect slice level EnC from OAM (using SA5 defined services). Compare the collected EnC to the thresholds in the policy data.

-      Apply/revoke the AM policy, SM policy and/or BDT policy by monitoring the EnC according the EnC policy per slice.

# * * * * End of changes * * * *