SA WG2 Meeting #S2-168	S2-2502847

07 - 11 April, 2025, Goteborg, Sweden

# Liaison Statement

| Liaison Statement Title: | LS to SA2 About Requirements Concerning Automatic Resume Where You have Left Off |
| --- | --- |

| Source Meeting Information |  |  |
| --- | --- | --- |
| Meeting Number | Meeting Date | Meeting Location |
| IMSDCAS22 | February 17, 2025 | Online |

| Document Details |  |  |
| --- | --- | --- |
| Document Number: | Creation Date: | Document Author: |
| IMSDCAS22_005 | February 17,2025 | Walter Zielinski/Huawei |
| Originating GSMA Source: | Deadline for response: | Liaison Statement Contact |
| TSG IMSDCAS | ASAP | GSMALiaisons@gsma.com |
| Security classification: | Non-confidential |  |

| Recipients: |  |
| --- | --- |
| Internal recipients: |  |
| External recipients: | 3GPP SA2, with Copy to 3GPP SA4 and CT1 |

# Background

GSMA Terminals Steering Group (TSG) and GSMA Network Group (NG) has completed phase 1 of data channel profiling with the publication of GSMA TS.66 [1] as per information contained in S4-241435. Phase 1 of data channel specifies minimal set of requirements to support both the protocol UNI/NNI interworking and the application layer API interworking.  And currently GSMA members are submitting contributions to update GSMA PRD NG.134 [2] and GSMA PRD TS.66 [1] so they are in-line with 3GPP Rel-18 features. That work is expected to be completed by Q1.2025.

GSMA TSG IMSDCAS members are now proposing new set of data channel requirements related to the feature “Automatic Resume from the point where you have left off”. This feature is available on the digital platforms, e.g. YouTube, and provides both convenience and engagement benefits. The feature allows to continue interacting with the service from a last point that the user left off. People watching video or downloading files are well acquainted with this feature. In the rest of this text the feature will be referred in short as the Automatic Resumption. The use case details will be described in the next version of GSMA PRD NG.129.

The typical use cases for Automatic Resumption include

- Abnormal or normal session termination by the network or user: in that case the Automatic Resumption allows to continue application session pass the single IMS session duration enhancing user engagement with the platform

JavaScript Application Exception : when JavaScript exception forces the data channel application recovery the Automatic Resumption supports application continuity over exceptional events

- System Notification: when user receives system notification, e.g. security notification, the Automatic Resumption allows user to suspend the application to address the notification and latter resume the usage at a proper point.

The Automatic Resumption feature applies equally to the consumer and enterprise segments. This is convenience feature that helps with increasing user engagement and increasing attention within the application. The Automatic Resumption apply in the case of peer to peer session when A calls B but also with the call transfer case when A call is transferred to C together with the state.

The technical realisation involves data channel application state retention using UE or the network based repositories. It has been confirmed that some companies have already implemented such capabilities in their data channel Software Development Kits. However, due to lack of standards those implementations remain proprietary.

In both, peer to peer and call transfer, cases it is required to have a persistent identifier that serves as the unique key into the database that stores the application context/state. Such a unique ID needs to be exchanged between peers and it has configurable life time. In the rest of the text the proposal for handling state retention in peer to peer and call transfer cases is described.

Data Channel Peer to Peer Session Support for Resume Where You Have Left Off

GSMA IMSDCAS discussed proposed technical solution for resumption in peer to peer scenario based on signalling exchange that require that SDP be modified to carry the state key. And, specifically two alternatives for SDP modification were proposed:

- The first method would extend “3gpp-req-app-value” as defined in section 6.2.13.2 of 3GPP TS 26.114 with a new parameter “3gpp-req-prevappstate-id”, with the new syntax defined as

- 3gpp-req-app-value = req-app-id [";" app-dc-info] *(";" 3gpp-req-app-opt) * (";" 3gpp-req-prevappstate-id)

- , resulting in SDP example in the form of

- a=3gpp-req-app:”application1”;7216-UE”;3gpp-req-prevappstate-id:“application2F344679To236536”

- The second method would define a new SDP attribute “3gpp-req-app-value”

- 3gpp-req- prevappstate -id = “3gpp-req-prevappstate-id:” token to SDP

,and that would be a source of new a line of the form
	a=3gpp-req-prevappstate-id:” application2F344679To236536”

The “3gpp-req- prevappstate -id” is unique ID and is generated by DCMTSI client in terminal as cryptographic Hash. The  Application-id, retrieved from the application context, as part of bootstrapping process, i.e. req-app-id   , originating party MSISDN and terminating party MSISDN are used as input to generate the hash value that serves as database key.

IMS data channel application shall use this unique-id to check if state is stored in the database so they can access application context accordingly and initiate application.

The two requirement to implement this use case are

The support  of “3gpp-req- prevappstate -id” parameter (or similar name)

The inclusion of “3gpp-req- prevappstate -id” parameter into 3GPP Offer/Answer

Data Channel Session Call transfer support for resume where you have left off using external storage and using File Transfer with HTTP

GSMA IMSDCAS has also discussed the call transfer case and the preliminary assessment is that this use case requires additional support beyond SDP body into SIP message header.

NOTE: 	GSMA PRD RCC.07 [4] specified upload and download procedures are also proposed to be reused but it is also possible that the application state can be transferred using directly the application data channel.

When party A and B are in an active call and one of those parties invokes the call transfer of ongoing IMS Data Channel session to C following section 10.19 of 3GPP TS 24.186 the transferring party need to check the IMS Data Channel state retention support from transferred to party C, i.e. capability check. Therefore, it is proposed that the transferor might use SIP OPTIONS capability mechanism with a newly proposed subtype feature tag in this document “+sip.app-subtype= dc-transfer-httpft” or “+sip.app-subtype= dc-transfer-nomedia”.

If transferred to party, i.e. C,  support state retention, then shall respond with the feature subtype in the SIP OPTIONS 200OK response

If transferred to party doesn’t support state retention feature, then transferring party shall not upload IMS Data Channel application state to HTTP file server or IMS Data Channel server instead stores in the local repository for later use

When the call transfer is initiated, transferor (A-party) uploads application context to HTTP storage server as per HTTP  file transfer mechanism defined in GSMA PRD RCC.07 [4] or to IMS Data Channel server.  And A-party sends REFER request to transfer the call to UE-C and adds Call-Info header with app state URL received after upload and prevappstateId, Contact header with “dc-transfer-httpft/“dc-transfer- nomedia“" based on the upload procedure

IMS network will follow the standard procedures defined in section 10.19 of 3GPP TS 24.186  where Re-Invite will be send to transferred to C-party with Call-Info header, Contact header with “dc-transfer-httpft"/“dc-transfer- nomedia“ and the session will be established between A-party and C-party. And the receiving side (C-party) checks for the “dc-transfer-httpft” feature tag in the contact header and downloads application state from the file content URL. Then C-party resumes data channel application from the point stored in the database.

It was proposed that the following extensions are required to support the call transfer case

- Call-info header defined in RFC 3261 to be extended with new parameters including

URL - URL link for the IMS Data Channel state data.

It is the URL received by the transferring party after uploading the Data Channel application state data and needed by the transferred to party to download the application state data.

prevAppStateId - unique-id to identify the application state.

- Used in offer answer update of SDP.

- Example

- Call-Info: url=https://ftcontentserver.rcs.mnc123.mcc456.pub.3gppnetwork.org/ftsf-f6b; prevAppStateId=application2F344679To236536

Contact header defined in RFC 3261 to be extended and include “+sip.app-subtype” media feature tag, with a newly defined value of “dc-transfer-httpft “/“dc-transfer- nomedia “

The new media feature tag would indicate to the user C that user A supports IMS data channel transfer.

- File Transfer HTTP XML body defined in section 4.2.4 of GSMA PRD RCC.07 [4] is used to support the use case and new parameter/namespace added for Data channel app state upload as shown below

<file-correlation-id> Call-Id can be used to understand for which session this app state belongs to.

<other-participant> This field can tell who was other participant when app state is shared.

<Referred-by> This Field carries the referrer's identity

<timestamp> Timestamp when user uploads file

<prevAppStateId> App state id of previous session with same UE

<file-correlation-id> Call-Id can be used to understand for which session this app state belongs to.

<other-participant> This field can tell who was other participant when app state is shared.

<Referred-by> This Field carries the referrer's identity

<timestamp> Timestamp when user uploads file

<prevAppStateId> App state id of previous session with same UE

File Transfer HTTP XML body Section 4.2.4 of GSMA PRD RCC07 – <?xml version="1.0" encoding="UTF-8"?> <xs:schema targetNamespace="urn:gsma:params:xml:ns:rcs:rcs:fthttp" xmlns:xs="http://www.w3.org/2001/XMLSchema" xmlns="urn:gsma:params:xml:ns:rcs:rcs:fthttp"

The possible call transfer flow  scenarios differ depending whether the external server is present or absent as described below.

- Application state transfer in the presence of external server

In this case the home network operator of the party that is transferring the call provides a storage server required for call transfer. The Fully Qualified Domain Name (FQDN) of the storage server is configured at the device and the IP address of the storage server is obtained by resolving the FQDN using Domain Name Service protocol. Upon successful upload of the file as per the procedure defined in GSMA PRD RCC.07[4], a URL pointing to the stored file will be sent to the transferee as part of the SIP REFER.

Figure 1 - Call Transfer Scenario Using External State Server

- Application state transfer in the absence  of external server

When there is no external server to support the storage of the application state for the call transfer scenario then the transferring party can use the data channel directly to transfer the application state to the transferee. That is the application layer protocol is transferring meta-data describing the application resumption state. This might be applicable in cases of domestic or international roaming, if the serving operator does not support storage functionality or the storage server in home network cannot be accessed.

Figure 2 - Figure 1 - Call Transfer Scenario without External State Server

The state retrieval logic differentiates whether the retrieval is for the peer to peer or call transfer scenario. Based on the call context for the peer to peer case the state is retrieved from the device and for the call transfer case the state is retrieved from device and uploaded to the external server.

At this time GSMA TSG IMSDCAS is only planning to update GSMA PRD NG.129 [3] with the Automatic Resumption use case description and at latter stage after receiving further feedback from 3GPP, we might add some additional description into GSMA PRD NG.134 [2] or GSMA PRD TS.66 [1].

# Request to 3GPP SA2

GSMA TSG IMSDCAS kindly requests 3GPP SA2 to help with the following clarifications related to the Automatic Resumption use cases and the technical feature

- Confirm whether 3GPP SA2 or 3GPP SA4 has considered the use cases of the class “Resume from the point where you have left off” and what was the conclusion on this topic?

- Confirm whether the Automatic Resumption technical feature and state preservation might interact with any other 3GPP specified features?

- Confirm whether the described feature is in any way related to the session continuity or might impact the supplementary services?

- Confirm if 3GPP SA2 plans to define and capture the requirements related to “Automatic Resume from the point where you have left off” and whether 3GPP SA2/SA4 plans to specify technical solution for this feature?

- Advice on the feasibility of described technical implementation?

- Advise whether it would be possible to realize such state retention functionality without the need to introduce SDP modifications or any other modifications to the existing standards, i.e. by using session cookies, or exchanging the state data as part of application or bootstrap data channel protocol, including that of consensus?

- Confirm whether 3GPP SA2 / SA4 have considered scenarios where DCS stores an application context or other application data and DCS sends it as part of bootstrap process to UE or when a such data is requested by data channel application

# Contact

In the case of further questions and/or feedback these can be directed to Paul Gosden, GSMA Terminals Director [mailto:paul.gosden@gsma.com].

# References

[1] TS.66 v1.0 IMS data channel API specification, https://www.gsma.com/get-involved/working-groups/gsma_resources/ts-66-v1-0-ims-data-channel-api-specification

[2] NG.134 IMS data channel, https://www.gsma.com/newsroom/wp-content/uploads//NG.134-v1.0-3.pdf

[3] NG.129 v1.0.1 IMS Data Channel White Paper, https://www.gsma.com/newsroom/wp-content/uploads//NG.129-v1.0-1.pdf

# [4] RCC.07 RCS Advanced Communications Services and Client Specification

# NEXT IMSDCAS MEETINGS:

TSG#59		10th - 14th March 2025, Shenzhen China

TSG#60 		9th - 13th June 2025, Stavanger Norway

TSG#61		1st - 5th September 2025, Bermuda

TSG#62		1st - 5th December 2025 Malaga Spain