3GPP TSG-SA WG2 Meeting #168	S2-2502940

Goteborg, Sweden, April 07 – 11, 2025

Source:	OPPO

Title:	Key issue for WT#1: Standardized UE data collection based on UP connection between UE and 5GC.

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes the key issue for WT#1 on how to transfer of standardized data over user plane between the UE and 5GC to support the standardized UE data collection.

- Discussion

In the RAN LS RP-242389, the following agreements on UE side data collection are captured as bellow:

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. only UE proprietary information can be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of MNO controllability and visibility possible via solution 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 could not reach consensus on the feasibility of UP solution in options 2 and 3. |  |  |  |  |

SA2 has discussed and approved a R20 FS_AIML_CN_Ph2 SID which is captured in SP-250413. The aim of this study work is to investigate and identify potential architecture and system level enhancements to support AI/ML enhancements. Specifically, the objectives include:

WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

Note 1: The WT#1 scope is based on UP Data Collection for Option2 documented in RAN LSs RP-243316 and RP-242389.

Note 2: The requirements are elaborated in LSs RP-242389 and R2-2411152 which will be considered during the study phase.

Note 3: WT#1 impact on user consent and privacy will be addressed in cooperation with SA3.

In RAN LSs RP-243316, it is clearly mentioned transfer of data over UP for Solution 2 in SA2 should be studied and provide feedback on fulfillment of visibility and controllability requirements. 	Transfer data over UP including the following aspects:

- Which 5GC NF will be used to support the UE data collection.

- How the 5GC NF determine to establish a new UP connection or select the existing UP connection for the UE data collection.

- Whether there will be one or more UP connection established between the UE and the 5GC NF for the different UE data collection request.

- How to maintain the UP connection between the UE and the 5GC NF.

- How to release the UP connection when UE data collection is completed.

- Whether and how to fulfill the visibility and controllability requirements when support the standardized UE data collection over UP connection.

- Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-04.

* * * Start of Change * * * *

5.x	Key Issue #x: Transfer of standardized data over user plane

5.x.1	Description

In RAN LSs RP-243316, it is explicitly stated that the transfer of data over the User Plane in SA2 should be studied, with feedback provided on whether it fulfills visibility and controllability requirements.

SA2 should conduct a study on how to establish a UP-based connection between the UE and the 5GC to facilitate UE data collection. This key issue studies whether and how to support transfer standardized UE data based on UP connection between UE and 5GC. In particular, this key issue will study:

- Which 5GC NF will be used to support the UE data collection.

- How the 5GC NF determine to establish a new UP connection or select the existing UP connection for the UE data collection.

- Whether there will be one or mutiple UP connection established between the UE and the 5GC NF for the different UE data collection request.

- How to maintain the UP connection between the UE and the 5GC NF.

- How to release the UP connection when UE data collection is completed.

NOTE 1:	Any data to be collected from UE by 5GC NF for AI/ML for NR air interface operation with UE-side model training is assumed to be defined by RAN WGs.

NOTE 2:	Solution should fulfill the visibility and controllability requirements when support the standardized UE data collection over UP connection.

* * * End of Change * * * *