3GPP TSG-SA2 Meeting #168	S2-2503025

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5398 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AF specific UE ID generation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To address the issue described in LS (S2-2502845), this CR proposes to support generation of AF specific UE ID in the 5GC. This CR proposes to enable NEF to generate the AF specific UE ID and store it in UDR. The following principle should be applied: - UDM shall be able to distinguish the following two cases, (case-1) the AF specific UE ID is not stored because AF is not authorized to get AF specific UE ID and no use consent is given, (case-2) the AF specific UE ID is not stored while AF is authorized to get AF specific UE ID and user consent is also given. - If an AF is authorized to get AF specific UE ID and if the related user consent is given, the 5GC can decide to generate AF specific UE ID, store it in the UDM/UDR, and provides it via NEF to AF. - The generation of AF specific UE ID is not allowed for any AF. Only if such a new service specific parameter, i.e., AF specific UE ID, is authorized to be provisioned in the 5GC for a given UE and AF, the AF specific UE ID will be generated and provided. It is assumed that such authorization can be based on SLA between AF provider and operator, or operator policy. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support generation of the AF specific UE ID |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Retrieve AF specific UE ID service operation can be supported only in the limited scenario and cause unintended error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

### 4.15.10	AF specific UE ID retrieval

This clause contains the detailed description and the procedures for the AF specific UE ID retrieval. The AF specific UE Identifier is represented by the External Identifier as defined in TS 23.003 [33].

NOTE 1:	After retrieving AF specific UE ID, the AF can invoke NEF provided services (e.g. location monitoring).

NOTE 2:	As described in sub-clauses of clause 4.3.6, NEF can invoke steps 3 to 6 of this procedure to get the assigned UE IP address, SUPI and DNN and S-NSSAI, and if HR-SBO applies, the DNN and S-NSSAI is the HPLMN's, and also an indication that the PDU Session is working in HR-SBO mode.

Figure 4.15.10-1: AF specific UE ID retrieval

1.	AF requests to retrieve UE ID via the Nnef_UEId_Get service operation. The request message shall include UE address (IP address or MAC address) and AF Identifier, it may include, Port Number associated with the IP address, MTC Provider Information, Application Port ID, IP domain. The MTC Provider Information identifies the MTC Service Provider and/or MTC Application. If available, the AF may also provide the corresponding DNN and/or S-NSSAI.

NOTE 3:	The MTC Provider Information can be used by any type of Service Providers (MTC or non-MTC) or Corporate or External Parties for, e.g. to distinguish their different customers.

NOTE 4:	The combination of IP address and Port Number can be used by 5GC to derive the UE private IP address assigned by 5GC if the UE is behind a NAT, see steps 3-6 below.

NOTE 5:	The Application Port ID is as defined in Nnef_Trigger_Delivery.

NOTE 6:	The NEF can validate the provided MTC Provider Information and override it to a NEF selected MTC Provider Information based on configuration. How the NEF determines the MTC Provider Information, if not present, is left to implementation (e.g. based on the requesting AF).

2.	The NEF authorizes the AF request. If the authorisation is not granted, the NEF replies to the AF with a Result value indicating authorisation failure; otherwise the NEF proceeds with the following steps. The NEF determines corresponding DNN and/or S-NSSAI information: this may have been provided by the AF or is determined by the NEF based on the requesting AF Identifier, MTC Provider Information.

If the NEF has received a Port Number in step 1, based on configuration, the NEF may recognize the address received is an IP address which is different from the actual private UE IP address assigned by 5GC, i.e. the UE is behind a NAT in UPF. If so, the NEF performs steps 3 to 6. Otherwise, steps 3 to 6 are skipped.

3.	The NEF uses the Nnrf_NFDiscovery service operation to obtain the address of the UPF implementing NAT functionality for the UE (public) IP address. The request includes the UE (public) IP address. The NEF may also include the DNN and S-NSSAI associated with the AF ID, as well as the IP domain.

4.	The NRF responds with a Nnrf_NFDiscovery response message including the UPF address of the UPF implementing NAT functionality for the UE (public) IP address.

5.	The NEF uses the Nupf_GetUEPrivateIPaddrAndIdentifiers_Get service operation to request UE's (private) IP address from the UPF. The request includes the UE (public) IP address and Port Number and optionally IP domain, DNN and S-NSSAI associated with the AF ID.

6.	The UPF responds with the Nupf_GetUEPrivateIPaddrAndIdentifiers_Get response message including UE's IP address and optionally, the IP domain. If the UPF has applied a NAT functionality, the UE's IP address returned by the UPF is the private UE IP address. If IP domain of UE private IP address is returned from UPF, it always takes precedence regardless of whether the IP domain information also provided by AF when it invokes Nnef_UEId_Get service operation. If UPF has the SUPI or GPSI of the UE, the UPF may return SUPI or GPSI and in this case steps 7-8 are skipped.

For HR-SBO case as described in clause 4.3.6.1 and in TS 23.548 [74], an indication that the UE PDU session is working in HR-SBO mode, SUPI and HPLMN DNN and S-NSSAI of the PDU session are also provided by UPF.

NOTE 7:	The SUPI/GPSI is only available when the SMF provides it to the UPF for the purposes defined in TS 29.244 [69].

7-8.	The NEF uses the Nbsf_Management_Discovery service operation with UE address and IP domain and /or DNN and/or S-NSSAI to retrieve the session binding information of the UE. If no SUPI is received in the session binding information from the BSF, the NEF replies to the AF with a Result value indicating that the UE ID is not available.

9.	The NEF interacts with UDM to retrieve the AF specific UE Identifier via the Nudm_SDM_Get service operation. The request message includes SUPI or GPSI and at least one of Application Port ID, MTC Provider Information or AF Identifier.

10.	The UDM responds to the NEF with an AF specific UE Identifier represented as an External Identifier for the UE which is uniquely associated with the Application Port ID, MTC provider Information and/or AF Identifier.

10a. [CONDITIONAL] If the AF specific UE Identifier is not available at UDM/UDR and if it is authorized to provide AF specific UE Identifier, the UDM responds to the NEF with an AF specific UE Identifier represented as an External Identifier for the UE which is uniquely associated with the Application Port ID, MTC provider Information and/or AF Identifier.

10b. [CONDITIONAL] The NEF stores the generated AF specific UE ID, which is uniquely associated with the Application Port ID, MTC provider Information and/or AF Identifier, in the UDR.

11.	The NEF further responds to the AF with the information (including the AF specific UE Identifier represented as an External Identifier) received from the UDM.