3GPP TSG-WG SA2 Meeting #168	S2-2503139

Gothenburg, Sweden, 7th – 11th April, 2025

Source:	Huawei, HiSilicon

Title:	New solution to support UE data collection and transfer for AI based positioning

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: A new solution is proposed to support UE data collection and transfer for AI based positioning.

# 1. Introduction/Motivation

SID SP-250413 has WT#1 to address RAN requirements:

WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

Note 1: The WT#1 scope is based on UP Data Collection for Option2 documented in RAN LSs RP-243316 and RP-242389.

Note 2: The requirements are elaborated in LSs RP-242389 and R2-2411152 which will be considered during the study phase.

Note 3: WT#1 impact on user consent and privacy will be addressed in cooperation with SA3.

According to RAN study, there are mainly three use cases related to UE data collection and transfer: CSI prediction, AI based positioning (case 1, 2a), and beam management. In Release-19, it is newly specified in TS 23.273 that NWDAF can invoke Nlmf_DataExposure service to get data from LMF for LMF-side model training. Furthermore, it is already supported that UE can setup User Plane connection with LMF to positioning parameters/data transmission. Thus, for AI based positioning use case, we can build a solution by reusing existing LMF data exposure service and UP connection procedure to UE.

# 2. Proposal

It is proposed to capture the following changes to TR 23.700-04.

# * * * * First change (All text are new) * * * *

## 6.X	Solution to support UE data collection and transfer for AI based positioning

### 6.X.1	Description

According to RAN study, there are mainly three use cases related to UE data collection and transfer: CSI prediction, AI based positioning (case 1, 2a), and beam management. In Release-19, it is newly specified in TS 23.273[x] that NWDAF can invoke Nlmf_DataExposure service to get data from LMF for LMF-side model training. Furthermore, it is already supported that UE can setup User Plane connection with LMF for positioning parameters/data transmission. Thus, for AI based positioning use case, we can build a solution by reusing existing LMF data exposure service and UP connection procedure to UE as defined in clause 6.18.1 of TS 23.273 [x].

### 6.X.2	Scope and assumptions of the solution

We assume that a dedicated NWDAF is responsible for UE data collection and exposure to the OTT server. We call it Data Exposure NWDAF (DE-NWDAF). How DE-NWDAF exposes data to the OTT server will not be addressed in this solution.

Editor’s note: Alternatively, OTT server may get the data from LMF via NEF. How OTT server gets the data from LMF via NEF is FFS.

### 6.X.2	Procedure

The procedure of UE data collection for AI-based positioning case 1 and 2a via User Plane is illustrated by Figure 6.X.2-1.

Figure 6.X.2-1 UE data collection via User Plane for AI-based positioning

- DE-NWDAF determines to collect UE data for AI-based positioning based on either data request from the OTT server, or its internal trigger.

- DE-NWDAF discovers an LMF serving an AoI and supporting the Nlmf_DataExposure service. The NRF selects one or more LMFs based on the AoI, the Nlmf_DataExposure service and sends an Nnrf_NFDiscovery_Request Response which includes the profiles of the selected LMFs to the NWDAF.

- DE-NWDAF subscribes to UE data from LMF by invoking Nlmf_DataExposure_Subscribe service operation. The NWDAF includes an AoI and a data context ID representing the use cases (case 1 or case 2a).

Editor’s note: It is FFS if it needs to distinguish case 1 and case 2a.

- LMF gets the UE list in the AoI and checks user consent as described in step 1-4 in the procedure of clause 6.22.1-1 in TS 23.273 [x]. Then LMF initiates the User Plane connection procedure with the UE as described in clause 6.18.1 in TS 23.273 [x].

- LMF gets measurement data from the UE via User Plane connection. LMF may also get labels from UE depending on whether the labels should be generated by the UE or LMF. UE may also include its cell ID into the data sent to the LMF. The cell ID is used by the DE-NWDAF to match the AoI of further data requests from the OTT server(s).

NOTE: The procedure to collect data by the LMF from the UE is up to RAN WGs. The data collection or measurement procedure between NG-RAN and UE is up to RAN WGs.

- LMF provides the measurement data and labels to DE-NWDAF using Nlmf_DataExposure_Notify service.

Editor’s note: Whether and how the network (e.g., LMF, DE-NWDAF) can be aware whether the data collected from the UE is the same from the one measured by the UE is FFS.

### 6.X.3	Impacts on services, entities and interfaces

-	LMF: Nlmf_DataExposure service is enhanced to support cases of UE data collection.

-	AMF: Namf_EventExposure service may be enhanced to support new event ID to provide the list of UEs who support the UE data collection feature.

-	NWDAF: A dedicated NWDAF is needed to support UE data collection and exposure to the OTT server.

# * * * * End of changes * * * *